3GPP TSG-SA2 Meeting #169	S2-2504624

Fukuoka, Japan, 19th May 2025 - 23rd May 2025

# Title:	[DRAFT] Reply LS on AI/ML Positioning Case 3a

# Response to:	R3-252542, S2-2504525, LS on AI/ML Positioning Case 3a

# Release:	Rel-19

# Work Item:	NR_AIML_air-Core

Source:	CEWiT (To be SA WG2)

To:	RAN WG3

CC:

Contact person:	Anusuya Balasubramanian

anu@cewit.org.in

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2504625 (Discussion Paper)

# 1	Overall description

SA2 thanks RAN3 for the LS on AI/ML Positioning Case 3a. SA2 would like to answer RAN3’s query on checking the feasibility of solution proposed by RAN3 for AI/ML Positioning Case 3a.

SA2’s Response for Solution #1:

Solution #1 is feasible from the SA2 perspective. SA2 can add the proposed procedure in a new clause in TS 23.273. In addition, SA 2 would like to provide following feedback:

- LMF may not share the location of the UE in case of UE Positioning procedure failure or User consent check failure. So SA2 suggests RAN 3 to add a failure indication in step 4 considering a possibility of failure in LMF response.

SA2’s Response for Solution #2:

Solution #2 is feasible from the SA2 perspective but SA2 would like to point some issues and suggest appropriate modifications to the procedure as follows:

- The existing service Nlmf_Location_DetermineLocation is associated with a single UE. Use of this service for every UE in the RAN suggested UE list, may increase the signalling load in the CORE.

- Consolidation of responses by the AMF in step 5 may increase the load and affects the regular AMF functionalities.

- Solutions suggested for issues identified above:

Rather than the AMF consolidating the UEs, if the positioning information request from the NG-RAN can be forwarded to the appropriate LMF(s) and letting the LMF(s) manage the consolidation shall be a better approach.  AMF can use the existing LMF service Nlmf_DataExposure_Subscribe as defined in TS 23.273 section 8.3.4.2 to convey the NG-RAN request to LMF. As response to Nlmf_DataExposure_Subscribe, the LMF can use Nlmf_DataExposure_Notify service to send the consolidated positioning information for the list of UEs to the AMF.

- Also, SA2 would like to suggest to RAN 3 that, for LMF to use the same NRPPa message to report the positioning information to NG-RAN as in step 4 of solution #1 instead of step 5 and 6 of solution #2.

  - The advantage of this option shall be that the positioning information consolidation by AMF or LMF for the list of UEs can be avoided.

  - This also avoids storage of training data of every UE for longer duration in NG-RAN, facilitating stored training data at NG-RAN to be utilized as and when the positioning information of the UE is received. This enables the load distribution of the training data handling at RAN.

- In step 2 the list of UE(s) provided in the NGAP message can be optional as LMF can fetch the list of UEs from AMF using Namf_EventExposure service, as already defined in TS 23.502 clause 5.2.2.3.

# 2	Actions

To RAN3 group

ACTION: 	SA2 asks RAN3 group to consider the feedback provided for the solutions and update the procedure if needed.

# 3	Dates of next TSG SA WG2 meetings

SA2#170:	25 - 29 August 2025	Goteborg, SE

SA2#171:	13 - 17 October 2025	TBD, CN