3GPP TSG-WG SA2 Meeting #169	S2-2504645

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Ofinno

Title:	Command Procedure Update

Document for:	Approval

Agenda Item:	19.4.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Clause 6.2.3 is updated to add more details about the command procedure.

# 1. Introduction/Discussion

It is proposed to update command procedure 6.2.3. The proposal is highlighted:

- Adding the “follow on command indication” in step 7.

- Adding, the RAN A-IoT device NGAP ID allocation and usage in step 7.

- Adding, step 8b for Command Cancellation. The AIOTF may send a Command Cancellation message after step 7, if none of the AIoT Device(s)’s authentication/verification is successful in step 11 of the procedure for Inventory. For the case, no single Command procedure (e.g., the inventory was for a single AIoT device, and the authentication/verification was failed) will be triggered but the NG-RAN node is waiting for the Command Request Message based on the “follow on command indication”, it is proposed to send a Command Cancellation message to the NG-RAN. Accordingly, the NG-RAN can determine a completion of the Inventory and command procedure.

For the same Inventory procedure (inventory and command), an authentication/verification of some AIoT Devices are successful but some are not. In this case, the AIOTF may include a last command indication in the Command Request message if the Command Request is a last one for the same service (same correlation id).

- Merged the existing Step 9 ~ 10 to Step 9 and proposed to refer the RAN specification.

- Adding, step 10b, which is already agreed from RAN3.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

### 6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF additionally includes a “follow-on command indication” in the Inventory Request.

-  In step 10, the NG-RAN allocates and include RAN A-IoT device NGAP ID for each AIoT NAS PDU, in the Inventory Response.

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If no Inventory Response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8a.	In Step 11 of procedure for Inventory specified in clause 6.2.2 was successful, the AIOTF sends Command Request message (Correlation ID, RAN A-IoT device NGAP ID [Reader ID], NAS Command Request message, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. If the Command Request message is the last command associated with the correlation ID, the AIoTF additionally includes a ‘last command indication’ in the Command Request message.

8b. In step 11 of procedure for Inventory specified in clause 6.2.2 was not successful for any of the AIoT Devices in the Inventory Report, the AIOTF sends Command Cancellation message (Correlation ID) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in the clause 6.2.4. Step from 9 onward will not be progressed. The AIOTF and the NG-RAN can release a context associated with the Correlation ID.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends/receives the AS R2D message (NAS Command Request) and the AS D2R message (NAS Comment Response) with the AIoT Device. Details are described in TS 38.XXX.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

10a. 	For successful procedure for the command, the NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4

10b. For unsuccessful procedure for the command, the NG-RAN responds with a Command Failure message (Correlation ID, cause for the failure).

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

# * * * * End of changes * * * *