3GPP TSG-SA2 Meeting #169	S2-2504662

19- 23 May 2025, Fukuoka, Japan

Source:	China Telecom

Title:	23.700-19: New solution of setup UP bearer for vioce for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper propose solution of setup UP bearer for vioce for KI#1

1Discussion

There is a demand that a UE using GEO satellite access should be provided both voice service and other services, e.g. internet, VPDN. In existing NB-IoT specification the supported maximun number of user plane radio bearers for a UE is 2. It is possible that when an new UP bearer for voice needs to be activated, while the number of current activated UP bearers of the UE has reached the supported maximum number of user plane radio bearers(existing NB-IoT specification supports 2 UP bearers at most). Since the voice service has high priority than other services, it is feasible to temporarily suspend other UP bearer and activate the UP bearer for voice. The similiar method was already supported in CS fallback in E-UTRAN access. A solution to suspend other UP bearer for other service and activate the UP bearer for voice is proposed in the paper.

2 Proposal

It is proposed to agree the solution of setup UP bearer for voice for KI#1 into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | X |  |  |  |

* Second change (all New)*

6.X	Solution #X: Setup UP bearer for voice connection

6.X.0	High-level solution Principles

This solution propose to suspend other UP bearer and activate the new UP bearer for voice, to ensure the voice service, if current activated UP bearers of the UE has reached the supported maximum number of user plane radio bearers. It is supplementary solution to the UP only solution or combined CP/UP solution. For UP only solution, the GEO call uses two UP bearers. For combined CP/UP solution, the GEO call uses one UP bearers.

6.X.1	Description

When an new UP bearer for voice needs to be activated, while the number of current activated UP bearers of the UE has reached the supported maximum number of user plane radio bearers(existing NB-IoT specification supports 2 UP bearers at most). To ensure the setup of UP bearer for voice, the MME temporarily suspends a UP bearer for other PDU connection and activate the UP bearer of voice in IMS PDN connection. At the end of the vioce call, the UP bearer of voice is deactivated and then the MME resums the previously suepended bearer.

6.X.2	Procedures

6.X.2.1 Bearer suspend and resume to support voice bearer

Figure 6.X.2.1-1: Bearer suspend and resume to support voice bearer setup

- The UE has established bearer for other APN corresponsing to other service. Bearer for IMS signalling is already established. UE is within a IMS SIP session.

- When an new UP bearer for IMS voice needs to be activated, the PGW1(for IMS PDN connection) sends Create Bearer Request to MME to setup dedicated bearer for voice. The detailed procedure is assumed to be described in UP only solution or combined CP/UP solution

- If the number of current activated UP bearers of the UE has reached the supported maximum number of user plane radio bearers(existing NB-IoT specification supports 2 UP bearers at most), the MME determines to temporarily suspend a beare of other PDN connection, in order to ensure the setup of bearer for voice. The MME can decide the PDN connection, e.g. based on configuration on prioroty of APNs.

The MME sends E-RAB Release Command to eNB to release allocated resources for the bearer. The MME can indicates to UE the bearer is suspended for voice.

- The eNB sends the RRC Connection Reconfiguration message including the EPS Bearer Identity to release and the information of bearer being suspended for voice to the UE. The UE should not try to ask for activate the bearer. Then the eNB sends E-RAB Release response to MME.

- The MME sends Suspend Notification to PGW2 for the PDN connection to be suspended. It is similiar approach as used for CS fallback as described in TS 23.272.

- Step 5 follows the procedures to activate dedicated bearer(for voice) as described in clause 5.4.1-1 of TS 23.401

- After the end of voice call, the procedures to deactivate dedicated bearer(for voice) as described in 5.4.4.1-1 of 23.401 are performed.

- The MME sends E-RAB Setup Command to eNB to assign resources for the bearer of the PDN connection suspended before. The MME can indicates to UE the bearer is resumed from voice.

- The eNB sends the RRC Connection Reconfiguration message including the information of bearer being resumed to the UE. Then the eNb sends E-RAB Setup response to MME.

- The MME sends Resume Notification to PGW2 for the PDN connection to be resumed. It is similiar approach as used for returing back after CS fallback as described in TS 23.272.

6.X.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

MME:

- Determination to temporarily suspend a beare for other PDN connection, in order to ensure the setup of bearer for voice, if the number of current activated UP bearers of the UE has reached the supported maximum number of user plane radio bearers(existing NB-IoT specification supports 2 UP bearers at most)

- Suspending PDN connection before activating bearer for voice, and resuming the PDN connection after deactivating bearer for voice

UE:

- If a bearer is suspended for voice as indicated by network, the UE should not try to ask for activate the bearer.

* End of change *