3GPP TSG-SA2 Meeting #169	S2-2504670

# 19 - 23 May, 2025, Fukuoka, Japan

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0378 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to implementation of 23.167_CR0377R2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 23.167 CR0377R2 contained a reference to a new 'Annex X' which was not modified to 'Annex N' on implementation. This CR corrects the mis-implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change reference to Annex X to 'Annex N' in clause 6.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference to non-existing Annex X remains in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.1	Proxy-CSCF

-	Handle registration requests with an emergency registration indication like any other registration request, except that it may reject an emergency registration request if the IM CN subsystem that the P-CSCF belongs to can not support emergency sessions for the UE (e.g., due to operator policy or UE is not within IM CN subsystem's geographical area or IP-CAN not supported).

-	Detect an emergency session establishment request.

-	Reject/allow unmarked emergency requests.

-	Reject/allow anonymous emergency requests.

-	Prevent non-emergency requests that are associated with an emergency registration.

-	May query IP-CAN for location identifier.

-	May query IP-CAN for additional subscriber related identifier(s).

-	Select an Emergency CSCF in the same network to handle the emergency session request. The selection method is not standardized in the present document.

-	Alternatively, for non-roaming subscribers and when the request is received over a non-emergency registration, the P-CSCF may forward an emergency session to an S-CSCF if so instructed by operator policy or local regulation.

NOTE:	This can be for example the case if the P-CSCF recognizes that an emergency session was not received via a security association for a UE previously authenticated with digest type proxy authentication.

-	Do not apply emergency session detection if requested using private network traffic and forward the session to the S-CSCF, except if operator policy requires the P-CSCF to detect emergency session requests and treat detected emergency session requests as if they are part of public network traffic.

-	For UEs without credentials, forward the equipment identifier to the E-CSCF that was received from the UE.

-	For UEs without credentials and subjected to local regulation, forward the additional subscriber related identifier(s) received from IP-CAN to the E-CSCF.

-	Prioritize the emergency session.

-	Check the validity of the caller TEL-URI if provided by the UE and shall provide the TEL-URI in the session establishment request if it is aware about the TEL-URI associated with the Public User Identifier used for an emergency registration.

-	May respond to a UE with an emergency service indication as a result of detecting a non UE detectable emergency session establishment request

-	May respond to the UE with an indication, IMS emergency registration required as a result of processing the emergency session establishment attempt.

-	Should be able to identify the service data flow associated with emergency service and inform PCRF accordingly.

-	Upon IMS registration failure the P-CSCF may indicate to the UE whether anonymous IMS emergency sessions are supported.

-	P-CSCF may, based on operator policy, insert attestation information related to the asserted calling identity associated with an emergency session, if operating in a network that supports calling number attestation and signing.

-	P-CSCF may assert Resource-Priority information for an emergency session if configured through operator policies.

-	For a test eCall as defined in Annex N, detect a session establishment request for a test eCall. The detection of test eCall can be based on e.g. Request-URI set to a configured value of non-emergency service URN (e.g. urn:service:test.sos.ecall).

# * * * * End of changes * * * *