3GPP TSG-WG SA2 Meeting #169	S2-2504671

Fukuoka, Japan, May 19 – 23, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	New solution to KI#4 on sensing data and the associated information transport

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#4 on sensing data and the associated information transport for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes a new solution to address KI#4 on sensing data and the associated information transport. This solution refers to the clause 6.18 of TS 23.273 on LCS-UP connection establishment procedure.

This solution has the following assumptions:

- The sensing data and associated information are transmitted between the Sensing Entity and the Sensing Function via user plane.

2)  The user plane connection establishment procedure is carried out on control plane signalling interaction.

3)  The Sensing Function provides its user plane information (e.g. IP address) and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity.

4)  The Sensing Entity sends binding information via established user plane connection to Sensing Function for connection binding.

5)  The Sensing Entity can be UE or NG-RAN.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6.X	Solution #X: Sensing data and associated information transport via user plane

6.X.1	Description

This solution has the following assumptions:

1)	The sensing data and associated information are transmitted between the Sensing Entity and the Sensing Function via user plane.

2)	The user plane connection establishment procedure is carried out on control plane signalling interaction.

3)	The Sensing Function provides its user plane information (e.g. IP address) and binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity.

4)	The Sensing Entity sends binding information via established user plane connection to sensing function for connection binding.

5)	The Sensing Entity can be UE or NG-RAN.

6.X.2	Procedures

Figure 6.x.2-1: Procedure to establish User Plane connection between Sensing Function and Sensing Entity

1.	The Sensing Function determines to utilize user plane for sensing data and associated information transport.

2.	The Sensing Function sends the user plane information to AMF in a N1 or N2 container to indicate the selected sensing entity (i.e. UE or NG-RAN node) to utilize user plane for sensing. The user plane information includes the user plane sensing address of the Sensing Function. The binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) associated with the user plane connection is also included in the user plane information.

3.	AMF sends the user plane information to the Sensing Entity via N1 or N2 message.

4.	After the Sensing Entity receives the user plane information, the Sensing Entity establish user plane connection with the Sensing Function according to the user plane sensing address of the Sensing Function. If the Sensing Entity is a UE, the UE shall establish a PDU Session or use an existing PDU Session to establish user plane connection with the Sensing Function. After the user plane connection has been established successfully, the Sensing Entity sends the binding information received in step 3 to Sensing Function via the established user plane connection to enable the Sensing Function to perform the correlation of the Sensing Entity or Connection ID with the user plane connection.

5.	The Sensing Entity sends an acknowledgement to Sensing Function through AMF to indicate a success of user plane connection establishment for sensing service.

6.	AMF sends the acknowledgement received in step 5 to the Sensing Function.

7.	If Sensing Function or Sensing Entity determines to utilize the user plane connection for sensing and the user plane connection is established, messages are transferred between Sensing Entity and Sensing Function for sensing data and the associated information transport.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AMF:

-	Support to send the user plane information received from Sensing Function to Sensing Entity via N1 or N2 message.

Sensing Function:

-	Support to provide user plane information to AMF in N1 or N2 container.

-	Support to receive binding information from the Sensing Entity via user plane for user plane connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Entity via user plane.

Sensing Entity:

-	Support to establish user plane connection with the Sensing Function according to the user plane information received from AMF.

-	Support to send binding information to the Sensing Function via user plane for user plane connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Function via user plane.

* * * * End of Changes * * * *