# 3GPP TSG-WG SA2 Meeting #169			S2-2504673

# Fukuoka, Japan, May 19 – 23, 2025               	       (revision of S2-220xxxx)

Source: 	OPPO

Title: 	23.700-14: Update to KI#2 on Sensing area authorization

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes to update the key issue #2 on Sensing area authorization.

1. Discussion

The WT-2 study addresses the need to dynamically authorize and revoke sensing services based on operator policies, regulatory constraints, and service consumer requirements. As Integrated Sensing and Communication (ISAC) supports diverse applications (e.g., public safety monitoring, commercial collision avoidance), robust mechanisms are required to validate service requests, enforce access control, and terminate sensing operations when policies are violated. A critical subset of this study focuses on ​service area authorization, which restricts sensing activities to predefined geographical zones to comply with regulations or commercial agreements.

Service Authorization and Revocation Background
Sensing services must dynamically adapt to contextual changes, such as:

- ​Regulatory compliance: Restricting sensing operations in protected areas (e.g., military zones, privacy-sensitive regions).

- ​Commercial constraints: Enabling paid services within specific areas (e.g., premium real-time mapping in industrial campuses).

- ​Dynamic revocation: Terminating sensing activities upon policy violations (e.g., unauthorized data collection) or emergency events.

Service Area Authorization Requirements
To fulfill SA1-defined use cases (e.g., intruder detection in restricted UAV zones), this Key Issue investigates:

- ​Policy acquisition and distribution:

- How to acquire and store allowed/forbidden area definitions from operator policy systems, third-party regulators or AFs.

- How to distribute service area information to core network functions (e.g., AMF, SMF) and sensing devices (transmitters/receivers) via standardized interfaces.

- ​Request validation against service areas:

- How to validate sensing requests by verifying target/sensor locations against authorized areas.

- ​Enforcement and revocation mechanisms:

- How to enforce restrictions on transmitters and receivers.

NOTE: Security aspects will be coordinated with SA3.

Thus, it is proposed to agree the following changes to 3GPP TR 23.700-14.

2. Proposal

Start of Changes

## 5.2	Key Issue #2: Authorization and Revocation to Support Sensing Service

The key issue is to address the authorization and revocation for a particular sensing service, specifically:

-	authorization of sensing service requests and revocation of an on-going sensing service; and

-	authorization and revocation for the sensing entities if applicable.

For the authorization of the sensing service request, and the revocation of an on-going sensing service, the study needs to address the following aspects:

-	whether and how to acquire and store authorization information (e.g. allowed/forbidden area) from operator or third-party for regulatory or commercial use cases.

-	whether and how to perform sensing service request authorization if required, and the criteria for the sensing service request authorization, based on the requirements from TS 22.137 [2];

-	whether and how to perform sensing service revocation if required, and the criteria for the revocation, based on the requirements from TS 22.137 [2];

-	whether, how and which entity(/entities) performs the sensing service request authorization and sensing service revocation if required;

-	whether and where to store the sensing service request authorization information and sensing service revocation information, and when and how to update the authorization and revocation information if required.

For the authorization and revocation for the sensing entities if applicable, the study needs to address the following aspects:

-	whether, when and which entity performs the authorization and revocation of the sensing entity(/entities);

-	whether and how to retrieve and distribute authorization information (e.g. allowed/forbidden areas) to sensing entities

-	whether and how to enforce service area restrictions on sensing entities;

-	if needed, the granularity of the sensing entity authorization, e.g. based on TS 22.137 [2];

-	if needed, storage and management of the authorization information for the sensing entities.

NOTE:	The authorization and revocation for the sensing entities can be related to the key issue #3 on sensing entities discovery and selection.

End of Changes