3GPP TSG-SA2 Meeting #169	S2-2504681

Fukuoka, Japan, May 19 – May 23, 2025

# Title:	[DRAFT] Reply LS on RTP retransmission

# Response to:	S2-2504566 / S4-250739

# Release:	Rel-19

# Work Item:	5G_RTP_Ph2, XRM_Ph2

Source:	InterDigital [to be SA2]

To:	SA4

Cc:	RAN2

Contact Person:	Michael Starsinic

Michael.starsinic@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

- Overall description

SA2 thanks SA4 for their LS on RTP retransmissions. In the LS, SA4 asked the following question of SA2:

Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC.

SA4 also provided the following information.

- In the LS, SA4 stated “According to RFC 4588, source and retransmission PDUs associated to a media flow are transmitted in separate RTP streams”.

- In the LS, SA4 stated “Retransmission PDUs have stricter delay requirements to ensure meeting the decoding deadlines of corresponding ADUs at the receiver”.

SA2 discussed the matter and would like to provide the following comments.

- If two types of traffic (i.e. source flows and retransmission flows) have different delay requirements, then both types of traffic should be mapped to one QoS flow whose PDB will fulfill the most strict delay requirement (i.e. the PDB of the retransmission flow) OR the flows should be mapped to different QoS flows.

- It is preferred that the traffic be mapped to different QoS flows to avoid overprovisioning the level of service for the flow with the less strict delay requirement (i.e. the source flow). Thus, there is benefit to mapping source flows and retransmission flows to different QoS flows.

- It is already possible to map RTP streams to different QoS flows. SA2 would like to refer SA4 to the procedure for Setting up an AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502. The flow description information can include an SSRC value. Please also see clause 6.1.3.27.7 of TS 23.503 and clause 5.7.6.2 of TS 23.501.

- Thus, it seems that SSRC is already sufficient information for mapping source flows and retransmission flows to different QoS flows.

- Actions

To SA4:

ACTION: 	SA2 kindly asks SA4 to consider the information above.

# 3	Dates of next TSG SA meetings

3GPP TSG SA2#170	  	25th August – 29th August, 2025		      Goteborg, SE

3GPP TSG SA2#171	             13th October – 17th October, 2025	      China