3GPP TSG-SA WG2 Meeting #169	S2-2504682

Fukuoka, Japan, May 19 – 23, 2025

Source:	vivo, MediaTek Inc., ZTE, OPPO

Title:	Solution for IMS optimization to reduce call setup time

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#2.

1. Discussion

In the context of NB-IoT (GEO) voice communication scenarios, ‌the dominant factor in call setup latency stems from SIP message transmission delays‌. The IMS signaling sequence typically involves ‌8 message exchanges‌ (INVITE → 100 Trying → 183 Progressing → PRACK → 200 OK → UPDATE → 200 OK → 180 Ringing), with the ‌aggregate SIP message payload‌ generally approximating ‌10 KB‌ in total size. This cumulative data transfer requirement significantly impacts voice call establishment processes over bandwidth-constrained NB-IoT networks.

To improve IMS signaling efficiency in NB-IoT environments, the following optimizations are proposed:

‌1. Signaling Reduction via Precondition Disabling‌

- Disable the precondition feature (RFC 4032) to eliminate precondition-related message exchanges.

‌2. Minimalist Message Composition‌

- Enforce strict compliance with RFC 3261 (SIP) and RFC 4566 (SDP) requirements during INVITE procedures, mandating that UEs and P-CSCFs include ‌only essential headers and parameters‌.

3. ‌Context-Aware Information Reuse‌

- Leverage the P-CSCF’s ability to cache UE-specific data (e.g., contact, hostname, etc.) acquired during the IMS REGISTER phase. This eliminates redundant data transmission in subsequent INVITE messages.

- Prevent duplicate information across different part of the SIP message (e.g., avoiding To header redundancy when already specified in Request-URI).

4. ‌Compression Enhancement‌

- Deploy Signaling Compression (SigComp) between the UE and P-CSCF to reduce SIP message payload sizes‌, mitigating transmission overhead in bandwidth-constrained NB-IoT links.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	RFC 3261: "SIP: Session Initiation Protocol".

[y]	RFC 4566: "SDP: Session Description Protocol".

[z]	RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)"

* * * Second of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * Third of Change * * * *

6.X	Solution #X: Reduce SIP message size and number during INVITE procedure

### 6.X.1	High level principles

In the context of NB-IoT (GEO) voice communication scenarios, to improve IMS signaling efficiency, the following optimizations are proposed in this solution:

‌1. Signaling Reduction via Precondition Disabling‌

- Disable the precondition feature to eliminate precondition-related message exchanges.

‌2. Minimalist Message Composition‌

- Mandate that UEs and P-CSCFs include ‌only essential headers and parameters‌ sent to each other during INVITE procedures complying with RFC 3261 [x] and RFC 4566 [y].

3. ‌Context-Aware Information Reuse‌

- P-CSCF caches UE-specific data obtained during the IMS REGISTER phase to eliminate redundant data transmission in subsequent INVITE messages.

- Prevent duplicate information across different part of the SIP message.

4. ‌Compression Enhancement‌

- Deploy signalling compression (SigComp) as specified in RFC 5049 [z] on the UE and P-CSCF.

The solution does not presume whether an EPS bearer for voice data is pre-established or not, as EPS-related aspects will be addressed by resolutions for Key Issue #1. If the EPS bearer for voice data is not pre-established, a B2BUA becomes necessary to facilitate SIP transaction mediation with remote endpoints, and two options are available:

Option 1: P-CSCF acts as B2BUA (c.f. clause 6.X.3.2 for MO and 6.X.3.3 for MT).

Option 2: IMS AS acts as B2BUA (c.f. clause 6.X.3.4 for MO and 6.X.3.5 for MT).

### 6.X.2	Description

This solution address Key Issue #2: "IMS Enhancements for GEO NB-IoT NTN Access".

According to RFC 3261 [x] clause 20 and 20.1, following header fields are mandatory for SIP messages considering UDP transport protocol is used:

| Header field | where | ACK | BYE | CAN | INV | OPT | REG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Call-ID (i) | c(1) | m | m | m | m | m | m |
| Contact (m) | R | o | - | - | m | o | o |
| Contact (m) | 2xx | - | - | - | m | o | o |
| Content-Type (c) (NOTE) |  | * | * | - | * | * | * |
| CSeq | c(1) | m | m | m | m | m | m |
| From (f) | c(1) | m | m | m | m | m | m |
| Max-Forwards | R | m | m | m | m | m | m |
| To (t) | c(2) | m | m | m | m | m | m |
| Via (v) | R | m | m | m | m | m | m |
| Via (v) | rc(1) | m | m | m | m | m | m |
| (1): header field is copied from the request to the response. (2): copied from the request to the response with possible addition of tag. |  |  |  |  |  |  |  |

Some header fields (e.g., Allow, Supported) are recommended to be included in SIP messages related to call setup (e.g., INVITE request, 2xx response), but in voice over NBIoT (GEO) scenario, the minial parameters of those header fields can be required to avoid negotiation of them.

The Content-Length header field is required when stream-based protocol (such as TCP) is used as a transport, this solution propose to use UDP as transport to avoid this mandation.

According to RFC 4566 [y], following type lines are mandatory for SDP:

v=  (protocol version)

o=  (originator and session identifier)

s=  (session name)

c=  (connection information)

t=  (time the session is active)

m=  (media name and transport address)

a=* (zero or more media attribute lines)

NOTE:	Although "a=" line is optional, but it is necessary for audio media type. The "c=" line is included in all media or at session level.

According to RFC 3261 [x]: "initial Request-URI of the message SHOULD be set to the value of the URI in the To field", UE does not provide duplicated information in the To header field if already provided in Request-URI.

### 6.X.3	Procedures

6.X.3.1	Registration

The following figure depicts the IMS registration procedure in the NB-IoT (GEO) scenario.

Figure 6.X.3.1-1: IMS registration procedure

1.	If RAT type is NBIoT (GEO), the UE sends SIP REGISTER to the P-CSCF, wherein sigcomp is indicated and the information already provided in Request-URI is not provided in To header field.

REGISTER tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060;comp=sigcomp;sigcomp-id:"urn:uuid:0C67446E-F1A1-11D9-94D3-000A95A0E128";branch=z9hG4bKnashd8

f:<sip:+8613512345678@vs.ims.mno.3g.org>;tag=192830

t:<sip:A>;tag=201563

i:a84b

CSeq:31 REGISTER

Max-Forwards:70

Authorization:Digest username="13512345678",realm="vs.ims.mno.3g.org",uri="sip:+8613512345678@vs.ims.mno.3g.org",response="",nonce=""

If RAT type of the UE is NBIoT (GEO), the P-CSCF restore value of To header field according to Request-URI (i.e., replase "A" with "tel: +8613512345678") when forwards the SIP REGISTER to the IMS core.

2.	The P-CSCF receives 401 Unauthenticated message from IMS core, it sends 401 Unauthenticated message to the UE. If RAT type is NBIoT (GEO), the P-CSCF indicates sigcomp can be used to the UE.

SIP/2.0 401

v:SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060;comp=sigcomp;sigcomp-id:"urn:uuid:0C67446E-F1A1-11D9-94D3-000A95A0E128";branch=z9hG4bKnashd8

f:<sip:+8613512345678@vs.ims.mno.3g.org>;tag=192830

t:<sip:A>;tag=201563

i:a84b

CSeq:31 REGISTER

WWW-Authenticate:Digest realm="vs.ims.mno.3g.org",nonce="qlwqVapVqlWqVapVqlWqVUUQA5HEt9VVZ3t1TM221cg=",algorithm=AKAv1-MD5

3.	The UE sends SIP REGISTER to the P-CSCF, which includes Contact and P-Access-Network-Info header fields. The SIP REGISTER and subsequent SIP messages are transferred in secure way (e.g., over IPSec)

REGISTER tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060;branch=z9hG4bKnashd9

f:<sip:+8613512345678@vs.ims.mno.3g.org>;tag=192830

t:<sip:A>;tag=201563

i:a84b

CSeq:32 REGISTER

Max-Forwards:70

m:<sip:+8613512345678@vs.ims.mno.3g.org>,q=0.7;expires=3600

P-Access-Network-Info:3GPP-NB-IoT(GEO);utran-cell-id-3gpp=C359A

Authorization:Digest username="13512345678",realm="vs.ims.mno.3g.org",uri="sip:+8613512345678@vs.ims.mno.3g.org",response="a3f549b13f477562f4445b7277cd83c1",nonce="qlwqVapVqlWqVapVqlWqVUUQA5HEt9VVZ3t1TM221cg=",algorithm=AKAv1-MD5

The UE sends P-Access-Network-Info (PANI) in the SIP REGISTER. If RAT type of the UE is NBIoT (GEO), the P-CSCF stores information in Via, From, and Contact header field. Considering the serving satellite and associated eNB will not change, the P-CSCF also stores information in PANI.

4.	The P-CSCF receives 200 OK message from IMS core, it sends 200 OK message to the UE.

During the registration procedure, the IMS AS is aware of the UE's access network information according to the PANI header field.

6.X.3.2	Mobile originating, P-CSCF as B2BUA if EPS bearer not pre-established

The following figure depicts the mobile originating IMS INVITE procedure in the NB-IoT (GEO) scenario. If EPS bearer for voice data needs to be activated during the INVITE procedure, the P-CSCF acts as B2BUA.

Figure 6.X.3.2-1: Mobile originating, P-CSCF as B2BUA if EPS bearer not pre-established

0.	The UE has registered in IMS as described in clause 6.X.3.1. The user dials number to initiate a voice call.

1.	The UE may perform Service Request procedure to transit from RRC-IDLE state to RRC-CONNECTED state. EPS bearer for voice data may be activated during the Service Request procedure.

2.	If RAT type is NBIoT (GEO), the UE sends SIP INVITE with SDP offer that only indicates supported codecs for GEO voice to the P-CSCF, wherein the information already provided in Request-URI is not provided in To header field and information already provided during registration procedure is not included.

INVITE tel:+8613587654321 SIP/2.0

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

m:<sip:A>

i:a9gH

CSeq:81 INVITE

Max-Forwards:70

c:application/sdp

l:120

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.1.100

m=audio 49155 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

The P-CSCF sends SIP 100 to the UE.

SIP/2.0 100

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

Editor’s note: Whether SIP 100 between UE and P-CSCF can be omitted is FFS.

3.	If RAT type is NBIoT (GEO), the P-CSCF instructs the AGW to allocate audio resource for network side as well as allocate and configure audio resource for UE side according to the SDP offer.

4.	The P-CSCF restores information according to stored information during registration procedure and Request-URI of the INVITE message, and forwards the modified SIP INVITE with SDP offer to IMS core. The SDP offer contains audio resource information allocated by AGW for network side. The P-CSCF may act as B2BUA to add indication of supporting 100rel and precondition in the modified SIP INVITE request (e.g., if selected solution for KI#1 requires EPS bearer to be activated during call setup).

INVITE tel:+8613587654321 SIP/2.0

v: SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060; branch=z9hG4bK886431UKf234t

v: SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060; branch=z9hG4bK901butKNw56ub

f: <sip:+8613512345678@vs.ims.mno.3g.org>; tag=a2Xwhg47uT90

t: <sip: tel:+8613587654321>; tag=702Krb82gIkv

m: <sip:+8613512345678@vs.ims.mno.3g.org>

i: 9byT8XUwhL5k

CSeq: 81962507 INVITE

Max-Forwards: 69

P-Access-Network-Info: 3GPP-NB-IoT(GEO); utran-cell-id-3gpp=C359A

Support: 100rel, precondition

c: application/sdp

l: 345

v=0

o=- 94567123 89561264 IN IP4 10.47.16.5

s=-

c=IN IP4 192.168.2.100

m=audio 51893 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

a=cur:qoslocal none

a=cur:qosremote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

5.	After step 3, the P-CSCF uses the resource information for UE side to trigger the EPS procedure for bearer configuration (e.g., TFT configuration in PGW). This step may require EPS bearer activation.

NOTE 1:	This step is addressed by solution for Key Issue #1.

6-8.	The P-CSCF may receive SIP 183 Progressing with SDP answer from IMS core. The P-CSCF delegates UE to send SIP PRACK to IMS core, and receives SIP 200 OK (PRACK) from IMS core.

9-10.	The P-CSCF may send SIP UPDATE with SDP offer to IMS core, which may be after receiving notification from PCRF that the bearer resource is ready, then receives SIP 200 OK (UPDATE) from IMS core.

11.	The P-CSCF receives SIP 180 Ringing from IMS core. The P-CSCF sends SIP 180 to the UE.

SIP/2.0 180

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

When the UE receives the SIP 180, it plays ring tone.

12.	The P-CSCF receives SIP 200 OK from IMS core. The P-CSCF sends SIP 200 with SDP answer to the UE, and the SDP answer contains the audio resource information allocated by the AGW for UE side.

SIP/2.0 200

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

m:<tel:+8613587654321>

i:a9gH

CSeq:81 INVITE

c:application/sdp

l:98

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 93524 RTP/AVP 0

t=0 0

a=rtpmap:0 PCMU/6000

a=ptime:80

13.	The UE sends SIP ACK to the P-CSCF, wherein the information already provided in Request-URI is not provided in To header field and information already provided during registration procedure is not included.

ACK tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP A;branch=z9hG4bK654321

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:54 ACK

Max-Forwards:70

14.	The P-CSCF restores information according to stored information during registration procedure and Request-URI of the ACK message, and forwards the modified SIP ACK to IMS core.

The total SIP messages size (including "\r\n") that impact call setup time before compression during call setup is about 335B (INVITE) + 117B (100) + 117B (180) = 569B, applying sigcomp (processed by toolkit of Doubangou + zlib 1.2.3, compression rate is around 55%), the total SIP message size after compression is about 569 * 55% ≈ 310B.

6.X.3.3	Mobile terminating, P-CSCF as B2BUA if EPS bearer not pre-established

The following figure depicts the mobile terminating IMS INVITE procedure in the NB-IoT (GEO) scenario. If EPS bearer for voice data needs to be activated during the INVITE procedure, the P-CSCF acts as B2BUA.

Figure 6.X.3.3-1: Mobile terminating, P-CSCF as B2BUA if EPS bearer not pre-established

1.	The P-CSCF receives SIP INVITE with SDP offer from IMS core, which may indicate support of precondition.

2.	The P-CSCF instructs the AGW to allocate audio resource for UE side.

3.	If RAT type is NBIoT (GEO), the P-CSCF sends a SIP INVITE without precondition and with SDP offer that only indicates supported codecs for GEO voice to the UE, wherein the information already provided in Request-URI is not provided in To header field. The SDP offer contains resource information allocated by the AGW for UE side.

INVITE tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

m:<sip:+8613587654321@vs.ims.mno.3g.org>

i:a9gH

CSeq:81 INVITE

Max-Forwards:70

c:application/sdp

l:120

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 93524 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

The SIP INVITE may trigger EPS to perform network triggered service request procedure towards the UE, and EPS bearer for voice data may be activated during the procedure.

4.	The UE sends SIP 183 with SDP answer to the P-CSCF.

SIP/2.0 183

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

c:application/sdp

l:98

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 49155 RTP/AVP 0

t=0 0

a=rtpmap:0 PCMU/6000

5.	The P-CSCF instructs the AGW to configure resources for UE side according to the SDP answer, as well as allocate and configure resources for network side.

6.	The P-CSCF uses the resource information for UE side to trigger the EPS procedure for bearer configuration (e.g., TFT configuration in PGW). This step may require EPS bearer activation.

NOTE:	This step is addressed by solution for Key Issue #1.

7.	The sends SIP 183 Progressing to IMS core. The P-CSCF may act as B2BUA to add indication of requiring 100rel and precondition in the SIP 183 response (e.g., if selected solution for KI#1 requires EPS bearer to be activated during call setup).

8.	The P-CSCF may receive SIP PRACK from IMS core, then sends SIP 200 OK (PRACK) to IMS core.

9.	If the P-CSCF receives SIP UPDATE from IMS core, the P-CSCF sends SIP 200 OK (UPDATE) to IMS core, which may be after receiving notification from PCRF that the bearer resource is ready.

10.	The UE sends SIP 180 to the P-CSCF and alerts the user.

SIP/2.0 180

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

The P-CSCF sends SIP 180 Ringing to IMS core.

11.	The user answers the call, the UE sends SIP 200 to the P-CSCF.

SIP/2.0 200

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

m:<sip:A>

The P-CSCF sends SIP 200 OK to IMS core, wherein the Contact header field is set according to the stored Contact header field during IMS registration procedure. If the P-CSCF has not send SDP answer to IMS core, it includes SDP answer in the SIP 200 OK towards IMS core.

12.	The P-CSCF receives SIP ACK from IMS core. The P-CSCF sends SIP ACK to the UE.

ACK tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK098765

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:17 ACK

Max-Forwards:70

6.X.3.4	Mobile originating with IMS AS acting as B2BUA

The following figure depicts the mobile originating IMS INVITE procedure in the NB-IoT (GEO) scenario.

The IMS AS acts as B2BUA assuming the selected solution for KI#1 requires EPS bearer for voice data to be activated during the INVITE procedure.

Figure 6.X.3.4-1: Mobile originating with IMS AS acting as B2BUA

0-3.	Same as described in steps 0-3 of clause 6.X.3.2 with difference that the EPS bearer for voice data is not activated during the Service Request procedure.

4.	Same as described in step 4 of clause 6.X.3.2 with difference that the P-CSCF does not add "Supported: 100rel, precondition", which will be added by IMS AS who acts as B2BUA according to the access network information received during IMS registration procedure.

5-8.	Same as described in steps 5-8 of clause 6.X.3.2 with difference that EPS bearer for voice data is activated in step 5, so the IMS AS acts as B2BUA to interact with terminating network side.

9-10.	When notification of bearer resource ready is received from PCRF, the P-CSCF sends a SIP MESSAGE to IMS AS to indicate voice resource is ready. This SIP message contains the IP address allocated in AGW to be used by the terminating network. The IMS AS sends SIP UPDATE with SDP offer to terminating network side after receiving the SIP MESSAGE, then receives SIP 200 OK (UPDATE) from terminating network side.

11-14.	Same as described in steps 11-14 of clause 6.X.3.2.

6.X.3.5	Mobile terminating with IMS AS acting as B2BUA

The following figure depicts the mobile terminating IMS INVITE procedure in the NB-IoT (GEO) scenario.

The IMS AS acts as B2BUA assuming the selected solution for KI#1 requires EPS bearer for voice data to be activated during the INVITE procedure.

Figure 6.X.3.5-1: Mobile terminating with IMS AS acting as B2BUA

1-6.	Same as described in steps 1-6 of clause 6.X.3.3 with the differences that the IMS AS does not indicate precondition in the SIP INVITE sent towards the UE in step 1-b, and the EPS bearer for voice data is not activated in step 3.

7.	The P-CSCF sends SIP 183 Progressing for the SIP INVITE in step 1-c to the IMS AS. The IMS AS acts as B2BUA according to the access network information received during IMS registration procedure, and sends SIP 183 Progressing with indication of requiring 100rel and precondition to terminating network side.

8.	The IMS AS receives SIP PRACK from terminating network side, then sends SIP 200 OK (PRACK) to terminating network side.

9.	When notification of bearer resource ready is received from PCRF, the P-CSCF sends a SIP MESSAGE to IMS AS to indicate voice resource is ready. This SIP message contains the IP address allocated in AGW to be used by the terminating network. After receiving the SIP MESSAGE and the SIP UPDATE from terminating network side, the IMS AS sends SIP 200 OK (UPDATE) to terminating network side.

10-12.	Same as described in steps 10-12 of clause 6.X.3.3.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	Further impact is FFS.

UE:

-	Support not providing duplicated information in a SIP message that was already sent during registration procedure or is already provided in another place of the SIP message.

-	Support sigcomp.

P-CSCF:

-	Support restore information for a SIP message if stored during registration procedure or if already provided in another place of the SIP message.

-	May support sending SIP MESSAGE to IMS AS to notify voice resource ready, or may support B2BUA.

-	Support sigcomp.

IMS AS:

-	May support receiving SIP MESSAGE from P-CSCF for notifying voice resource ready.

* * * End of Change * * * *