3GPP TSG-SA WG2 Meeting #169	S2-2504683

# Fukuoka, Japan, May 19 - 23, 2025											 (revision of S2-2503583)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1629 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on early media handling in UE-Satellite-UE scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In AE.5.2 “IMS AGW relocation and media routing path change due to change of satellites”, step 4 describes that the P-CSCF requests IMS AGW on the target satellite to configure and reserve the IP address for the IMS session but does not release the IMS AGW on the source satellite. In step 12, both the IMS AGW on target and source satellites are used to transfer media data. Until step 22 after the SDP re-negotiation has been done, the P-CSCF releases the IMS AGW on source satellite. Considering the P-CSCF supporting satellite change already needs to be able to manage two IMS AGW in a period, this capability also can be applied to early media handling, i.e., MO P-CSCF and MT P-CSCF can negotiate resources of AGWs prior to call ringing and no re-negotiation after call answered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update AE.5.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user experience on early media service is not good. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change ---

## AE.5.3	At call setup with early media

During the call setup with both originating and terminating UEs using regenerative NR satellite access, the originating P-CSCF uses the IMS-AGW on ground to transfer early media then, after the IMS session is answered, releases the IMS-AGW on ground and selects the IMS-AGW on satellite to transfer regular media. See Figure AE.5.3-1 that describes the procedure.

In this procedure, the IMS AS for early media uses existing method for providing early media and updating the IMS session when answered.

Figure AE.5.3-1: IMS session establishment procedure with early media

1-11.	Steps 1-11 of clause AE.5.1 apply with the difference that the IMS AS serving early media allocates early media resources with MRF, as well as includes the P-Early-Media header field and a modified SDP answer for early media in the 18X response.

The IMS AS may instruct the MRF to play early media any time before step 12.

12.	Based on the identification of the satellite serving UE B and the P-Early-Media header field in step 11, P-CSCF A determines to perform UE-satellite-UE communication with early media. P-CSCF A selects an IMS-AGW A on satellite but does not release the IMS-AGW A on ground in order to use it for early media.

12a-12b.	P-CSCF A interacts with IMS-AGW A on ground for early media to allocate and configure transport resources for IMS access side as well as configure transport resources for IMS core network side.

13.` step 13 of clause AE.5.1 applies.

The early media is able to be sent to the UE A via the IMS AGW A on ground, the UPF A on ground and the UPF A on satellite.

14.	Step 14 of clause AE.5.1 applies with the difference that P-CSCF A includes in the SIP 18X response an SDP answer containing the transport address of IMS-AGW A on ground obtained in step 12.

15.	Steps 15 of clause AE.5.1 applies.

16.	The P-CSCF A interacts with the IMS-AGW A on satellite selected in step 12 for regular media to allocate and configure transport resources for both IMS access side and IMS core network side.

17.	Step 16 and 18 of clause AE.5.1 apply, in which the SDP offer is for regular media.

18-19.	Steps 17and 19 of clause AE.5.1 apply and UE B is ringing.

20.	The UE B answers the IMS session and sends SIP 200 response for the INVITE.

21.	The IMS AS serving early media instructs the MRF to stop playing early media.

22.	The IMS AS serving early media forwards the SIP 200 response towards originating side.

23.	The P-CSCF A releases the IMS AGW A on ground.

24.	The P-CSCF sends the SIP 200 response with the SDP offer for regular media containing the transport address of IMS-AGW A on satellite obtained in step 16.

24.	The P-CSCF sends the SIP 200 response with the SDP offer for regular media containing the transport address of IMS-AGW A on satellite obtained in step 16.

After receiving the SIP 200 response for the INVITE, the UE A is able to send regular media and is ready to receive regular media. The regular media uses UE-satellite-UE communication path.

25.	The UE A sends SIP ACK with an SDP answer to the P-CSCF A. The P-CSCF A forwards the SIP ACK without the SDP answer towards terminating side.

After receiving the SIP ACK, the UE B is able to send regular media.

## ---End of the Change---