3GPP TSG-WG SA2 Meeting #169	S2-2504695

# Fukuoka, Japan, May 19 – May 23, 2025

Source: 	vivo

Title: 	KI#1, new solution: Energy consumption calculation enhancements

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose the new solution for KI#1 for TR 23.700-67.

1. Introduction

This contribution proposes a new solution for KI#1 for the r20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| x | x |  |  |

SECOND CHANGE

6.x	Solution # x: Energy consumption calculation enhancements

6.X.0	High-level solution principles

The principles of this solution are to enhance the energy consumption information mathematical approach defined in Rel-19 to reduce impact of the energy consumption for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring, the energy consumption when the node is in idle/inactive state). The aim of the solution is to improve the accuracy and robustness of the energy consumption calculation in EIF. This solution proposes to calculate the per bit energy consumption for the measurement time interval by using the total energy consumption and total data volume information within the two consecutive measurement time intervals.

The ratio of the energy consumption difference to the total data volume difference between the two consecutive time intervals are considered as the energy consumed per bit data transmission and processing at the node within the time interval.

6.x.1	Description

This solution applies to Key Issue #1.

In the release 19 features, the calculation formulas for energy consumption in EIF was proposed. The details of the calculation model are captured in Annex T in TS23.501[2].

In the release 19 calculation formulas, it is based on the transmitted data volume information at the related granularity and the coefficients of per bit energy consumption at the node. For example, the energy consumption at UE level at the UPF over a time interval T is calculated as follows:

EUE,UPF=EUPFDVUPFDVUE,UPF (1)

The main principle of this calculation is that  EUPFDVUPF  is coefficient that is common across all granularities, within the time interval T, for the network entity UPF and is the value that related to the energy consumption per bit at UPF.

The main issue of the current Data volume based calculation formula (1) is that the coefficient of energy consumption per bit EUPFDVUPF is not accurate and robust.

EUPF , which is obtained from OAM, is the total energy consumed at the UPF within the time interval T, which includes the following two parts:

- the energy consumed for data processing/transmission related tasks; and

- the energy consumed for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring).

Regards to energy consumption for part (a), it is closely related with the data processing/transition at the node. So the energy consumption for part (a) would be proportional to the processed/transmitted data volume DVUPF at the node.

Regards to energy consumption for part (b), it is related with the energy consumption when the node is at idle/inactive state and also related with some functions for basic operation of the node. This energy consumption for part (b) may increase when the load of the node increases, but can not be proportional to the processed/transmitted data volume DVUPF at the node. So the relationship of the total energy consumption EUPF and total data volume DVUPF at the node UPF is similar to the following curve shown in Figure 6.x.1-1.

Figure 6.x.1-1 Example for the relationship of total energy consumption and total data volume at the node

Considering the fact that the EUPF includes two parts and can not be proportional to the processed/transmitted data volume DVUPF at the node UPF, it is not suitable to use the coefficient EUPFDVUPF  as the energy consumption per bit at the node UPF. In this case, this coefficient EUPFDVUPF  is not robust and accurate enough to reflect or evaluate the energy consumed per bit data transmission and processing at the node UPF.

In order to make the data volume based calculation formulas more accurate and robust, it is proposed to enhance the calculation of the energy consumed per bit data transmission at the node UPF over the time interval T.

Let’s take the energy consumption at UPF for example. It is proposed to measure at least two consecutive time interval T1, T2 at the node UPF, then the EIF could get from OAM the total energy consumption and data volume information at UPF over the two time interval T1 and T2:

For time interval T1: the total energy consumption EUPF,T1 and data volume DVUPF, T1 at UPF over T1

For time interval T2: the total energy consumption EUPF,T2 and data volume DVUPF, T2 at UPF over T2

Then, the EIF calculates the energy consumed per bit at UPF for time interval T1 as following:

For time interval T1: EUPF, per bit, T1=EUPF,T1-EUPF,T2DVUPF,T1-DVUPF,T2  (2)

In this formula (2), EUPF,T1-EUPF,T2 is the difference of the energy consumption at UPF between the two consecutive time interval T1 and T2. Considering the time intervals T1 and T2 are consecutive, the energy consumption for part(2) in EUPF,T1 and EUPF,T2 are very close and could be considered as the same. So the energy consumption difference over the two time interval T1 and T2, i.e., EUPF,T1-EUPF,T2 , is due to or corresponds to the difference of the total data volume processed and transmitted at the UPF over the time interval T1 and T2, i.e., (DVUPF,T1-DVUPF,T2). In this case, the ratio of (EUPF,T1-EUPF,T2) to (DVUPF,T1-DVUPF,T2) could reflect the energy consumed per bit data transmission and processing at UPF at the time interval T1.

Based on the above principles, the details of the calulation formulas are described in the following clause.

6.x.2	Procedures

It is proposed to enhance the calculation formulas based on the principles explained in formula (2). The EIF can obtain the total energy consumption information and data volume information at the node (e.g., UPF, gNB) over time interval T from OAM as defined in the procedure in clause 4.29.2 of TS 23.502[3].

In this solution, it is proposed that the ratio of (EUPF,T1-EUPF,T2) to (DVUPF,T1-DVUPF,T2) as defined in formula (2) could be considered as the coefficient of the energy consumed per bit data transmission and processing at UPF over the time interval T1.

So based on the above coefficient defined in formula (2), the energy consumption at UPF for certain granularities within time interval T1 could be defined as following:

EUPF, UE, T1=EUPF,T1-EUPF,T2DVUPF,T1-DVUPF,T2DVUPF,UE,T1 (3)

EUPF,S-NSSAI,T1=EUPF,T1-EUPF,T2DVUPF,T1-DVUPF,T2DVUPF,S-NSSAI,T1 (4)

E UPF,Session,T1=EUPF,T1-EUPF,T2DVUPF,T1-DVUPF,T2DVUPF,Session,T1 (5)

EUPF,Flow,T1=EUPF,T1-EUPF,T2DVUPF,T1-DVUPF,T2DVUPF,Flow,T1 (6)

Similarly, the energy consumption at gNB for certain granularities within time interval T1 could be defined as following:

EgNB,UE,T1=EgNB,T1-EgNB,T2DVgNB,T1-DVgNB,T2DVgNB,UE,T1  (7)

EgNB,S-NSSAI,T1=EgNB,T1-EgNB,T2DVgNB,T1-DVgNB,T2DVgNB,S-NSSAI,T1 (8)

EgNB,Session, T1=EgNB,T1-EgNB,T2DVgNB,T1-DVgNB,T2DVgNB,Session,T1 (9)

EgNB,Flow,T1=EgNB,T1-EgNB,T2DVgNB,T1-DVgNB,T2DVgNB,Flow,T1 (10)

So based on the formulas (3)-(10), it is proposed to derive the energy consumed in the network in the time interval T1 by a UE (EUE), S-NSSAI (ES-NSSAI), PDU Session (ESession), QoS flow (EFlow) by adding the energy consumption for the corresponding granularity at time interval T1 at all nodes for the data transmission and processing, as defined in annex T of TS 23.501[2].

For example, the network energy consumption for the UE level at time interval T1 at all the nodes are derived as following:

EUE=gNBi∈gNBUET1EUE, gNBi+ UPFi∈UPFUET1EUE, UPFi

where gNBUET1 are all gNBs used by the UE in an interval T1, UPFUET1 are all UPFs used by the UE in an interval T1.

Accordingly, the energy consumption at certain granularity, e.g., S-NSSAI (ES-NSSAI), PDU Session (ESession), QoS flow (EFlow) can also be derived.

6.x.3	Impacts on existing services, entities and interfaces

EIF:

-	Support the new formulas for energy consumption calculation of required granularities. These new formulas are the enhancements on the release 19 formulas.

End of CHANGES