3GPP TSG-WG SA2 Meeting #169	S2-2504765

Fukuoka, Japan, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, New Solution: Traffic Pattern Analytics Supporting Efficient Performance of UP

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: A new NWDAF Analytics, Traffic Pattern Analytics, is proposed to detect abnormal and malicious User Plane traffic to help user plane performance improvement.

1. Introduction/Discussion

This solution is to address use case #1: NWDAF provide traffic pattern analytics to mitigate the impact of such potential abnormal or malicious traffic pattern in the user plane and improve the user plane performance.

UPF must be robust and resilient against abnormal data traffic, for examples, large amount of abnormal or malicious IP packets, flows, bursts, syn-ack signalling to avoid UPF performance degradation. Since abnormal traffic pattern can vary a lot, to prevent UPF from the impact of abnormal traffic in user plane, NWDAF needs to capture the characteristics of the traffic patterns of abnormal traffic in a dynamic and efficient way. In the solution, we basically consider but not limited to the following features to detect User Plane anomaly:

- It may cause a large number of small data packets (e.g., SYN ACK) aimed to specific UEs when those UE addresses are used in e.g., SYN attack or reflection attack;

- Due to AS misconfiguration or flooding attacks, it may cause dramatic deviation source IP address distribution when many DL packets are from ASs that seldom communicate with the UEs in the network;

- It may cause small data packets are sent to a large number of UEs at the same time because of the incorrect configuration in the AS, or AS notifies new feature of the application to the UE.

- When the AS is compromised its abnormal traffic may have unusual SSL/TLS fingerprint.

Hence, based on knowing features of abnormal traffic and the analytic capability of NWDAF, it could be possible to identify dynamic abnormal traffic and mitigate the impact on UPF, and consequently reduce the load of N3 interface and Uu interface.

The proposed solution allows NWDAF to provide traffic pattern analytics to detect User Plane anomaly thus help to improve the user plane performance.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #x |  | x | x |  |

# * * * * Second change (all new text) * * * *

6.X	Solution #X: Traffic Pattern Analytics supporting efficient performance of UP

6.X.1	High level principles

The high-level principles of the solution are:

- The consumer of the traffic pattern analytics can be PCF, the consumer subscribes to the analytics based on the created SM policy.

- The NWDAF provides the analytics of specific packet filters, in the form of statistics or predictions or both, to the consumer NF. The NWDAF collects the traffic pattern related input data from AF and/or UPF.

- The PCF generates or updates the PCC rules based on the analytics result to indicate the UPF to drop specific packets or degrade the QoS level of the traffic for the specific time period.

- The UPF processes the traffic with the created/updated N4 rules.

6.X.2	Description

NWDAF provides traffic pattern analytics to detect User Plane anomaly thus help to improve the user plane performance.

The service consumer may be a PCF.

The consumer of this analytics may indicate the following information in the request or subscription:

-	Analytics ID = "Traffic Pattern".

-	Target of Analytics Reporting: any UE.

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Application identifier;

-	Packet filter set as defined in clause 5.7.6 of TS 23.501 [2];

-	Area of Interest.

-	optionally, Preferred level of accuracy of the analytics.

-	in a subscription, Notification Target Address (+ Notification Correlation ID).

The following input data may be used by the NWDAF for generating the analytics.

Table 6.X.2-1: Input data from AF related to traffic pattern

| Information | Source | Description |
| --- | --- | --- |
| UE address distribution | AF | For a specific application, the UE address has communicated with the application. |
| Normal SSL/TLS fingerprint (NOTE 1) | AF | SSL/TLS fingerprints of normal traffic flow communicated with the AS. |
| Average and variance of the number of UE communicated with the AS | AF | The average and variance of the number of UE communicated with the same AS in a time window. |
| Traffic pattern Information (1…max) | AF | Indicates the traffic characteristics of different traffic patterns. |
| >Traffic flow filter information |  | Identifies the traffic flow from the DN, e.g. application identifier, or packet filter set as defined in TS 23.501 [2]. |
| >Communication life-time |  | The average duration of the communication. |
| >UL data rate |  | UL data rate of this communication. |
| >DL data rate |  | DL data rate of this communication. |
| >Maximum burst size |  | Maximum data burst volume of the traffic flow. |
| >burst periodicity |  | Average burst periodicity of the traffic flow. |
| NOTE 1: Alternatively, NWDAF can have such information by configuration. |  |  |

Table 6.X.2-2: Input data from UPF related to specific traffic

| Information | Source | Description |
| --- | --- | --- |
| AS address distribution | UPF | The address distribution of the AS which have communicated with the UPF before. |
| Traffic Flow characteristic Information (1…max) | UPF | Traffic characteristics information related with specific traffic flow from DN. |
| > Traffic flow filter information | UPF | Identifies the traffic flow from the DN, e.g. application identifier, or packet filter set as defined in TS 23.501 [2]. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL/DL Data volume | UPF | Measured UL/DL data traffic volume for the flow. |
| > UL/DL Data rate | UPF | Measured UL/DL data rate for the flow. |
| > UL/DL/roundtrip packet delay | UPF | UL/DL/roundtrip packet delay generated by QoS monitoring as defined in clause 5.45 of TS 23.501 [2]. |
| Congestion information | UPF | congestion information (i.e. a percentage of congestion level) generated by QoS monitoring as defined in clause 5.45 of TS 23.501 [2]. |
| Syn-Ack signalling information | UPF | Measured syn-ack signalling characteristic of an application communicating with the UE, e.g. number of syn-ack packets in a period. |
| Detected SSL/TLS fingerprint | UPF | Calculated SSL/TLS fingerprint of an encrypted connection between the UE and application. |

Editor’s Note: Whether other inputs can also be considered by NWDAF is FFS.

The NWDAF shall be able to provide statistics as follows:

Table 6.X.2-3: Statistics for traffic pattern analytics

| Information | Description |
| --- | --- |
| Traffic pattern information (1…max) |  |
| > traffic filter information | Indicates the traffic filter of the traffic related with the input data from UPF, e.g. packet filter set, application identifier, etc. |
| > Traffic class | Indicates the traffic class of the traffic flow (e.g. normal traffic or abnormal traffic). |
| > traffic life time | Indicates the traffic life time of the traffic flow. |

The NWDAF shall be able to provide prediction as follows:

Table 6.X.2-4: Predictions for traffic pattern analytics

| Information | Description |
| --- | --- |
| Traffic pattern information (1…max) |  |
| > traffic filter information | Indicates the traffic filter of the traffic related with the input data from UPF. |
| > Traffic class | Indicates the traffic class of the traffic flow (e.g. normal traffic or abnormal traffic). |
| > traffic life time | Indicates the traffic life time of the traffic flow. |
| confidence | Confidence of the prediction. |

6.X.3	Procedures

Figure 6.x.3-1 shows the procedure for the NWDAF to provide the traffic pattern analytics to a consumer NF (i.e. PCF).

Figure 6.x.3-1: Procedure for Traffic Pattern Analytics

1. The PCF requests or subscribes to the traffic pattern analytics for specific traffic from NWDAF.

2a-2b. The NWDAF may subscribe to the traffic pattern data as defined in clause 6.x.2 from AF by invoking Naf_EventExposure_Subscribe service operation. The AF sends the collected data to the NWDAF by invoking Naf_EventExposure_Notify service operation. For the untrusted AF, the NWDAF obtains the related data via NEF.

3a-3b. The NWDAF may subscribe to the traffic flow information as defined in clause 6.x.2 from UPF by invoking the Nupf_EventExposure_Subscribe service operation. The UPF notifies the NWDAF the traffic flow information by invoking the Nupf_EventExposure_Notify service operation.

4-5. The NWDAF derives the analytics and notifies the analytics to the PCF.

6. Based on analytic result from NWDAF, PCF generates or updates the PCC rules which includes the filter of abnormal traffic and sends it to the SMF. The created or updated PCC rules may indicate to drop packet from the abnormal traffic filter (e.g. change the gate status of the abnormal traffic to close) or degrade the QoS level of the abnormal traffic filter (e.g. modify the 5QI or limit the DL-MBR of the abnormal traffic) during the traffic life time.

Editor’s Note: Whether other updates for PCC rule can also be done by PCF is FFS.

7. The SMF updates N4 rules accordingly, where PDR includes the filter of abnormal traffic and FAR indicates drop action or the QER indicates the gate is closed or a lower DL-MBR, and sends them to the UPF.

8. The UPF handles the traffic based on the received N4 rules, for example, drop the packets, forward the traffic with degraded QoS level.

6.X.4	Impacts on services, entities and interfaces

PCF:

-	Request for traffic pattern analytics from NWDAF.

-	Generate or update PCC rule(s) to indicate traffic dropping or degrade the QoS level of the traffic flow, based on analytic result from NWDAF.

NWDAF:

-	Collect traffic flow information, traffic pattern information from UPF and AF.

-	Derive the traffic pattern analytics and provide the analytics to the consumer.

UPF:

-	Provide the traffic flow information to NWDAF.

AF:

-	Provide traffic pattern information to NWDAF.

# * * * * End of changes * * * *