3GPP TSG-SA WG2 Meeting #169	S2-2504774

# 19-23 May 2025, Fukuoka, Japan	(Revision of S2-2502914)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1605 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Corrections to DC AS initiated Session Update to add BDC to an existing IMS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some corrections to DC AS initiated session update to add BDC to an existing IMS session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the parameters. Handle the addition of AF Identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AG.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AG.2.1.4	DC AS initiated bootstrap data channel addition to an existing IMS session

Figure AG.2.1.4-1 depicts the call flow for adding bootstrap data channel towards UE B to an existing IMS session. In this scenario, the MF-2 is used to anchor the bootstrap data channel between UE B and DCSF2.

Figure AG.2.1.4-1: Procedure of adding bootstrap data channel to an existing IMS session

An IMS voice session between UE A and UE B has been established.

0.	DC AS decides to add a bootstrap DC to the existing IMS session, which is notified based on the event notification of the related IMS session to which the DC AS subscribed to.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add a bootstrap data channel to the existing IMS session.

The DC AS includes the AF Identifier, the session ID of the existing IMS session to be updated, the operation type as Adding, media operation set including the parameters for the media type set as BDC, the UE B as the target of the BDC, in the Nnef_ImsSessionManagement_Update request. The DC AS may include a Notification Target Address and Correlation ID in the Nnef_ImsSessionManagement_Update request to be notified of the session update progress.

2.	If the NEF authorizes the request, the NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE B.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 to add a BDC to the specific IMS session. The received session ID of the existing IMS session, the operation type as Adding, the media type as BDC, the UE B as the target of the BDC, and the MDC1 media endpoint address shall be included in the request. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session update progress.

4.	The IMS AS-2 determines whether the BDC request should be notified to DCSF-2 based on subscription information and DC capability of UE B. If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, or then the request shall be rejected with appropriate cause and subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the bootstrap DC is already established, the request shall be rejected with an appropriate cause and the following steps are skipped.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from media operation set parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

7.	Step 7 to 10 in clause AC.7.1 applies.

8.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the bootstrap data channel and existing voice media and sends the request to UE B.

9.	UE B responds 200 OK with SDP answer.

10.	Step 17 to 18 in clause AC.7.1 applies.

11.	IMS AS sends ACK to UE B to complete the IMS session modification procedure.

12. If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in Step 3, the IMS AS-2 notifies the NEF MediaChangeSuccess event with the Nimsas_ImsSessionManagement_Notify request.

13. If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in Step 3, the NEF notifies the DS AS MediaChangeSuccess event with the Nnef_ImsSessionManagement_Notify request.

14-15.	The DC AS and NEF return responses.

# * * * End of changes * * * *