3GPP TSG-SA2 Meeting #169	S2-2504793

# Fukuoka, Japan 19 – 23 May, 2025

Agenda item:	19.49

Source:	China Telecom

Title:	SA2 specification impact on FS_MINT_Ph2

WI/Release:	Rel-19

Document for:	Discussion

1	Introduction

This paper provides the expected impact on SA2 specification based on the conclusion of FS_MINT_Ph2 (TR 24.812 v19.1.0).

2	Discussion

2.1	Overview of CT1 Study

CT1 has studied on MINT support in EPS for 5G-only national roaming UE based on the service requirements that SA1 had specified for Rel-19.

TR 24.812 v19.0.0 was approved in CT#107. The approved version (CP-250140) includes conclusions for Key Issues #1, 2, 3, 4, and 6.

In CT1#154, conclusions for the rest Key Issues were agreed and captured in TR 24.812 v19.1.0, which requires approval in CT#108 in June 2025.

In CT1#154, CT1 agreed to send an LS (S2-2504501/C1-252559) to kindly ask SA2 to take above information into consideration and to update their specifications accordingly.

2.2	CT1 conclusions and expected SA2 impact

Conclusions of the eight Key Issues in TR 24.812 and their corresponding SA2 impact are listed below. In addition, a general description of MINT_Ph2 feature should be added in TS 23.501 and TS 23.401.

Key Issue #1: Notification of disaster condition to the UE

- Conclusion:

Solution #1 for Key Issue #1 shall be progressed for normative work. The network:

-	informs the UE about disaster condition over 3GPP access; and

-	broadcasts minimal information over E-UTRAN of a 4G PLMN offering disaster roaming service to UEs of a 5G PLMN with disaster condition and the UE having 5G-only national roaming access to a VPLMN to obtain 4G connectivity service. The information broadcast is sufficient to enable the UEs to determine that a disaster condition applies to the 5G PLMN with disaster condition.

NOTE:	the design of the SIB messages with broadcast indication is defined by RAN WG2.

No further enhancement is needed for handling security risks resulting from using broadcast signalling to convey information related to disaster roaming.

- Expected SA2 impact:

None.

Key Issue #2: Indication of accessibility from EPS to the UE

- Conclusion:

Solution #2 for Key Issue #2 shall be progressed for normative work. The solution:

- The UE can be provisioned by the HPLMN of the UE with the information on whether the UE having 5G-only national roaming access to a 5G VPLMN can obtain disaster roaming service from a 4G VPLMN

- The network (e.g., HPLMN or VPLMN providing 5G-only national roaming access to the UE) may provide the "list of PLMN(s) providing 4G service to be used in disaster condition", to the UE during a successful attach, tracking area update procedure or during the registration procedure in 5GS. The "list of PLMN(s) providing 4G service to be used in disaster condition" may also be provided in the USIM or pre-configured in the non-volatile memory of the ME.

- The 4G VPLMN offering disaster roaming service shall indicate accessibility for disaster inbound roamers through SIB messages. The indication may contain the list of PLMN(s) with disaster condition for which disaster roaming service in EPS is offered.

NOTE:	The design of the SIB message structure to be defined by RAN WG2.

- Expected SA2 impact:

Update MME and AMF functionalities to provide the above information to UE during a successful attach, tracking area update procedure or during the registration procedure in 5GS.

Key Issue #3: Attach to the 4G VPLMN providing disaster roaming services in case of disaster condition

- Conclusion:

Solution #3 for Key Issue #3 which is able to distinguish attach request for normal service from the attach request for disaster roaming services shall be progressed for normative work.

When the UE having 5G-only national roaming access to a VPLMN to obtain 4G connectivity service under disaster conditions, performs an attach procedure in a 4G VPLMN which supports disaster roaming services, the UE indicates a new EPS attach type in the ATTACH REQUEST message to differentiate between normal attach and attach due to disaster roaming. The 4G VPLMN that provides disaster roaming service should be able to distinguish the attach request for normal service from the attach request for disaster roaming services. The 4G VPLMN that provides disaster roaming services determines a registration area for the UE such that the EPS tracking area list contains only those tracking area identities (TAIs) that cover the area of the disaster condition.

If the UE does not have a valid mapped 4G-GUTI assigned by the 5G PLMN with disaster condition and the 5G PLMN with disaster condition is not the HPLMN of the UE, in addition to the new EPS attach type, the UE also indicates the 5G PLMN with disaster condition in the ATTACH REQUEST message.

The higher priority PLMN search when the UE is registered for disaster roaming service which was introduced as part of Rel-17 MINT shall be reused as much as possible when the UE is registered for disaster roaming services in 4G PLMN.

In addition, when a disaster condition is detected on the 5G PLMN according to the conclusion of Key Issue #1, and the UE is registered in the 5G PLMN, the UE may initiate a tracking area update procedure to perform an inter-system change from 5GS to EPS of the PLMN for disaster roaming services. This tracking area update procedure may include a new EPS update type (e.g., "disaster roaming update") to indicate that the procedure is triggered by a disaster condition.

- Expected SA2 impact:

Update MME functionality to identify a new EPS attach type and a new EPS update type.

Update MME functionality to determine a registration area only covering the area of the disaster condition.

Key Issue #4: PLMN selection when disaster condition applies for the 5G VPLMN of 5G-only national roaming UE

- Conclusion:

Solution #4 for Key Issue #4 can be progressed for normative work. The solution:

When the 5G-only national roaming UE is unable to find an allowable PLMN and UE finds PLMN(s) that provide disaster services in EPS for the PLMN with disaster condition of the UE, then

- if the PLMN that provides disaster services in EPS is in the forbidden PLMN list of the UE, then UE ignores the forbidden condition and selects the PLMN providing disaster services in EPS; and

- if the above PLMN has the same PLMN ID as that of the UE determined PLMN with disaster condition, the UE may ignore any E-UTRAN restriction for this PLMN ID and select the PLMN /E-UTRAN combination by the following:

- if S1 Mode capability was disabled for the PLMN above, then S1 capability is enabled; and

- if RAT restriction for E-UTRAN is applicable for the PLMN above, then RAT restriction for E-UTRAN shall be ignored for the PLMN above; and

- if TAI of any cell broadcasting the PLMN ID of the PLMN above is in forbidden TAI list, then the TAI shall be removed from the forbidden TAI list.

Automatic PLMN selection in 3GPP TS 23.122 [r23122] subclause 4.4.3.1.1 is updated to enable the UE to select the PLMN and E-UTRAN access technology combination providing disaster roaming services by reusing steps vi and vii of 3GPP TS 23.122 [r23122] subclause 4.4.3.1.1.

- Expected SA2 impact:

None.

Key Issue #5: RAT restriction under disaster conditions

- Conclusion:

Solution #5 for Key Issue #5 shall be progressed for normative work.

For the case when the network and the UE support both the Enhancement of controlling access technology utilization feature (ECRATU) and the Minimization of Service Interruption defined in Rel-19 and a disaster condition occurs in a specific RAT (i.e., NG-RAN) while the UE is restricted in the other existing RAT(s) or supported RAT(s) (i.e., E-UTRAN), the solution indicates:

-	the UE ignores the Enhancement of controlling access technology utilization feature (ECRATU).

- Expected SA2 impact:

None.

Key Issue #6: Notification that disaster condition is no longer applicable to the UEs

- Conclusion:

Solution #6 for Key Issue #6 shall be progressed for normative work.

The UE registered for disaster roaming services in EPS determines that the disaster condition has ended on the PLMN providing 5GS services when:

-	UE has successfully registered over non-3GPP access on a PLMN;

-	the UE receives an indication from the MME of 4G VPLMN that disaster roaming has ended; or

-	the UE performing periodic scan determines that PLMN providing service in 5GS is available. The higher priority PLMN search when the UE is registered for disaster roaming service which was introduced as part of Rel-17 MINT shall be reused as much as possible when the UE is registered for disaster roaming services in 4G PLMN.

- Expected SA2 impact:

Update MME functionality to provide the indication that the disaster condition has ended in GUTI Reallocation message, tracking area update reject message, service reject message, or detach request message.

Key Issue #7: Prevention of signalling overload in the VPLMN providing disaster roaming services in EPS

- Conclusion:

Solution #7 and solution #9 for Key Issue #7 shall be progressed for normative work.

The UE and the PLMN (HPLMN or VPLMN providing 5G-only national roaming access):

-	the UE indicates support of MINT_Ph2 in the REGISTRATION REQUEST message;

-	the AMF or the MME of the HPLMN or the registered VPLMN (e.g. the VPLMN providing 5G-only national roaming access) sends a disaster roaming wait range, analogous to the one defined in MINT;

-	once the UE is informed about disaster condition as in conclusions for Key Issue #1, the UE is only allowed to perform registration attempt to the VPLMN providing disaster roaming services in EPS upon expiration of the disaster roaming wait range timer.

The PLMN providing a disaster roaming services:

-	while eNB of the PLMN providing disaster roaming services broadcasts disaster related information as in conclusions for Key Issue #2, the eNB can provide additional information on the access barring parameters to be applied to the inbound disaster roaming UEs;

The UE registering or registered for disaster roaming services:

-	NAS shall notify the AS layer (i.e. RRC layer) that the current access attempt is for disaster roaming services.

-	the RRC of the UE performs a barring check for disaster roaming;

-	depending on the result of barring check, the access attempt will be allowed or barred for the UE.

NOTE:	The details of barring information provided via SIB message will be determined in RAN WG2.

- Expected SA2 impact:

Update MME functionality to provide disaster roaming wait range timer in ATTACH ACCEPT message, TRACKING AREA UPDATE ACCEPT message, or GUTI REALLOCATION COMMAND message.

Key Issue #8: Prevention of signalling overload by returning UEs in the VPLMN providing 5G-only national roaming

- Conclusion:

Solution #8 for Key Issue #8 shall be progressed for normative work. The solution:

-	requires the AMF to provide the UE with a “disaster return wait range” timer via specified NAS messages (i.e., REGISTRATION ACCEPT, CONFIGURATION UPDATE COMMAND); or

-	requires the MME in the VPLMN providing disaster roaming services to provide the UE with a "disaster return wait range" timer via specified NAS messages (i.e., ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT, GUTI REALLOCATION COMMAND, TRACKING AREA UPDATE REJECT, DETACH REQUEST, SERVICE REJECT).

In both cases, the UE receiving the "disaster return wait range" timer shall wait until the expiration of this timer before initiating registration to the VPLMN providing 5G-only national roaming, thereby preventing signalling overload due to simultaneous return of multiple UEs.

- Expected SA2 impact:

Update MME functionality to provide disaster return wait range timer in ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT, GUTI REALLOCATION COMMAND, TRACKING AREA UPDATE REJECT, DETACH REQUEST, SERVICE REJECT messages.