3GPP TSG-SA2 Meeting #169	S2-2504800

# Fukuoka, Japan, 19 – 23 May 2025

Source: 	Transsion Holdings

Title: 	KI#1: New Solution – UE-side model training data collection via DCCF and ADRF in CN over UP

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 to support UE-side model training data collection via CN over UP.

1	Discussion

This paper proposes a solution to support UE data collection via DCCF and ADRF over UP for UE-side model training.

The DCCF and ADRF have been introduced to 5GC since R17. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection (DCCF discovery principles are defined in clause 6.3.19 of TS 23.501 [2]). And ADRF is allowed by 5G System architecture to store and retrieve the collected data, analytics and ML Model(s). The DCCF may determine the ADRF and interact directly or indirectly with the ADRF to request or store data or analytics.

Figure 4.2.1-1 in TS23.288 shows architecture with DCCF and ADRF for Analytics, Collected Data and ML Model(s) as below,

This architecture with DCCF and ADRF could be re-used for UE-side model training data collection coordination and storage.

The approved SID in SP-250413 shows that WT#1 is to study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

In TR 38.843, it clearly states that “For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection. Only for model inference, when required data comes from other entities, there is a latency requirement for data collection.” . Online model training is not mentioned in TR38.843. Actually, online trained model usually cannot be used due to no model validation and model testing and even not needed.

The nature of AI model training is to extract patterns from huge amount of data over a certain probability distribution through mathematical algorithms such as neural networks and decision trees, forming generalized model(s). In order to get the right pattern(s) to form correct generalized model(s), the collected data for UE-side model training should has the following characteristics,

- No latency requirement, offline collection is applicable

- Huge amount

- From UEs, but non UE specific

- Note: Data collected for model inference is out of scope of this paper and latency requirement need to be considered for model inference.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #x> | <Key Issue #y> |
| #x | x |  |

Next Change (all new text)

6.X	Solution #X: UE-Side model training data collection via DCCF and ADRF in CN over UP

6.X.1	Description

This solution is for Key Issue#1 for UE data collection over UP.

The DCCF and ADRF have been introduced to 5GC since R17. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection (DCCF discovery principles are defined in clause 6.3.19 of TS 23.501 [2]). And ADRF is allowed by 5G System architecture to store and retrieve the collected data, analytics and ML Model(s). The DCCF may determine the ADRF and interact directly or indirectly with the ADRF to request or store data or analytics.

- In this release, the DCCF and ADRF can also be used to collect, store UE-side model training data from UE according to data consumer’s request, wherein the data consumer could be entity in CN or AF (e.g. OTT server) to train UE-side ML model. Data consumer requests UE-side model training data from DCCF, if the AF (e.g. OTT server) is an untrusted AF, the data collection request should be transferred to the DCCF via NEF. The DCCF check whether the UE-side model training data is available in ADRF, if the UE-side model training data is available in ADRF, DCCF send the data to the data consumer. If the UE-side model training data is not available in ADRF, then DCCF starts to collect data from UEs, UEs collect and send collected data to ADRF through user plane, ADRF may send the data after the required data is ready, DCCF purge and send the UE-side model training data to the data consumer.

6.X.2	Procedures

6.x.2.1 UE-side mode training data collection via DCCF and ADRF in CN over UP

Figure 6.X.2.1-X: UE-side model training data collection via DCCF and ADRF in CN over UP

- The Data consumer (AF) requests UE training data from DCCF, if the AF (e.g. OTT server) is an untrusted AF, the data collection request should be transferred to the DCCF via NEF. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection. The data consumer requests data via the DCCF, using Ndccf_DataManagement_Subscribe service operation.

- The DCCF sends a request to the ADRF, using Nadrf_DataManagement_RetrievalSubscribe (UE-side model training data Specification, Notification Target Address=DCCF) service operation.

- The ADRF responds to the DCCF with an Nadrf_DataManagement_RetrievalNotify response indicating whether the ADRF can supply the UE-side model training data. If the UE-side model training data can be provided, the procedure continues with step 11, otherwise step 4 to 10 is performed.

- The DCCF sends request to NG-RAN(s) via AMF to collect UE-side mode training data.

- NG-RAN(s) sends the requests to a group of UEs (e.g. by broadcasting to all UEs in the cell or paging a group of UEs)

- UEs collect UE-side model training data.

- UEs establish PDU session(s).

- UEs utilize user plane with the established PDU session(s) to transfer UE-side model training data to ADRF.

- The ADRF merges and restructures and stores UE-side model training data together with historically collected UE training data.

- The ADRF sends the DCCF with an Nadrf_DataManagement_RetrievalNotify response indicating the ADRF can supply the UE-side model training data is ready.

- The DCCF fetches the UE-side model training data from ADRF and perform purging to clean the UE label and other sensitive information.

- The DCCF sends the purged UE training data to notification endpoint (data consumer) via Ndccf_DataManagement_Notify.

6.X.3	Impacts on services, entities and interfaces

DCCF:

-	Sends data collection request to NG-RAN via AMF to collect UEs data;

-	Allocates and sends UP information for data collection to NG-RAN via AMF.

- -  DCCF purge the UE training data to clean the UE label and other sensitive information

- -  DCCF send the de-sensitive data to data consumer.

.

ADRF:

-	Receives data collected by the UEs via UP connection from the UEs and store collected data in ADRF;

- -  ADRF merge, restructure and store UE training data together with historically collected UE training data.

.

AMF:

- -  Receive request(s) including data type, UP information from DCCF and send the request(s) to NG-RAN

- .

NG-RAN:

- -  Receive request(s) including data type, UP information from AMF and send the requests to a group of UEs or all UEs in the cell

- .

UE:

-	Receives UP information for data reporting originated from DCCF, and establishes UP connection with the DCCF to report data;

-	Receives data collection request originated from DCCF, and collects corresponding data;

- 	Sends the collected data via the UP connection to the DCCF.

End of Changes