3GPP TSG-SA WG2 Meeting #169	S2-2504814

Fukuoka, Japan, May 19th – 23rd 2025

Agenda item:	19.4.2

Source:	Nokia

Title:	Discussion on overlapping PDRs

WI/Release:	EnergySys /Rel-19

Document for:	Discussion and Decision

# 1	Discussion

At SA2#168 a 23.502 CR S2-2504297 was agreed. It provides that the SMF uses N4 PFCP procedures to collect usage data at the requested granularity by EIF from UPF.  The matching 23.501 CR was withdrawn to further think about the behaviour when URR that the SMF provides to the UPF due to EIF request overlap with URRs the SMF provides to UPF upon request by PCF.

For instance:

Example 1

EIF URR1 for PDR A matching IP@1 and Protocol P

PCF URR2 for PDR B matching IP@1

Example 2

EIF URR3 for PDR C matching IP@1 and IP@2

PCF URR4 for PDR D matching IP@1 and IP@3

The issue is liked to this text in TS 29.244:

On receipt of a user plane packet, the UP function shall perform a lookup of the provisioned PDRs in the UP function to identify only one PDR in a PFCP session according to the following steps:

-    identify first the PFCP session to which the packet corresponds; and

-    find the first PDR matching the incoming packet, among all the PDRs provisioned for this PFCP session, starting with the PDRs with the highest precedence and continuing then with PDRs in decreasing order of precedence. Only the highest precedence PDR matching the packet shall be selected, i.e. the UP function shall stop the PDRs lookup once a matching PDR is found.

For example 1, therefore, care needs to be taken so that the precedence of the PDR A is set so the PDR A is processed before PDR B. Then, URR2 is linked to both PDR A and PDR B, while URR1 is only linked to PDR A. Note this type of conflict can already happen today when PCF requests data for protocol P that is discounted from the data for IP@1.

Observation 1: overlap of "nested" PDRs needs to be covered already today and is addressed by the PDR precedence mechanism.

For Example 2, the SMF can split the PDR C and PDR D into

PRD F matching IP@1

PRD G matching IP@2

PRD H matching IP@3

Then URR3 is associated with the PDR F and PDR G, URR4 is linked to PDR G and PDR H.

Observation 2: overlap of PDRs that overlap because they have overlapping components logically grouped in a single PDR can be resolved by creating non overlapping PDRs.

# 2	Conclusions

Based on the discussion above it is proposed to agree that logic at SMF can resolve the conflicts if they arise, and this needs not be further specified. The 23.501 CR in S2-2504816 adheres to this agreement and should be approved.