SA WG2 Meeting #169	S2-2504820

19th – 23rd May, 2025, Fukuoka, Japan	(Revision of S2-xxx)

Source:	Samsung

Title:	KI#4 New Sol: Sensing Data and the Associated Information Collection and Transport

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC /Rel-20

Abstract of the contribution: This contribution proposes a new solution for Sensing KI#4 on Sensing Data and the Associated Information Collection and Transport.

# Discussion

As it has been documented in clause 5.4 of TR 23.700-14, for KI#4 Sensing Data and the Associated Information Collection and Transport, the following questions are to be studies by SA2:

-	How to collect sensing data (and the associated information, if any) for sensing result generation?

-	Which Sensing Function performs sensing result generation?

-	Whether and what the above associated information is required for sensing result generation

NOTE:	Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

This contribution provides the end-to-end solution for KI#4 Sensing Data and the Associated Information Collection and Transport.

# Proposal

It is proposed to adopt the following changes into TR 23.700-14.

*** Start of the change ***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #4> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | x |  |

*** Next change (all new text) ***

## 6.X	Solution #X: Support of Sensing Data and the Associated Information Collection and Transport

### 6.X.1	Description

This contribution provides the end-to-end solution for KI#4 Sensing Data and the Associated Information Collection and Transport.

Sensing data and associated information needs to be collected by sensing entities and transported to SSF (Sensing Service Function) to support sensing results calculation. This contribution provides general solution for sensing data transmission to cover different sensing operations by assuming that the sensing entities (receivers) could be UEs and/or gNBs. When the sensing entity is a UE, it may transfer sensing data to SSF via CP and/or UP, based on SSF configuration. When the sensing entity is a gNB, the gNB may report sensing data to SSF directly (e.g. based on sensing protocol) or transfer sensing data to SSF via AMF.

### 6.X.2	Procedures

Figure 6.x.2-1 procedure of Sensing Data and the Associated Information Collection and Transport

0.	[Optional] sensing service consumer may send sensing service request to SSF.

The consumer may include the following information in the sensing service request: AOI and time window of the required sensing service, sensing objects (e.g. human, animals, etc.), sensing detection events (e.g. moving objects, new hazards, etc.), required accuracy level of sensing results, required detection probability and false alarm probability of sensing results, metrics for sensing performance evaluation, candidate list of sensing entities (if available) and other assistance information that is helpful for SSF to determine sensing configuration or sensing results.

1.	SSF determines to the configuration and control information for a sensing service. This might be triggered by the request received in step 0 from the consumer, operator’s policies, or SSF internal logics.

SSF may determine the sensing operation for the corresponding service (e.g. mono-static or bi-statics), sensing entities, configuration of the radio signals for sensing purposes, path of sensing data transfer (e.g. via CP or UP), etc.

NOTE 1: sensing entities selection is addressed in KI#3.

NOTE 2: the configuration of the radio signals for sensing purposes may require coordination with RAN WGs.

2.	The SSF sends the sensing configuration and control information to the selected sensing entities. The SSF may also include a sensing service correlation ID.

3.	[Optional] if the SFF does not trigger the sensing signal transmission and reception at sensing entities at this step 2, it may send sensing data collection and/or transfer request to sensing entities, when needed.

4.	The selected sensing receivers and sensing transmitters behave as configured. The sensing receivers perform sensing data collection based on configuration.

5.	The sensing entities report the collected sensing data to SSF for sensing results calculation. The sensing entities may include the sensing service correlation ID in the report.

A sensing entity (e.g. a UE) may report the sensing data via user plane and/or control to the SSF.

A sensing entity (e.g. a gNB) may report sensing data to SSF directly (e.g. based on sensing protocol)  or via AMF.

Editor’s Note:	the detailed procedure of sensing data transfer via user plane is FFS.

Editor’s Note:	it is FFS on whether sensing protocol for data transfer between gNB and SFF is needed or not.

6.	The SSF determines sensing results based on the sensing data and optional the sensing assistance information reported from sensing entities.

The SSF may apply different weights to the sensing data reported from different sensing entities when deriving the sensing result to achieve the required accuracy level.

The SSF may apply perform sensing entity SE selection again to select new sensing entities if the sensing result does not satisfy the service requirements in the Service Request.

7.	[Optional] the SSF may notify the sensing results and other associated information (if any) to sensing service consumer.

### 6.X.3	Impacts on services, entities and interfaces

SSF:

-	Trigger sensing data collection and/or transmission.

-	Configure radio signals for sensing and indicate the configuration to sensing entities.

-	Reception of sensing data reported from sensing entities.

-	Derive sensing results based on (aggregated) sensing data.

-	Expose sensing results to sensing service consumer.

UPF, AMF:

- Support sensing data transfter from sensing entities to SSF, if needed.

*** End of the change ***