SA WG2 Meeting #169	S2-2504838

Fukuoka, Japan, May 19 – 23, 2025

Source:	NTT DOCOMO

Title:	KI#3: New Sol: UP path adjustment based on energy objective

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#3 where the UP paths of PDU sessions are adjusted based on the objective of the operator’s energy policy.

# 1	Discussion

Energy consumption of the user plane (UP) of the 5G core is significant due to the huge amount of the traffic. In the real-world deployments, the UP of the 5GC is not just a single UPF. For several reason, as depicted in the Figure 1, more than one UPF may be deployed in the operators’ network such as

- Handling the peak traffic (which is beyond the capacity of a single UPF)

- Deploying multiple Intermediate UPFs (I-UPF) for reliability objectives

- Deploying multiple Up Link Classifier UPFs (UL CL UPF) for traffic steering to local networks

- Deploying UPF instances for a specific functionality, e.g., a dedicated UPF for Lawful Interception

Figure 1. Scenarios of multiple UPFs deployment

Therefore, user plane traffic may be handled by a chain of UPFs interconnected via N9 interface, i.e., there is a user plane path for each PDU session in the 5GC.

Different UPFs may have different characteristics from energy perspective; e.g., a UPF may be powered (partially) by renewable energy sources, another one could be ultra energy efficient; and the operator may have various energy objectives e.g., maximizing energy efficiency or minimize carbon emission.

Putting these observations together, it is possible to adjust UP paths in the 5GC to implement the operator’s energy objective. This paper proposes a solution to adjust the UP path of PDU sessions to optimize the operator’s objective.

# 2 Proposal

It is proposed to update TR 23.700-67 according to the following text.

********** First Change (All new text) **********

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X |  |  | x |

## 6.x	Solution #X: UP path adjustment considering energy-related information

### 6.x.0	High-level solution principles

The high-level principles of the proposed solution are as follows:

-	Multiple UPFs (not necessarily the same from energy perspective) are deployed in the operator network and each PDU session has a UP path in the 5GC.

-	The operator may have various energy objectives such as maximizing renewable energy ratio or minimizing energy consumption. The objectives may be triggered by e.g., operator’s decision, time, unexpected energy consumption of PDU sessions, etc.

-	Upon a trigger, the SMF based on the operator’s energy objective and the energy-related information (from OAM and/or EIF) adjusts the UP path of some of the already established PDU sessions; i.e., reselects appropriate UPFs for the PDU session (subject to SSC mode) to optimize the operator’s energy objective.

-	The newly adjusted UP paths are implemented in the network using the existing mechanisms defined in clause 4.3.5 of TS 23.502.

### 6.x.1	Description

It is assumed that the operator configures the energy configuration (via OAM) in the SMF and/or PCF. Each configuration indicates the energy objective for example minimizing the total energy consumption or maximizing the renewable energy consumption ratio. Moreover, each configuration also specifies a list of activation triggers for example activation time window, energy consumption beyond a threshold, manual activation by the operator. It may also include some additional information such as the list of UEs which the energy optimization is applicable.

Based on the operator’s configurations, the SMF collects the corresponding energy related information from the OAM and/or EIF to determine when the objective should be activated. When the SMF detects a trigger of an objective, it attempts to optimize the objective via adjustment of UP path of PDU sessions.

For example, if the operator’s objective is to minimize the total energy consumption and the corresponding trigger is a threshold on energy consumption, the SMF will monitor the total energy consumption information of the UPFs in the network, and if the total energy consumption is beyond the threshold, then the objective is activated.

The main functionality of the SMF, upon detecting a configured trigger, is to optimize the corresponding objective. For this purpose, the SMF select a number of PDU sessions as the candidates to be rerouted. The SMF may take into account various criteria to select the candidate PDU sessions for example SSC mode, data volume, QoS requirement, the associate DNNI, predicted load of the UPFs (provided by NWDAF). Then, the SMF selects alternative UPFs for the sessions that contributes to the optimizing the energy objective.

Following the previous example, the SMF may decides to change the serving UPF of some PDU sessions containing large volume best-effort QoS flows to another UPF.

After determining new paths for the candidate PDU sessions, the paths are implemented using the existing mechanisms defined in clause 4.3.5 of TS 23.502

The following figure 6.x.2-2 illustrates an example of adjustment of UP paths of on-going/established PDU sessions to minimize the energy consumption (as the objective of the operator) where SMF selects to adjust UP paths of the green and blue PDU sessions in such a way that there is no load on three UPFs (which can be turned off later by OAM)

Figure 6.x.2-2: An example of UP Path adjustment

### 6.x.2	Procedures

The procedure depicted in Figure 6.x.3-1 describes how the UP paths are adjusted based on the operator's objective.

Figure 6.x.2-1: Procedure for Energy-Aware adjustment of PDU UP path

1.	In deriving PCC rules, based on operator's policy, the PCF may include information about PDU path adjustment (e.g., list of UEs subject to path adjustment, or thresholds to trigger path adjustment process) in the rules and inform the SMF via Npcf_SMPolicyControl_UpdateNotify.

2.	The SMF subscribes to energy-related information of UPFs defined in clause 6.7.3.1 of TS 28.554 [X].

3.	The SMF subscribes to energy-related information at the PDU session level to EIF by invoking Neif_EventExposure_Subscribe; and gets the information by Neif_EventExposure_Notify.

4.	[Optional] The SMF may subscribe to NWDAF for some Analytics IDs (e.g. NF Load Analytics ID) by invoking Nnwdaf_AnalyticsSubscription_Subscribe.

5.	The SMF initiates the process of adjustment of UP paths of PDU sessions based on some triggers which may be a locally configured operator's objective and/or PCC rules received from the PCF; for example, if the energy consumption of some PDU sessions is above a threshold, or starting a time period.

6.	Based on the operator's energy objective (e.g., minimizing energy consumption) configured in the SMF, and considering the SSC mode of PDU sessions, the SMF identifies a number of PDU sessions that adjusting the UP paths of the PDU sessions improves the objective; for example, by consolidating traffic on a limited number of PSA UPFs or replacing/removing UL CL UPFs. In this process, the SMF may take the load and energy-related information of the UPFs into account.

7.	Based on the outcome of the optimization process, the SMF may configure the newly adjusted paths using the existing procedures, e.g.,

- The SMF may remove additional PDU Session Anchor and Branching Point or UL CL using procedure defined in clause 4.3.5.5.

- In case of SSC mode 2, the SMF may change the PSA UPF using the procedure defined in clause 4.3.5.1.

- In case of SSC mode 2, the SMF may change the PSA UPF using the procedures defined in clause 4.3.5.2 or 4.3.5.3.

- The SMF may change the additional PSA UPF, BP UPF, or UL CL UPF using the procedures defined in clauses 4.3.5.6 or 4.3.5.7.

### 6.x.3	Impacts on existing services, entities and interfaces

The solution has the following impacts:

EIF:

-	Providing energy-related information per 5G Core network part of PDU sessions

PCF:

-	Providing energy-related configuration/policy information (e.g., UE Energy Saving indicator, PDU session-level energy threshold, etc.) to the SMF

SMF:

-	Collect energy-related information from OAM and/or EIF

-	Indicate trigger of configured energy objectives

-	Determining list of candidates PDU sessions for UP path adjustment

-	Updating/Adjusting UP path of candidate PDU sessions in order to optimize the energy objective

********** End of Changes **********