3GPP TSG-SA2 Meeting #169	S2-2504856

Fukuoka, Japan, May 19th-23rd 2025

Source:	Nokia

Title:	KI#3 new solution: Energy-aware NF and path selection

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes NF and path selection solution

# 1.	Introduction

This solution proposes that the NF profile is augmented with an Energy Priority attribute and if available its schedule, and Energy Saving state attribute. The schedule is so that the consumers can avoid the need to be updated with changes of the attributes when a schedule is known. This optimizes the signalling required between NRF and the NF consumers.

Further, it proposes that the SMF also selects the UPFs for the user plane based also on the information above.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| x |  |  | x |

# * * * * Third change (All text new) * * * *

## 6.X	Solution #X: Energy aware NF and path selection

### 6.X.0	High-level solution principles

This solution proposes the following principles to be agreed:

- The NF profile is augmented with Energy Priority Information.

- The NF profile is augmented with a Schedule of Energy Priority Information to enable providing a schedule of Energy Priority information, if one exists, instead of requiring additional updates at a time when the Energy Priority information is known to change.

- The NF profile is augmented with Schedule of EnergySaving/not EnergySaving State (See TS 28.310 [x]) to enable the NF service consumer to know when the NF may be taken out of service according to a schedule, to save energy.

- The UPFs are selected by the SMF taking into account the Attributes above in the NF profile, if any are present, based on a UP Path Selection policy.

### 6.X.1	Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

-	New optional Energy-related NF profile attributes are defined as in Table 6.X.1-1 and are stored at NRF

-	OAM provisions (and updates as needed) or the service producer NF registers (and updates as needed) Energy-related NF profile attributes in its NF profile at the NRF.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure. If a NRF provides a scheduling information about an Energy-related NF profile attribute, it shall update the NF service consumer which subscribe to NF profile updates only about unscheduled changes of Energy-related NF profile attribute.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF. The NF consumer can take Selection decisions based on the Energy-related NF profile attributes scheduling information it receives if any, without the need to contact the NRF further.

Table 6.X.1-1: Energy-related NF profile attributes

| Information | Description |
| --- | --- |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, e.g. the time of entering/existing the state regarding energy saving. (NOTE 1) |
| Energy Priority Information | This parameter indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The schedule of the Energy Priority Information if one exists. If this is present, the Energy Priority information is not present. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310[x]. This schedule can be used for e.g., UPF selection for PDU Session with SSC mode 1, and SMF may not select the UPFs that are scheduled to enter EnergySaving State. |  |

The Energy Priority Information indicates the priority to select a NF relative to other NFs of the same type considering the energy characteristics of the NF. If Energy is a criterion for selection of a NF, the NF Consumer can use Energy Priority Information parameter to select those with higher Energy Priority. The NF Service Consumer or the NRF discovery service may select the NF or NF instance considering the Energy Priority information in the discovered NF profile.

The value of the Energy Priority Information is determined by the Operator based on their policies outside the scope of 3GPP.

NOTE:	The information within NF profile specified in Table 6.X.2-1 is expected not to change frequently, e.g., constantly reflecting changes in energy consumption.

By knowing the Schedule of Energy Saving/not EnergySaving a NF service consumer can implement NF selection and NF reselection strategies taking this information into account. This may help to migrate Sessions from an SMF or UPF about to go out of service. Also, PDU sessions with SSC mode 1 should not be caused to use UPFs/SMFs that are known to enter EnergySaving state.

### 6.x.2	Procedures

The procedures are not impacted but the NF profile information is impacted.

### 6.x.3	Impacts on existing services, entities and interfaces

The impacts are:

-	NRF

-	support the Energy-related NF profile attributes as per Table 6.X.1-1

-	enhancement of NRF services and NRF logic to handle the new Energy-related NF profile attributes

-	OAM

-	support the enhancements to handle the new Energy-related NF profile attributes

-	other NFs

-	enhancement of NF service producers to register new Energy-related NF profile attributes

-	enhancement of NF service consumers to discover and select NFs by also considering Energy-related NF profile attributes

# * * * * End of change * * * *