3GPP TSG-WG SA2 Meeting #169	S2-2504885

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	AIoT Data and Approximate D2R Message Size

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR discusses the options for AIoT Data definition and proposes the way forward as well as the impacts on Approximate D2R Message Size.

1. Introduction

In SA2#168, there was a controversial topic about the AIoT Data. There was no consensus on whether 3GPP should standardize the command parameters (i.e., the AIoT Data) from the AF towards the AIoT Devices.

Some companies believe 3GPP should not step into the memory storage domain which belongs to application layer, while some companies believe it is essential to define the basic operations (e.g., read, write) between the network and the AIoT Device.

Tag Memory in RFID

In RFID, the memory structure is defined as below:

There is also some description about Tag memory as below:

Tag memory shall be logically separated into the four distinct memory banks shown in Figure 6.19, each of which

may comprise zero or more memory words. The memory banks are: ……

The logical addressing of all memory banks and User-memory files shall begin at 00h. The physical memory map

is Tag-manufacturer defined. ……

[Observation-1] The logic tag memory is specified in RFID protocol, while the physical memory map is Tag-manufacturer defined.

Option-A: If the network is not aware of command parameters

Assuming the network is transparent towards command parameters, the AF outside trusted domain sends Nnef_AIoT_Command Request with AF ID, Command Type, AIoT Device identification information, target area information, assistance information and AIoT data to the NEF. The NEF forwards the request to the AIOTF. The AIOTF knows the Command Type (Read, Write, or Disable), but is transparent towards AIoT data.

The information inside AIoT data is communicated between the AF and the AIoT devices, which is transparent towards the network. It means, there is an AIoT application layer on top of the AIoT NAS layer between the AF and the AIoT devices. The AF can put any information inside AIoT data, once agreed by the AIoT devices. Such information can be beyond command parameters. In fact, the command itself can also be put inside the AIoT data. The device can execute the command defined inside the AIoT data while simply ignore the command type provided by the AIOTF.

Such design brings flexibility towards the solution: The AF and the AIoT devices can support any functionalities once the function is defined in the AIoT application layer, and the AF and the AIoT devices have implemented it. There are no needs to be restricted by the functions supported by the network. For example, the AIoT application layer can define temporary disable/enable command pair inside AIoT application layer. Once implemented by the AF and the AIoT devices, such the temporary disable/enable can be supported within Rel-19 in a proprietary way.

On the other hand, there are some costs for such flexibility from network perspective.

- The AIOTF cannot perform any control or validation to ensure the Command Type and the AIoT data are aligned. In another word, it is also meaningless for the AIOTF to be informed about the Command Type, and further distribute it. Therefore, the Command Type parameter can be removed from the service operation parameters safely, as well as from the AIoT NAS parameters.

- The network does not care about the supported command. Once the AF and the AIoT devices support such command, it can be supported from end-to-end perspective. Therefore, the read, write, disable should be removed or mentioned as example commands.

- It also means, the AIOTF cannot perform the AF authorization on the service operation level effectively, as it is not very meaningful to check the Command Type parameter while the AIoT data can tell the AIoT devices something different. It is dangerous for the network transmits the command request/response between the AF and AIoT devices but without sufficient control. It is dangerous to provide the possibility for the AF to include a permanent disable command inside the AIoT data but tell the AIOTF it is a read request.

Also, without proper standardization organization to specify the interface between the AF and the AIoT devices, the interoperability cannot be ensured. AF may need to be customized per AIoT device vendor, or the AIoT device may need to be customized per AF.

As discussed in CC before SA2#169, within the Option-A, the two sub-options can be proposed as below:

- Option-A.1: the AF provides Command Type and AIoT data the AIOTF as specified today.

- Option-A.2: the AIOTF exposes a Command Transfer service operation towards the AF, the whole command is within the container, and thus the AIOTF is not aware of the command at all. It also means, the command type and AF authorization for the service operation are removed.

In our view, in Option-A.1, the unawareness of the AIoT data makes the Command Type meaningless towards the AIOTF. Option-A.2 is clean on the aspect that the AIOTF is transparent to the whole command and provides command transfer functionality. If Option-A is the decided way forward, Option-A.2 is better than Option-A.1.

Option-B: If the network is aware of command parameters

In case the network is aware of the command parameters, the AIoT data need to be standardized. The AIOTF receives the AIoT data form the AF, parses and check AIoT data and further provide them in NAS command parameters.

Before discussing AIoT data, the device memory logic structure may need to be figured out.

For the memory banks in RFID, the usage can be discussed in AIoT context:

- EPC: similar memory bank is needed to store Permanent Device ID

- TID: Not standardized in 3GPP.

- RESERVED: No standardization about Access Password and Kill Password. There may be similar needs to store device credentials, which is up to SA3 to standardize

- USER: It may be needed depends on the needs of the application.

Besides those memory banks, in AIoT, it is up to SA3 to decide whether there are needs to store the Temp ID and/or other security materials. Depends on implementation, there could be memory storage to store the AIoT device status (or state). So, in Ambient IoT, the memory banks may need to store the following device information: Permanent Device ID, Device credential: up to SA3 to determine, User, Temp ID: up to SA3 to determine, Status (or state)

Depends on whether all the memory banks are open for reading/writing, there can be two sub-options:

- Option-B.1: As all the memory banks are open for reading/writing, it is essential to define the logic memory map to ensure the inter-operability. 3GPP may define the logic memory map for the memory banks as RFID, or if it is not specified by 3GPP, the AF and the AIoT device may reach the agreements while utilizing the 3GPP network for reading/writing towards a certain position within a specific memory bank.

- Option-B.2: The read/write operations are limited to the user memory bank for application logic. For the other memory banks (regarding them as system memory banks), the access should be via other NAS messages which are different from read and write. In this option, there are no needs to define the logic memory map. The device vendors are free to design the memories.

For example, for the permanent device ID memory bank, the read operation in the device is initiated by AIoT paging which is caused by the Inventory Request, but not a read command. If the network needs to update the permanent device ID dynamically, an “Update Permanent Device ID” NAS command needs to be developed.

In our view, in Option-B.2, it makes sense to expose the user memory for read/write, while the system memories are allowed to be accessed via separate NAS messages but not directly via read/write operations. If there are no such needs, the relevant system memories will not be accessed by the network (i.e., the relevant NAS message will not be developed).

And Option-B.2 further decouples the logic memory mapping between the AIoT devices and the AF. It can enable the interoperability between the AIoT devices and the AF. If Option-B is the decided way forward, Option-B.2 is better than Option-B.1.

Making a comparison between Option-A and Option-B, Option-A offers a flexible solution with the following drawbacks:

- MNO cannot perform sufficient validation or control towards the command. And thus, the AF authorization on the service operation level cannot be performed.

- The management requirements in SA1 (enable, temporarily disable and permanent disable) are not applicable. Such functions can be implemented in the application layer by the AFs and the AIoT devices, but not relevant with the network.

- Without proper standardization, the AF and the AIoT devices are customized with each other, which brings more difficulties in the interoperability.

In our view, it is more important to for the network to authorize towards the commands provided by the AF and fulfil management requirements specified by SA1. And it is important to enable the interoperability for the Ambient IoT business. Considering the extreme low capability of the type 1 devices in Rel-19, the flexibility may be less important and less attractive.

[Proposal-1] Adopt Option-B.2 as discussed above.

There is an EN about approximate D2R message size in clause 7.2.3 and clause 7.4.3

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

Based on [Proposal-1], the AIOTF is aware of the command parameters of each command request, the AIOTF can also estimate the approximate D2R message size of the command response. To be more specific:

- For read, the AIOTF understands the length of data to be retrieved from the AIoT device.

- For write or disable, the device response can be success or failure with proper cause code.

[Proposal-2] The approximate D2R message size does not need to be provided by the AF as a separate parameter for the command.

[Proposal-3] Resolve the EN and let the AIOTF determine the approximate D2R message size for the command.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

### 4.5.1	AIoT Device

AIoT Device supports Inventory Response, Command Request and Command Response in AIoT NAS layer.

AIoT Device may contain an application memory to store application specific data and is accessed using read and write command in AIoT NAS layer.

AIoT Device may be permanently disabled by the disable command in AIOT NAS layer.

Editor’s note:	The definition of AIoT Device needs to align with the definition at RAN WGs.

* * * Next Change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Approximate D2R message size.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

* * * Next Change * * * *

### 6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information], [Approximate number of AIoT Devices],  Command specific parameters) message to NEF.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Command Type indicates the command to be performed on the AIoT Device(s), i.e., read, write, or disable.

Command specific parameters provide the required parameters for command operation, as specified in clause 7.4.3.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices],  Command specific parameters) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF determines NAS Command Request sent to the AIoT Device, using the Command Type and the Command specific parameters received in step 3. The AIOTF estimates the Approximate D2R message size of the NAS Command Response from the AIoT Device.

The AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

* * * Next Change * * * *

### 7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Disable.

5)	Command specific parameters:

-	For Read, the start address of the application memory and the length of data to read.

-	For Write, the start address of the application memory and the data to write.

-	For Disable, the type of disable (temporary or permanent).

NOTE X:	Temporary disable is not supported in this release.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

* * * Next Change * * * *

### 7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable), Command specific parameters (For Read, the start address of the application memory and the length of data to read; For Write, the start address of the application memory and the data to write; For Disable, the type of disable (temporary or permanent).

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

* * * End of Change * * * *