3GPP TSG-WG SA2 Meeting #169	S2-2504887

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 6.2.4 & 7.3.2 & 7.3.3: Update to Align with RAN3 on Correlation ID

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the procedure of indirect connectivity to align with RAN3 on Correlation ID.

1. Introduction

When discussing clause 6.2.4 “Procedures between AIOTF and NG-RAN for Indirect Connectivity” in SA2#168, the Correlation ID was discussed. As it is not a must for the AMF to perform message routing, it is preferred to be included inside AIoT Transfer Container, so that the AMF does not have to be aware of it.

In LS from RAN3 (R3-252481), there is a working assumption that the Correlation ID is also included outside the container:

On Inventory:

- Within all Inventory related NGAP messages:

  - The Correlation Identifier is included inside the AIOTF information container.

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is also included outside the AIOTF information container.

On Command after Inventory:

- Introduce a “follow-on command indication” in the AIOTF information container in the Inventory Request message.

- Working assumption: The NGAP Command Request procedure concerns a single device only.

- Within all Command related NGAP messages:

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is included inside and outside the AIOTF information container.

[Observation-1] There is a misalignment between SA2 and RAN3 regarding the Correlation ID. RAN3 made the working assumption that the Correlation ID is included outside the container.

To resolve the misalignment, it is essential to understand whether there are benefits of including the Correlation ID outside the container.

The interactions between the AIOTF and the AMF belong to “Subscribe-Notify”. The AIOTF requests the AMF to deliver the N2 message (e.g., inventory request) towards the NG-RAN, and then implicitly subscribes to the AMF. When receiving the corresponding N2 messages (e.g. inventory report) from the NG-RAN, the AMF notifies towards the AIOTF. In this implicit subscription, the AIOTF needs to provide a Notification Target Address and a Notification Correlation ID to the AMF, so that the AMF can notify the AIOTF properly. In this case, the Correlation ID is the Notification Correlation ID. And the AMF needs to be aware of it.

[Proposal-1] Introduce Correlation ID in the interactions between the AIOTF and the AMF, as well as between the AMF and the NG-RAN. The AIOTF provides Notification Target Address to the AMF in Namf_AIoT_MessageDelivery.

In clause 7.3.3, there is an EN about the alignment with RAN3 on the AIoT messages:

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

[Proposal-2] Update the AIoT messages to AIoT Transfer Container and further clarifies in clause 7.3.2 and 7.3.3. The EN can be resolved.

In clause 7.3.2, there is an EN about the Correlation ID and AIOTF ID in Namf_AIoT_MessageDelivery:

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

[Proposal-3] Based on [Proposal-1], both AIOTF ID and Correlation ID needs to be included in Namf_AIoT_MessageDelivery. The EN can be resolved

In clause 7.3.3, there is an EN about the Correlation ID in Namf_AIoT_Notify:

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

Using the Notification Target Address is usually not sufficient in the Notify message. A Notification Correlation ID needs to be provided additionally. In this case, the Notification Correlation ID is the Correlation ID.

[Proposal-4] Correlation ID is needed in the Namf_AIoT_Notify. The EN can be resolved.

In clause 7.2.1, there is an EN about the indirect connectivity:

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

[Proposal-4] Based on clause 6.2.4 and clause 7.3, the EN can be resolved.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

### 6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity

An AIOTF and NG-RAN can use an indirect interface via an AMF as described in clause 4.2.2.4. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4-1.

Figure 6.2.4-1: Procedure for AIOTF and NG-RAN for indirect connectivity via an AMF

1.	The AIOTF sends Namf_AIoT_MessageDelivery message (AIoT Transfer Container, NG-RAN ID, AIOTF ID, Correlation ID, Message Type for AIoT Transfer Container, Notification Target Address) to the AMF. The AIoT Transfer Container may be Inventory Request Transfer or Command Request Transfer. The AMF stores the mapping between the AIOTF ID, Correlation ID and Notification Target Address.

2.	The AMF sends an NGAP message (AIOTF ID, Correlation ID, AIoT Transfer Container) to the target NG-RAN.

3.	The NG-RAN sends an NGAP message (AIOTF ID, Correlation ID, AIoT Transfer Container) to an AMF. AIoT Transfer Container may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

4.	AMF determines the Notification Target Address and sends the Namf_AIoT_Notify message (AIoT Transfer Container, Correlation ID) to the AIOTF.

* * * Next Change * * * *

### 7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

* * * Next Change * * * *

### 7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data (i.e. AIoT inventory/command messages) towards NG-RAN or AIoT devices.

Inputs, Required:

1)	The AIoT Trasnfer Container delivered to NG-RAN. The AIoT Transfer Container may be Inventory Request Transfer or Command Request Transfer.

2)	NG-RAN ID.

3)	AIOTF ID.

4)	Correlation ID.

5)	Message Type for AIoT Transfer Container.

6)	Notification Target Address.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

* * * Next Change * * * *

### 7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	The AIoT Transfer Container received from NG-RAN. AIoT Transfer Container may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

2)	Correlation ID.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor's note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

* * * End of Change * * * *