3GPP TSG-WG SA2 Meeting #169	S2-2504891

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Clause 4.6: Update on Protocol Stack

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the clause of protocol stack to align with RAN3 and resolve the EN.

1. Introduction

In the TS 23.369 v0.3.0, there is an EN about the AIoT Reader Control:

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

In LS from RAN3 (R3-252481), RAN3 informs SA2 about their agreement on this aspect:

- Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement.

Based on the agreement in RAN3, there is no AIoT Reader Control layer on top of NGAP.

[Proposal-1] Remove AIoT Reader Control from the specification and resolve the EN.

In clause 8.2.1.3 of TS 23.501, the N2 SM information between the SMF and NG-RAN is depicted as below:

Legend:

-	N2 SM information: This is the subset of NG-AP information that the AMF transparently relays between the 5G-AN and the SMF, and is included in the NG-AP messages and the N11 related messages.

Figure 8.2.1.3-1: Control Plane between the 5G-AN and the SMF

NOTE 1:	From the 5G-AN perspective, there is a single termination of N2 i.e. the AMF.

NOTE 2:	For the protocol stack between the AMF and the SMF, see clause 8.2.3.

[Proposal-2] Reflect the N2 AIoT information in the protocol stack in the similar way as N2 SM information.

In the protocol stack figures, Nb and Nz are kept as the reference points between AIOTF and NEF, as well as between AMF and AIOTF. In SA2#168, the reference point names have been agreed:

AIOT3:	Reference point between the AIOTF and the AMF.

AIOT4:	Reference point between the AIOTF and the NEF.

[Proposal-3] Fix the reference point names in the protocol stack figures.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[x]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

* * * Next of Change * * * *

### 4.6.2	Protocol Stack between AIoT Device and AF

#### 4.6.2.1	General

The general protocol stack between an AIoT Device, NG-RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The NG-RAN may communicate to AIOTF via different connectivities, see clause 4.2.2.1. The NG-RAN protocol stack remains the same and is transparent to the different connectivities. The protocol stacks and routing of information between an NG-RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Connectivity: When the NG-RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Connectivity: When the NG-RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

#### 4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Legend:

-	NG Application Protocol (NGAP): Application Layer Protocol between the AIOTF and the NG-RAN as defined in TS 38.413 [x].

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN transparently.

#### 4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Legend:

-	NG Application Protocol (NGAP): Application Layer Protocol between the AMF and the NG-RAN as defined in TS 38.413 [x].

-	N2 AIoT information: This is the subset of NGAP information that the AMF transparently relays between the NG-RAN and the AIOTF. It is included in the relevant N2 messages (specified in TS 38.413 [x]) and the AIOT3 messages.

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

NOTE X:	From the NG-RAN perspective, there is a single termination of N2 i.e. the AMF.

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN and AMF transparently.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * End of Change * * * *