3GPP TSG-SA WG2 Meeting #169	S2-2504908

Fukuoka Japan, May 19-23, 2025

Source:	Ericsson

Title:	New Sol: Reuse existing LTE solution to establish IP-CAN for IMS voice over NB-IoT (GEO)

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: The contribution proposes a solution to enable IMS voice over NB-IoT(GEO) in EPS.

# 1. Introduction

This contribution proposes a solution to Key Issue #1:

-	Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

# 2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-19.

* * * Start of change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[y]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[z]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

* * * Next change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 |  |  |  |
| X | x |  |  |  |

* * * Next hange * * * *

., focusing on small and infrequent data transfers. As a result, it has fewer system features compared to WB-E-UTRAN. This solution suggests adding the essensial missing features needed for establishing the IP-CAN for IMS voice call in case of GEO NB-IoT.

Below are the key principles:

-	The UE indicates its support of IMS voice over NB-IoT (GEO) in Attach/TAU request.

-	The MME indicates IMS voice over PS session is supported to UE.

-	The UE and network establish SGi PDN connection in IMS APN using S1-U, the default bearer with QCI 5.

-	The UE and the network establish dedicated bearer with QCI 1 (GBR bearer) for the voice traffic.

-	The PGW allows GBR bearer to be created in IMS APN for GEO NB-IoT RAT.

-	The eNB serving NB-IoT (GEO) supports default and dedicated bearer.

### 6.x.2	Description

The IMS voice call over NB-IoT (GEO) can be supported by one IMS PDN connection, with default and dedicated bearer as existing IMS voice support over WB-E-UTRAN.

### 6.x.3	Procedures

#### 6.x.3.1 UE Attach

Figure 6.x.3.1-1 UE attach in NB-IoT (GEO)

During the attach procedure:

-	The UE indicates its support of IMS voice over NB-IoT (GEO) in the attach request message. It helps the network to distinguish the UE with only support of IMS voice over normal LTE access and the UE with enhanced functionatlity of supporting IMS voice over NB-IoT (GEO) access.

-	It is up to the operator’s policy whether the well-known IMS APN is used as default APN in the attach procedure. Using well-known IMS APN during UE attach can reduce the time for UE to be ready for IMS voice call and reduce the needed resources (e.g. number of DRBs), but consequently there is risk that UE may not get data service or even not authorized to attach to the network.

-	If the well-known IMS APN is used during attach:

-	The procedure described in clause 6.x.3.2 is skipped.

-	MME establishes default bearer with eNB using S1-U and UE (not shown in the figure).

-	The P-CSCF address is provided to the UE when activating the default bearer.

-	UE proceeds with IMS registration after successful attach. P-CSCF can obtain RAT type of the IP-CAN by subscribing to the IP-CAN Type Change with PCRF during the SIP registration, as described in 3GPP TS 29.213 [x] Annex B.1a.

-	One optimization to reduce the call setup time is to activate the dedicated bearer prior to the IMS voice call setup (e.g. already at the attach procedure), with following considerations:

-	The PGW creates uplink packet filter (see 3GPP TS 23.060 [z] clause 15.3.3.4 for examples) effectively stopping any packet flow in the uplink direction in this pre-established dedicated bearer for voice media. During the call setup phase, there will be a dedicated bearer modification procedure (see 3GPP TS 23.401 [5] clause 5.4.3) for updating the TFT, where 2 RRC messages (direct transfer) are used between UE and eNB instead of 3 RRC messages (reconfiguration/complete/direct transfer) if the dedicated bearer is established during call setup phase.

-	or PGW is configured with a range of destination IP address and a range of desition port numbers that will be used for the IMS voice media traffic. PGW creates uplink packet filters based on this configuration and provides to the UE. With this approach, the dedicated bearer modification is also skipped, thus lead to a saving of 3 RRC messages comparing to normal call setup signaling flow.

Editors’ note:	The above EPC optimization requires E2E evaluation together with the rest of the system. For example, the IMS SIP singaling transmission currently contributes most of the call setup time (CST), given the SIP signaling message size is larger than RRC message, whether saving of RRC messages would make comparable impact to CST is to be assessed.

-	It is up to the operator’s policy whether to enable attach without PDN connection to optimise the resources, e.g. reduce the total number of data radio bearers (DRBs) for the UE. If attach without PDN connection is enabled and used between UE and network, step 5-8 and step 11/12 are skipped.

-	The MME verifies with HSS if the subscription data allows the UE to use IMS voice over NB-IoT (GEO), and indicates IMS voice over PS session is supported to the UE in attach accept.

#### 6.x.3.2 PDN connection in IMS APN

Figure 6.x.3.2-1 PDN connection establishment for well-known IMS APN

This applies when there is no PDN connection established for the well-known IMS APN during attach and the subscriber is allowed to use IMS voice over NB-IoT (GEO) according to subscription data:

- The NB-IoT (GEO) RAT type is passed to PGW during the create session procedure. The PGW can make use of this information to grant dedicated bearer to be established for the IMS APN PDN connection.

- The MME instructs eNB to setup E-RAB for the default bearer of the PDN connection to the IMS APN.

- QCI-5 is used for the default bearer.

- In case the UE is attached without PDN connection, the S1-AP messages in step 7/9 are replaced with initial context setup request/response.

-	After the PDN connection established for IMS APN, the UE performs IMS registration. The P-CSCF can obtain the RAT type of the IP-CAN by subscribing to the IP-CAN Type Change with PCRF during the SIP registration, as described in 3GPP TS 29.213 [x] Annex B.1a.

-	The same optimization to activate the dedicated bearer during the PDN connection establishment can be applied as described in 6.x.3.1.

#### 6.x.3.3 Dedicated Bearer

Figure 6.x.3.3-1 Dedicated bearer estabishment for voice call

When an IMS voice call is initiated:

-	The PCRF receives the request from IMS, and triggers new PCC rule to be installed with PGW.

-	The PGW verifies that the new PCC rule is for IMS voice call in NB-IoT (GEO) and proceeds to create dedicated bearer.

-	QCI-1 is usesd for the dedicated bearer (GBR bearer).

-	The MME requests eNB to setup E-RAB with QCI-1 and activates dedicated EPS bearer with UE.

-	If P-CSCF has requested access network location info (e.g. UE location) in step 3, the PCRF provides it to P-CSCF in step 16, as described in 3GPP TS 29.214 [y] Annex A.10.

-	If the dedicated bearer is already established at attach or PDN connection establishment:

-	step 0 addtionally needs to include the UP setup for dedicated bearer.

-	step 7 to 14 can either be replaced by step 2 to 9 described in 3GPP TS 23.401 [5] clause 5.4.3, or skipped.

### 6.x.4	Impacts to Services, Entities and Interfaces

The following are the new impact when UE is accessing NB-IoT (GEO) via a satellite:

UE:

- Indicating support of IMS voice over NB-IoT (GEO) in Attach/TAU request.

- Support dedicated EPS bearer.

- Optionally, support more than two simultanous DRBs.

RAN:

- Support GBR bearer with QCI 1.

- Optionally, support more than two simultaneous DRBs per UE.

MME:

-	Support indicating IMS Voice over PS session supported to UE.

-	Optionally, control the maximum number of radio bearer user plane to be created.

HSS:

- Subscription data indicating whether IMS voice over NB-IoT (GEO) is allowed.

PGW:

- Allow voice bearer (i.e. GBR bearer with QCI=1) to be created in IMS APN.

Editor’s Note:	Whether the impact on RAN is feasible requires coordination with RAN WGs.

* * * End of change * * * *