3GPP TSG SA WG2 Meeting #169	S2-2504928

Fukuoka, JP, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	China Mobile

Title:	New Solution for KI#2 on user plane traffic pattern and behavior analysis

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Proposed a new solution for KI#2 on user plane traffic pattern and behavior analysis.

1. Discussion

This proposal proposes a new solution for KI#2 on 5GC Support for efficient performance of user plane based on user plane traffic pattern and behavior analysis.

5.3	Key Issue #2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

5.3.1	Description

Use of AI/ML can be studied to provide analytics on user plane traffic patterns and how to use the analytics to improve user plane performance. In this context, it is proposed to study:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | x |  |  |

# * * * * Second change (all new text) * * * *

6.X	Solution #X: User plane traffic pattern and behavior analysis to support efficient performance of user plane

6.X.1	High level principles

The high level principles of the solution are:

-  The NWDAF analyzes or predicts user plane traffic pattern and abnormal behaviour based on UP data collected from user plane.

-  The consumer subscribes or requests user plane analytics data from the NWDAF and executes the action to improve the network performance accordingly.

6.X.2	Description

This solution is to address Key Issue#2 for user plane traffic pattern and behaviour analysis to support efficient performance of user plane.

As abnormal packets may cause some issues such as UE malfunctions, UPF performance degradation, and system errors, it is necessary to detect and analyze the abnormal traffic to take further measures. Based on the analytical results, the network can take action to improve the general network performance.

- X.3	Procedures

Figure 6.X.3-1: Procedure for analysis and performance enhancement

- The NWDAF may collect one or more of the following data of user plane:

- User plane data information from UPF such as burst traffic, malformed packets, duplicate packets, traffic hijacking and traffic spoofing.

- QoS monitoring data from RAN and/or UPF.

- Video streaming metrics from UPF, e.g. latency and packet loss.

- UPF packet statistics such as bandwidth utilization, packet length distribution, packet loss rate, latency measurements, jitter analysis, packet interval timing and packet retransmission statistics.

Based on the collected data, the NWDAF (MTLF) performs model training to obtain a traffic prediction model and provides the trained ML model to the NWDAF(AnLF).

- The consumer subscribes to the NWDAF (AnLF) for analytics data by invoking the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation for analytics information on a specific UE or group of UEs.

- Based on the consumer requests, the NWDAF (AnLF) collects the UP plane data.

- The NWDAF (AnLF) performs model inference. The analytics data may include one or more of the following parameters: UPF ID, packet count, packet bandwidth, packet statistics (e.g. packet length, packet loss rate, latency, jitter, packet interval and packet retransmission) to represent the information about traffic variations.

- The NWDAF (AnLF) provides the analytics data to the consumer by invoking the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Requet response.

- After receiving the analytics results, the consumer may take the following actions:

- The UPF may control the number of DLDR (Downlink Data Report) messages sent to SMF by e.g., adjusting message frequency.

- The SMF may initiate UPF reselection to distribute the load across UPF instances upon detecting abnormal traffic patterns (e.g., RTT fluctuations, abnormal PPS).

- The SMF may identify anomalies and take preventive actions, e.g., notifying UPF to execute traffic gating or shaping, enforcing bandwidth parameters (e.g., rate limiting to 5Mbps) or adjusting QoS parameters.

- The UPF may apply downlink traffic suppression (e.g., selective packet dropping) for AS-initiated traffic bursts.

- The PCF may provision policies to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g., rate limiting to 5Mbps) or adjusting QoS parameters.

- The RAN may perform dynamic resource adjustments based on NWDAF's predictive analysis.

6.X.4	Impacts on services, entities and interfaces

NWDAF:

-	Provides statistics or predictions of user plane analytics data based on consumer requests.

UPF:

-	Supports collecting user plane information requested by NWDAF.

Consumer NF:

-	Executes the action to improve the network performance based on the analytics data from NWDAF.

# * * * * End of Changes* * * *