3GPP TSG-SA WG2 Meeting #169	S2-2504940

19 May – 23 April 2025,Fukuoka, Japan	(revision of S2-2503589, 3941, 4170,4419)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6233 | rev | 4 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Notification Control of PDU Set QoS with only one direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, [Huawei, Nokia] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the PDU Set QoS handling is applied in UL and/or DL, the NG-RAN uses the PDU Set QoS parameters PSDB and PSER for UL and/or DL instead of PDB and PER in the respective direction(s) to determine whether "GFBR can no longer be guaranteed" or "GFBR can again be guaranteed". If the NG-RAN determines that only in one direction the GFBR, the PDB or the PER cannot be fulfilled, the NG-RAN shall indicate QoS Notification Control message "GFBR can no longer be guaranteed" together with the corresponding direction. How the corresponding direction is provided to the AF is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the PDU Set QoS handling is applied in UL and/or DL, the NG-RAN uses the PDU Set QoS parameters PSDB and PSER instead of PDB and PER in the respective direction(s) for the actions described in this clause (e.g. to determine whether "GFBR can no longer be guaranteed" or "GFBR can again be guaranteed"). If the NG-RAN determines to send a notification that only in one direction "GFBR can no longer be guaranteed", the NG-RAN shall also indicate the impacted direction with the notification. In all other situations the NG-RAN shall not indicate a direction in the notification. NOTE 4: The notification sent by the NG-RAN is always describing the current situation with respect to the fulfilment/unfulfillment of the GFBR, the PDB and/or the PER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF cannot deduce which is the affected direction to adapt accordingly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.7.2.4.1a	Notification Control without Alternative QoS Profiles

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled, NG-RAN shall send a notification towards SMF that the "GFBR can no longer be guaranteed". Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not fulfilling the requested QoS profile for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to fulfil the GFBR, the PDB and the PER of the QoS profile again.

NOTE 1:	NG-RAN can decide that the "GFBR can no longer be guaranteed" based on, e.g. measurements like queuing delay or system load.

Upon receiving a notification from the NG-RAN that the "GFBR can no longer be guaranteed", the SMF may forward the notification to the PCF, see TS 23.503 [45].

When the NG-RAN determines that the GFBR, the PDB and the PER of the QoS profile can be fulfilled again for a QoS Flow (for which a notification that the "GFBR can no longer be guaranteed" has been sent), the NG-RAN shall send a notification, informing the SMF that the "GFBR can again be guaranteed" and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the "GFBR can no longer be guaranteed" whenever necessary.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the "GFBR can again be guaranteed" and therefore a frequent signalling of "GFBR can again be guaranteed" followed by "GFBR can no longer be guaranteed" is not expected.

NOTE 3:	If the QoS Flow is modified, the NG-RAN restarts the check whether the "GFBR can no longer be guaranteed" according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification for a QoS Flow that the "GFBR can no longer be guaranteed". The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. The accepted QoS Flows are included in the N2 Path Switch Request or N2 Handover Request Acknowledge message from the NG-RAN to the AMF. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that "GFBR can again be guaranteed" for this QoS Flow unless the SMF is also receiving a reference to an Alternative QoS Profile for this QoS Flow (which is described in clause 5.7.2.4.2). After the handover is successfully completed, the Target NG-RAN shall send a subsequent notification that the "GFBR can no longer be guaranteed" for such a QoS Flow whenever necessary. If the SMF has previously notified the PCF that the "GFBR can no longer be guaranteed" and the SMF does not receive an explicit notification that the "GFBR can no longer be guaranteed" for that QoS Flow from the Target NG-RAN within a configured time, the SMF shall notify the PCF that the "GFBR can again be guaranteed".

If the PDU Set QoS handling is applied in UL and/or DL, the NG-RAN uses the PDU Set QoS parameters PSDB and PSER  instead of PDB and PER in the respective direction(s) for the actions described in this clause (e.g. to determine whether "GFBR can no longer be guaranteed" or "GFBR can again be guaranteed").

If the NG-RAN determines to send an notification that only in one direction "GFBR can no longer be guaranteed", the NG-RAN shall also indicate the impacted direction with the notification. In all other situations the NG-RAN shall not indicate a direction in the notification.

NOTE 4:	The notification sent by the NG-RAN is always describing the current situation with respect to the fulfilment/unfulfillment of the GFBR, the PDB and/or the PER.

END of CHANGES