3GPP TSG-SA2 Meeting #169	S2-2505029

# 19 - 23 May, 2025, Fukuoka, Japan                                                         rev of S2-250xxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0738 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of data collection for NG RAN-sided models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution 1: Positioning Information collection for UEs under positioning is agrred in RAN3 in R3-252486. Based on the discussion in S2-250xxx4, there are SA2 impacts requiring LMF enhancements to support positioning Information collection for UEs under positioning including: - to retrieve the Part B label, including timing information and the LOS/NLOS indicator derived from UE location; - to send the Part B label and related correlation information to corresponding gNB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures of data collection for NG RAN-sided models. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning Information collection for UEs under positioning required by RAN 3 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new), 6.x.1(new), 6.x.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.x	Procedures of data collection for NG RAN-sided models for AI/ML positioning

6.x.1	General

This clause describes the procedure for data collection by the NG RAN for input data to train or monitor the performance of the ML model for NG RAN-sided models.

6.x.2	Data Collection for NG RAN-sided models for UEs under positioning

Figure 6.x.2-1 shows a procedure for data collection for NG RAN-sided models for UEs under positioning.

Figure 6.x.2-1: Data Collection for NG RAN-sided models for UEs under positioning

Precondition:	The UEs are under positioning, and the network assisted and network based positioning is used.

1-6.	The steps 1-6 of network assisted positioning procedure are used as defined in clause 6.11.2, and the information to indicate that data collection is needed for the UE being positioned is included in the network positioning message in steps 5-6 as defined in NRPPa signalling in TS 38.455 [15].

7.	To provide the data collection for the UE being positioned as indicated in step 6, if the UE location derived from the current UE positioning procedure cannot be used as the ground truth data, the LMF initiates the procedure to retrieve the ground truth data of the UE, otherwise, the UE location derived from the current UE positioning procedure is used as ground truth data of the UE.

Editor's note:	The user consent check for data collectionfor NG RAN-sided models is FFS.

8.	The LMF determines the data requested in step 6 based on the ground truth data of the UE.

9.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Positioning message to the NG-RAN node (gNB) requesting data collection for the UE. The service operation includes the Network Positioning message. The Network Positioning message includes information related to UE location as retrived in step 8 and correlation information that enables the NG-RAN to correlate the information related to UE location with information related to UL measurements as received in step 6 during the UE positioning procedure.

NOTE 1:	The information related to UE location and correlation information are defined by RAN3.

10.	The AMF forwards the Network Positioning message to the NG-RAN node in an N2 Transport message.

* * * End of Change * * *