3GPP SA WG2#169	S2-2505040

Fukuoka, Japan, 19-23 May, 2025

Source:	Tejas Networks Limited

Title:	Solution for KI#2: Authorization and Revocation to Support Sensing Service

Spec:	3GPP TR 23.700-14

Agenda item:	20.2.1

Work Item / Release: FS_Sensing_ARC/ Rel-20

Document for:	Discussion and decision

Abstract of the contribution: This contribution proposes a new solution to Key Issue#2: Authorization and Revocation to Support Sensing Service

1. Introduction

A new SID on architecture enhancement to support Integrated Sensing and Communication was approved in SP-250401 with several objectives. Amongst the approved work tasks within the SID include exploring solutions for service authorization and revocation (WI#2) of sensing service.

This paper proposes a new solution for enabling the authorization of a new sensing service and updation/revocation of an ongoing sensing service in the 5G system.

2. Text Proposal

It is proposed to adopt the following text within TR 23.700-14.

* * * First Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> | <Key Issue #6> |
| X |  | X |  |  |  |  |

* * * * Next Change (All New Text) * * * *

## 6.X	Solution #X: Authorization and Revocation of Sensing Service

### 6.X.1	Description

For establishing a new sensing service within the 5G system, the AF initiates the authorization procedure by sending a sensing service request. Following successful authorization, the service is provisioned by the Sensing Function (SF). In case, the parameters provided to the sensing service need to be changed during the service, or to revoke an ongoing sensing task, the SF can initiate an Update procedure.

Note: Although the SF has been shown as a single entity in the following section, it may comprise of other component NFs such as Sensing Control Function (SCF) responsible for handling the control plane functionality of sensing, Sensing Data Function (SDF) which handles the user plane aspect of sensing and Sensing Data Repository (SDR) which serves as database for sensing related data.

### 6.X.2	Procedures

### 6.X.2.1	Authorization of sensing service.

Figure 1 shows the procedure for authorization of a sensing service. The AF uses Nnef_ServiceParameter service to provide the service specific parameters to the SF.

Figure 1:Authorisation of sensing service.

- The AF creates a sensing service request. To create a new request, the AF invokes an Nnef_ServiceParameter_Create service operation. The request may include subscription information to the report of the outcome of sensing request.

  - To update or revoke an existing request, the AF invokes an Nnef_ServiceParameter_Update or Nnef_ServiceParameter_delete service operation together with the corresponding Transaction Reference ID which was provided to the AF in Nnef_ServiceParameter_Create response message.

  - The content of this service operation (AF request) includes the information described in Section 6.X.2.1.1.

  - If the AF subscribed to the outcome of sensing, the AF indicates where the AF receives the corresponding notifications.

- The AF sends its request to the NEF. The NEF authorizes the AF request. In case of a trusted AF, the sensing request can be directly sent to the SF.

- The NEF may perform the following:

  - (In the case of Nnef_ServiceParameter_Create): The NEF assigns a Transaction Reference ID to the Nnef_ServiceParameter_Create request.

  - (In the case of Nnef_ServiceParameter_Create or Update): The NEF may need to authorize the service specific parameter provisioning request with the SF by sending a Nsf_ServiceSpecificAuthorisation_Create service operation.

The NEF sends Nsf_ServiceSpecificAuthorisation_Create Request including the Service Description address to receive updates of the authorization from SF.

  - In case the SF(SCF) is unable to process this request then the SF responds to the NEF with a reject message and a failure cause in the Nsf_ServiceSpecificAuthorisation_CreateResponse message (Step 8). The NEF then forwards this response to the AF in the Nnef_ServiceParameter_CreateResponse message.

- The SF(SCF) authorizes the AF request received from the NEF and stores the information in the SDR (SF) as "Target Mapping Data". The Service Parameters are delivered to one or more entities in the target area by the SF (SCF) at/before the start of the sensing service

SF maps service description included in the request from the NEF to set of NFs and sensing entities. The mapped NFs depends on the type of sensing activity and the participating sensing entities and sensing functions.

- The SF(SDR) then sends a “SensingEntityAuthorization_Request” to obtains the authorization info for the mapped entities and NFs. In case the authorization information is not available for a given NF/entity, the SF(SCF) can obtain the information from the NRF.

- The sensing entities provide a “SensingEntityAuthorization_Response” to indicate their authorization status and possibly resource availability status to the SF (SCF).

- The received information including the authorizatio status, resource availability status etc is mapped to a service descriptor is then updated/stored by the SF(SDR).

- The SF then responds to the NEF with Nsf_ServiceSpecificAuthorisation_Create Response message

- The NEF then forwards this response to the AF in the Nnef_ServiceParameter_CreateResponse message.

- Additionally, whenever the service parameters are updated, the AF is notified of this event using the Nnef_ServiceParameter_Notify.

### 6.X.2.1.1 	Parameters for creation of an Application Function request

For provisioning a service, the Application Function (AF) provides a request to the NEF with the information listed below. If the AF is designated as a trusted AF, then the following parameters can be exchanged with the SCF directly.

- Service Description

Service Description is the information to identify a specific service request. This identifier should be unique within a given AF.

2)	Service Parameters.

Service Parameters are the service specific information which needs to be provisioned in the network, and possibly the sensing entities (transmitter(s) and/or receiver(s)) to support the service identified by the Service Description. This information may comprise response time, reporting frequency and duration, Quality of Service (QoS) Class details e.g., best-effort, sensing accuracy for e.g.  horizontal or vertical accuracy, possibility of false detection to be in a certain range etc.,

3)	Target Area.

This may comprise of a geographical area for which the sensing service is requested. The target area comprises of a set of sensing entities which may vary at different instants of time. The service parameters are provisioned to one or more sensing entities based on if they are participating in the sensing activity. The authorization of an entity can be given based on the capability of the sensing entity, availability of the entity and other area such as if the entities are marked for certain uses.

- Tracked Object Characteristics.

AF may optionally provide details regarding the type of object to be tracked, along with its physical characteristics such as RCS, mean velocity etc., altitude etc.

- Expected Service Time.

The sensing request may be required for a certain time duration. This is also passed to the NEF.

4)	Subscription to events.

The AF may subscribe to notifications about the sensing outcome due to service specific parameter provisioning. This event may be a one- off event, information regarding the presence of a UAV or periodic data, where the information related to the UAV (such as velocity, size, geographical co-ordinates etc.) may be transferred to the AF.

### 6.X.2.2	Updating the sensing service.

Figure 2 shows the procedure for updation/revocation of an ongoing sensing service. The initial steps illustrated using dashed lines (steps 0a and 0b) are optional and occur when the AF initiates the updation/revocation procedure.

Figure 2: Updation/revocation of sensing service.

0a. The AF sends an Nnef_SensingService_Authorization_Update request to the NEF. The request may include details such as Service Descriptor, transaction reference ID, Information, Status, Cause for updation.

0b. The NEF forwards the update request as a Nnef_SensingService_Authorization_Update message to the SF. This message may contain previously shared details such as Service Descriptor, transaction reference ID, Information, Status, Cause for updation etc.

0.SF(SDR) provides a successful authorization for the updating the service specific parameters/ updating authorization details for the service e.g., due to a revocation request from the AF, expiration of the sensing request duration, authorization changed in any of the participating NFs or sensing entities.

- The SF sends a Nsf_SensingEntity_Authorisation_Update Message containing (Service Descriptor, transaction reference ID, Information, Status, Cause) to the sensing entities involved in the sensing service to update authorization status for one or more sensing entities. The transaction reference ID denotes a specific transaction within a session (identified by service descriptor) between an NEF and an AF.

- SF then sends a Nsf_ServiceSpecificAuthorisation_UpdateNotify Request (Service Descriptor, transaction reference ID, Information, Status, Cause) message to the NEF to update authorization status for one or more NFs.

- The NEF sends Nsf_ServiceSpecificAuthorisation_UpdateNotify Response message to the SF to acknowledge the authorization update.

- If the authorization is revoked, the service specific parameters present in SF(SDR) are deleted and NEF removes the service specific parameters from other participating NFs.

- The NEF informs the AF that the service parameters authorisation status has changed by sending Nnef_ServiceParameter_Notify Request (Service Descriptor, Status, Cause) message to the AF to update the authorization/ inform that the service has been revoked.

- The AF responds to the NEF with Nnef_ServiceParameter_Notify Response message.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts are envisioned on the existing network functions:

NEF:

- For supporting sensing services, NEF performs functions such as exposure of capabilities and events for other NFs, translation of information between an AF and another NF, support of non-IP data delivery etc.

NRF:

- NRF maintains the NF profile of available NF instances related to sensing (e.g., SF/ (SCF, SDF and SDR)) and their supported services

NWDAF:

- For a given sensing task, NWDAF may support data collection from SF (SDR, SDF and/or SCF), whenever required. It also supports all tasks related to computation of analytics on sensing data such as data collection, model provisioning and training, accuracy information computation etc.

* * * * End of Changes * * * *