3GPP TSG-WG Meeting #169	S2-2505059

# Fukuoka, Japan, May 19 - 23, 2025

Source:	Rakuten Mobile, NIST

Title:	Pseudo-CR on New Solution Key Issue #2/Use Case #1: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane

Document for:	Approval

Agenda item:	20.3.1

Spec:	3GPP TR 23.700-04

Version:	0.1.0

Work Item:	FS_AIML_CN_Ph2 / Rel-20

# 1. Introduction/Discussion

This paper proposes a solution to Key Issue #1 for Use Case 2, AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane.

# Text Proposal

It is proposed to agree the following key issue(s) into TR 23.700-04.

* * * First Change * * * *

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Erroneous packets: Packets that have some kind of error but may still be readable or useable e.g., a packet with a corrupted checksum due to transmission errors.

Faulty packets: Packets that are considered defective or non-functional, typically making them unusable e.g., a packet with mismatched header fields.

Identical packets: Packets that are exactly the same (duplicate copies) and may cause redundancy or network congestion.

Malformed packets: Packets that do not conform to the protocol specifications, making them unprocessable e.g., a packet missing a required field, such as a GTP-U packet with an incorrect header length.

Unknown packets: Packets that the UPF does not recognize or cannot classify e.g., A packet with an unrecognized protocol identifier (e.g., a GTP-U packet with an unknown extension header type).

* * * Next Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| X |  | x | x | x |

* * * Next Change (All New Text) * * * *

## 6.X 	Solution #1: New Analytical ID to enhance the 5GC analytics to support efficient performance of user plane

### 6.X.0	High-level solution principles

A new analytic ID for NWDAF “User Plane Packet Analysis” is introduced to enhance the 5GC to support efficient performance of user plane.

In this solution, its proposed that UPF provides information pertaining to various abnormal packets (e.g., erroneous, faulty, identical, malformed, unknown etc.) or RTT, latency to NWDAF. The information can be provided by the UPF during a time window by taking a sample of data traffic. It is assumed that UPF does not continuously provide this information in order to minimize the load on Control Plane NFs thus without significantly increasing the reporting overhead from the UPF.

Based on the statistics the network operator can analyse which applications contribute most to the abnormal packets due to reasons such as change of IP addresses of the application server, incorrect configurations etc. and take corrective actions.

Based on the predictions, the NF consumer such as PCF or SMF can choose a different UPF (i.e., UPF re-routing), different slice at specific times when the possibility of abnormal traffic will be high for some reasons. PCF can also enforce revised QoS or rate-limiting policy for affected UEs.

### 6.x.1	Description

This solution addresses Key Issue # 2

The consumer of User Plane Packet Analytics may request the NWDAF analytics information regarding the scenario when User Plane data fails to (re)transmit, due to e.g., erroneous, faulty, malformed, identical, or unknown packets. This could be statistics for an Analytics target period in the past in a certain area or the likelihood of a faulty/erroneous packets for an Analytics target period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e. a Request-Response model).

The service consumer may be a NF (e.g. SMF, PCF, AF, UPF).

The request includes the following parameters:

-	Analytics ID = "User Plane Packet Analysis";

-	Target of Analytics Reporting: a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, a particular UPF or List of UPFs

-	Analytics Filter Information:

-	Application ID List: Target Application ID that User Plane Packet Analysis should be provided for;

-	Source or destination IP or port address

-	AF ID

-	Area of Interest (AoI) and/or S-NSSAI which restricts the scope of User Plane Packet Analysis to the specific area and/or slice.

-	DNN

-	S-NSSAI (optional);

Analytics Reporting Information:

-    Sampling period of 10 seconds, 5 minutes 30 minutes etc. (and the reporting time could be larger, once every 30 minutes)

-	Expected Report Time: indicates the time limit by which the analytics results need to be received, e.g. 10 seconds, 5 minutes, etc.;

- 	An Analytics target period indicates the time period over which the statistics or predictions are requested;

- 	Optionally, preferred level of accuracy of the analytics;

### 6.x.2	Input Data

The NWDAF collects input data as described in below tables.

The NWDAF may also use output of the User Data Congestion Analytics as specified in clause 6.8 as input data for analytics.

Table 6.x.2-1: Data Collected from the NF and OAM

| Information | Source | Description |
| --- | --- | --- |
| Slice related data | NSACF | Number of users and number of sessions per slice (optional) |
| Area of Interest | AMF | Tracking Area Identities (TAIs) or cell IDs associated with problematic packets |
| Time | AMF | Time zone information for temporal patterns |
| DNN | SMF | (Data Network Name) information |
| PDU Session | SMF | PDU session identifiers |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels, load level for particular NF (AMF, SMF, UPF etc.) |

Table 6.x.2-2: Data Collected from the UPF/AF

| Information | Source | Description |
| --- | --- | --- |
| Application Information |  |  |
| >Application ID | UPF or AF | Application identifier as defined in clause 5.8.2 of TS 23.501 |
| >Location of Application | UPF | List of Internet applications represented by DNAI(s). |
| UE Information |  |  |
| >UE IP address | UPF | UE IP address. |
| >S-NSSAI | UPF | Identifier of the slice. |
| >DNN | UPF | The DNN for which the PDU session is established. |
| Traffic measurement information |  |  |
| >Data Volume UL/DL |  |  |
| >Throughput UL/DL | UPF or AF | Average Throughput UL/DL over the measurement period. |
| >Throughput UL/DL (peak) | UPF or AF | Peak Throughput UL/DL over the measurement period. |
| >Achieved sampling ratio | UPF | Sampling ratio achieved by UPF |
| >Measurement period | UPF or AF | Measurement period. |
| >Timestamp | UPF or AF | Time when measurements are taken. |
| QoS Flow Information |  |  |
| >QoS Flow Identifiers | UPF | QoS Flow Identifiers (QFIs) associated with problematic packets |
| >IP Packet Filter Set | UPF or AF | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 |
| >QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| >QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Error Information |  |  |
| >Retransmit packets | UPF | The observed number of packet retransmission |
| >>Reason for retransmitting | UPF | Congestion, erroneous, faulty, malformed, identical, or unknown packets etc. |
| >Dropped packets | UPF | The observed number of packet dropped |
| >>Reason for dropped packets | UPF | Congestion, erroneous, faulty, malformed, identical, or unknown packets etc. |
| >>Time stamp | UPF | Timestamps of problematic packets |

### 6.x.3	Output Analytics

The output analytics of signalling storm provided by NWDAF is defined in Table 6.22.3-1, and Table 6.22.3-2.

Table 6.x.3-1: Statistics

| Information | Description |
| --- | --- |
| List of User Plane Packet Analytics (1..max) | List of User Plane Packet Analytics. |
| UPF(1..n) | List (Ranking) of UPF that contribute most to erroneous, faulty, malformed, identical, or unknown packets |
| >Area of Interest | A list of TAIs or Cell IDs |
| > Applicable Time Window | The time period that the analytics applies to. If a Temporal granularity size was provided in the request or subscription, the duration of the Applicable Time Window is greater than or equal to the Temporal granularity size. |
| >DNN | Data Network Name |
| > Network Status Indication | Congestion Level (High/Medium/Low (context for packet drops)) |
| Packet Error Statistics |  |
| > Number of erroneous packets | such as CRC failures, checksum errors. |
| >Number of faulty packets | Such as Out-of-sequence, retransmissions. |
| >Number of malformed packets | Such as Invalid headers (e.g., truncated TCP). |
| >Number of identical packets | Such as Duplicates (e.g., retransmission storms). |
| >Number of unknown packets | Such as Unclassified/unrecognized protocols. |
| Latency Metrics |  |
| > RTT | (Round-Trip Time): Spikes correlate with congestion. |
| > Latency | UL/DL delays hint at UPF processing bottlenecks. |
| Top Applications |  |
| > List of top applications in UL (0..NU) | The list of applications that contribute the most to the “erroneous, faulty, malformed, identical, or unknown packets “in the UL direction. |
| >> Application ID | Application identifier as defined in clause 5.8.2 of TS 23.501 |
| >> IP Packet Filter Set | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| > List of top applications in DL (0..ND) | The list of applications that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the DL direction. |
| >> Application ID | Application identifier as defined in clause 5.8.2 of TS 23.501 |
| >> IP Packet Filter Set | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| Top UEs |  |
| > List of top UE in DL (0..ND) | The list of top UEs that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the DL direction. |
| > List of top UE in UL (0..ND) | The list of top UEs that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the UL direction. |

Table 6.x.3-2: Predictions

| Information | Description |
| --- | --- |
| List of User Plane Packet Analytics (1..max) | List of User Plane Packet Analytics. |
| UPF(1..n) | List (Ranking) of UPF that contribute most to erroneous, faulty, malformed, identical, or unknown packets |
| >Area of Interest | A list of TAIs or Cell IDs |
| > Applicable Time Window | The time period that the analytics applies to. If a Temporal granularity size was provided in the request or subscription, the duration of the Applicable Time Window is greater than or equal to the Temporal granularity size. |
| >DNN | Data Network Name |
| > Network Status Indication | Congestion Level (High/Medium/Low (context for packet drops)) |
| Packet Error Statistics |  |
| > Number of erroneous packets | such as CRC failures, checksum errors. |
| >Number of faulty packets | Such as Out-of-sequence, retransmissions. |
| >Number of malformed packets | Such as Invalid headers (e.g., truncated TCP). |
| >Number of identical packets | Such as Duplicates (e.g., retransmission storms). |
| >Number of unknown packets | Such as Unclassified/unrecognized protocols. |
| Confidence |  |
| Latency Metrics |  |
| > RTT | (Round-Trip Time): Spikes correlate with congestion. |
| > Latency | UL/DL delays hint at UPF processing bottlenecks. |
| Top Applications |  |
| > List of top applications in UL (0..NU) | The list of applications that contribute the most to the “erroneous, faulty, malformed, identical, or unknown packets “in the UL direction. |
| >> Application ID | Application identifier as defined in clause 5.8.2 of TS 23.501 |
| >> IP Packet Filter Set | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| > List of top applications in DL (0..ND) | The list of applications that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the DL direction. |
| >> Application ID | Application identifier as defined in clause 5.8.2 of TS 23.501 |
| >> IP Packet Filter Set | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 |
| >> Percentage | The application's throughput as a percentage of the total throughput in the Area of Interest. |
| Confidence |  |
| Top UEs |  |
| > List of top UE in DL (0..ND) | The list of top UEs that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the DL direction. |
| > List of top UE in UL (0..ND) | The list of top UEs that contribute the most to the " erroneous, faulty, malformed, identical, or unknown packets “in the UL direction. |
| Confidence |  |

### 6.x.4	Procedures

The NWDAF can provide information on network signalling storm as follows (Figure 6.x.4-1).

Figure 6.x.4-1: User Plane packet analytics provided by NWDAF

1. The consumer NF sends a request to NWDAF for the User Plane Packet Analytics The request additionally includes thresholds such as confidence level and accuracy of detection:

-	The Analytics ID is set to “User Plane Packet Analytics”. The target for analytics reporting is set to a particular or set of UPFs. Analytics filters may be provided as shown in clause 6.x.1

-	The consumer NF can request statistics or/and predictions for a given Analytics target period.

-	The consumer NF may also subscribe to "NF load analytics" as depicted in clause 6.5 and trigger the signalling storm analytics based on output of the NF load analytics, e.g. CPU usage is over 80%.

2.	The NWDAF retrieves Input data from NF’s and OAM, using Nnf_EventExposure_Subscribe, or from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation. If the request is to provide the list of applications that contribute the most to the abnormal traffic, then the NWDAF collects input data from the AF for the applications being served by AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or collected from the UPF or collected from both, AF and UPF.

3. The NWDAF derives the required analytics with the collected data described in clause 6.x.2

4. The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

5. The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF

6. NWDAF subscribes to UPF, AF, OAM and optionally to NSACF and collects the information. If a request is to provide the list of applications that contribute the most to the traffic, the NWDAF subscribes to the service data from AF by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) or from the UPF or from both.

7. The NWDAF derives requested analytics.

8. The NWDAF provides the analytics to the NF.

9. A change of UPF load corresponding to crossing a threshold set by the NWDAF at steps 7-8 is detected by OAM and notified to NWDAF.

10. The AF notifies the NWDAF with the input data as defined in table 6.x.2-2.

11. The UPF provides the NWDAF with the input data as defined in table 6.x.2-2.

12. The NWDAF derives new analytics.

13. The NWDAF provides a notification for analytics for the user data congestion to the NF.

### 6.X.5	Impacts on services, entities and interfaces

UPF:

-	Collects User Plane Packet Analysis data information upon request from NWDAF.

-	Sends User Plane Packet Analysis for data reporting to NWDAF as Input Data.

NWDAF:

-	Supports providing subscriptions and/or requests User Plane Packet Analysis.

-	Supports deriving statistics and/or predictions based on Analytics consumer requests.

Consumer NF (SMF, PCF, AF):

-	Supports subscribing and/or requesting UP traffic pattern analytics from NWDAF.

-	Supports executing the UPF performance enhancement based on the analytics data from NWDAF.