3GPP TSG-SA2 Meeting #169	S2-2505143

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0739 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Positioning Information Collection for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR captures the stage 2 impacts of RAN 3 LS S2-2504525 "LS on AI/ML Positioning Case 3a". The CR proposes the procedures for positioning information collection for NG-RAN and it covers the cases for Positioning Information Transfer by LMF for an ongoing UE positioning and for UE(s) with no ongoing positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 6.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the positioning information collection for NG-RAN cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change* * * *

## 6.X	Procedures for Positioning Information Collection for NG-RAN

### 6.X.1	Positioning Information Transfer by LMF for ongoing UE positioning procedure

NG-RAN, as part of data collection for its AI/ML assisted positioning, may require positioning information from LMF. This procedure is used for transferring positioning information to NG-RAN for a UE currently undergoing positioning. The procedure is initiated only when the NG-RAN requests for positioning information collection for the UE being positioned.

NOTE:	The procedure is applicable only for Uplink-based UE Positioning method.

Figure 6.X.1-1: Positioning information transfer for ongoing UE positioning procedure

Pre-condition: Uplink based Positioning procedure is ongoing for a UE in LMF.

1.	As part of an ongoing UE positioning, the LMF sends NRPPa measurement request to multiple NG-RAN nodes to collect UE measurement data.

2.	Upon receiving measurement request from LMF, NG-RAN node(s) may decide to initiate data collection for the UE.

3.	NG-RAN node(s) may include data collection indication to LMF in NRPPa measurement response.

4.	The LMF checks whether the UE, with its corresponding SUPI, provides the user consent for data collection with UDM using Nudm_SDM_Get including subscription data type "User consent". If user consent is not granted, then the remaining steps are skipped, and a failure indication is to be sent to NG-RAN.

5.	Once LMF has the positioning information, it sends a Positioning Data Collection Report to the NG-RAN. The message contains information related to UE location and relevant correlation information. The correlation information enables the NG-RAN to map the information related to the UE’s location to the corresponding UE’s positioning measurement data stored within it.

If LMF is unable to provide the positioning information, a failure indication is to be sent to NG-RAN.

Editor’s Note:	The failure indication to NG-RAN and the contents of the Positioning Data Collection Report are to be decided by RAN WGs and FFS.

### 6.X.2	Positioning Information Transfer by LMF for selected UE(s)

The NG-RAN initiates this procedure for requesting positioning information from the Core Network. The procedure involves the UE(s) selected either by NG-RAN or by Core Network.

Figure 6.X.2-1: Positioning information transfer by LMF for selected UE(s)

1.	NG-RAN decides to initiate data collection. NG-RAN may choose the serving UE(s) whose positioning information needs to be collected.

2.	NG-RAN sends the Positioning Information Request to the AMF. This message may contain the NGAP IDs (RAN-UE NGAP ID and AMF-UE NGAP ID pairs as per Clause XX TS 38.413) of the UEs served by the NG-RAN.

Editor’s Note:	The Positioning Information Request is a NGAP message. The contents of the message to be decided by RAN WGs is FFS.

3.	The AMF interprets the Positioning Information Requests and maps the corresponding NGAP IDs of the UEs to SUPIs, if present in the request from NG-RAN. The AMF initiates Nlmf_DataExposure_Subscribe service of LMF which contains the interpreted NG-RAN requested parameters, requesting NG-RAN information and the list of UEs, in the form of SUPIs.

4.	If the list of UEs is not provided by NG-RAN, the LMF subscribes to the AMF(s) to retrieve the list of SUPIs located in an area of interest(like RAN Node ID) and optionally the UE Positioning Capability for each UE using Namf_EventExposure_Subscribe service request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest, Event Filter="RAN Node ID", [indication of requesting UE Positioning Capability]).

5.	The AMF sends Namf_EventExposure_Subscribe response containing the list of SUPIs in the area of interest. If UE Positioning Capability is also requested in step 4, AMF includes UE Positioning Capability and optionally UE User Plane Positioning Capabilities, if available, for each UE in the response message sent to LMF.

6.	The LMF checks whether each SUPI provides the user consent for data collection with UDM using Nudm_SDM_Get including subscription data type "User consent". If user consent is not granted, then the remaining steps are skipped for that UE, and a failure indication is to be sent to NG-RAN.

7.	The LMF initiates UE positioning and obtain the information related to UE location.

8.	Once LMF has the positioning information, it sends a Positioning Data Collection Report to the NG-RAN. The message contains information related to UE location and relevant correlation information. The correlation information enables the NG-RAN to map the information related to the UE’s location to the corresponding UE’s positioning measurement data stored within it.

If LMF is unable to provide the positioning information, a failure indication is to be sent to NG-RAN.

Editor’s Note:	The failure indication to NG-RAN and the contents of the Positioning Data Collection Report are to be decided by RAN WGs and FFS.

# * * * * End of the Change* * * *