3GPP TSG-WG SA2 Meeting #169	S2-2505167

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxx)

Source:	Huawei, HiSilicon

Title:	KI#3, New Sol: NF selection profile

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposed the NF selection as the solution of the KI#3.

# 1.	Introduction

In the approved SID SP-250417, the NF selection is considered as one of the candidate key issues to be resolved for energy study in Rel-20:

- WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

Current NF selection defined in clause 6.3 of TS 23.501 mainly considers criteria such as Location information, Data Network Name, slicing information, capability, local policy, load condition etc. There is no consideration on energy related criteria, such as maximize the energy efficiency of the network, trade the performance with energy consumption following the SLA agreement, etc., and may lead to a sub-optimal result from the energy perspective.

This document uses Rel-19 resolution as the starting point, and proposes 2 solutions solution based on Energy Priority information.

-	Energy Priority Information. Additional and optional for the selection. It indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. It is determined based on operator policy and provided to NRF considering also the various energy characteristics of the NF, for example the energy consumption in function of the load, the actual energy consumption, whether is powered with renewable energy, etc. Which is the energy related information and how they are considered in order to determine the Energy Priority Information parameter is based on operator configurations.

-	Energy Priority Information. Additional and optional for the selection. It indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. It is determined based on operator policy and provided to NRF considering also the various energy characteristics of the NF, for example the energy consumption in function of the load, the actual energy consumption, whether is powered with renewable energy, etc. Which is the energy related information and how they are considered in order to determine the Energy Priority Information parameter is based on operator configurations.

This solution proposes to enhance the NF discovery and selection related functionalities for energy aware NF selection..

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| Y |  |  | X |
| X |  |  | X |

# * * * * 2nd change (All text new) * * * *

## 6.Y	Solution #Y: NF selection based on energy information priority

### 6.Y.1	High level principles

This solution is for Key Issue #3: Whether and how to support NF (re)selection based on energy related information.

The Energy Priority Information, which indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF, is proposed to be added to the NF Profile.

The principles of Energy Priority Information are:

-	If the energy is the criterion for selecting an NF, the NRF provides such criteria altogether with the other criteria and if more them one NF matches the criteria the Energy Priority Information parameter can be used to select those with higher priority;

-	The definition of the priority is determined by the Operator based on their policies and considering the various energy characteristics of the NF, some possible example (not limited to the list below):

-	The NFs with renewable power supply have higher Energy Priority, to the NF with higher renewable ratio can be assigned higher priority;

-	The energy priority information can be changed when the NF enters in Energy Saving state;

The consumer NF of NRF discovery service may select the NF or NF instance considering Energy Priority Information in the energy related information in the discovered NF profile.

If the energy is a criterion for the selection of the NF in case of multiple NF satisfying the other criteria, the Energy Priority can be considered for selecting the NF with high priority, based on operator policy.

### 6.Y.2	Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

-	Service provider NF registers new energy related information in its NF profile at the NRF and updating such information at the NRF when needed. Or alternatively, such new energy related information can be (re-) configured by OAM.

-	Enhancing the NF Discovery procedure with new energy-related NF discovery query parameters (e.g., energy aware NF discovery policy/criteria) and based on new energy-related policies at the NRF;

-	NF service consumer taking into account the new energy related information from the NF profiles (defined in Table 6.Y.2-1) discovered from NRF using discovery procedure (and subsequent updates if subscribed to), for the selection of a target 5GC NF;

NRF maintains also energy related information as part of NF profile for registered NF instance. Such energy related information includes the example in Table 6.Y.2-1 described.

Table 6.Y.2-1: Energy related information

| Information | Description | Category |
| --- | --- | --- |
| Energy Priority Information | This parameter indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. | Optional |

The Energy Priority Information indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF.

NOTE:	The information within NF profile specified in Table 6.Y.2-1 is expected not to change frequently, e.g., constantly reflecting changes in energy consumption.

### 6.Y.3	Procedure

Existing procedure of NF selection defined in TS 23.501 [2] can be reused.

### 6.Y.4	Impact

The impacts are:

-	NRF

-	support the NF profile extension

-	extension of SBI to send request of NF based on energy indication support

-	other NFs

-	extension of SBI to send request of NF based on energy indication support

# * * * * 3rd change (All text new) * * * *

## 6.X	Solution #X: NF selection based on Scheduled Switch-Off Time information

### 6.X.1	High level principles

This solution is for Key Issue #3: Whether and how to support NF (re)selection based on Scheduled Switch-Off Time information.

It is proposed to add the Scheduled Switch-Off Time, which can be used for e.g., UPF selection for PDU Session with SSC mode 1, and SMF may not select the UPF about to be in energySaving state in that case.

When SMF selects the UPF, the SMF can also make use of the NF Profiles (i.e., Scheduled switch-off time) by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

### 6.X.2	Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

-	Service provider NF registers Scheduled Switch-Off Time information in its NF profile at the NRF and updating such information at the NRF when needed. Or alternatively, such new energy related information can be (re-) configured by OAM.

-	Enhancing the NF Discovery procedure with new energy-related NF discovery query parameters (e.g. energy aware NF discovery policy/criteria) and based on new energy-related policies at the NRF;

-	NF service consumer taking into account the Scheduled switch-off time information from the NF profiles (defined in Table 6.X.2-1) discovered from NRF using discovery procedure (and subsequent updates if subscribed to), for the selection of a target 5GC NF;

NRF maintains also the Scheduled switch-off time information as part of NF profile for registered NF instance. Such energy related information includes the example in Table 6.X.2-1 described.

The difference between the Scheduled Switch-Off Time information and the Energy state specified in TS 28.310 [X1] indicates the current state of the NF, i.e., the notEnergySaving state indicates that the NF is in the state where no Energy Saving is in progress, while the energySaving state indicates that the NF is in the state where no Energy Saving is progress. The Scheduled Switch-Off Time indicates the time when the NF will be switched off/on and therefore the time from which the NF will be no more available in the network or will be again available in the network.

The Scheduled Switch-Off Time may take into account when the NF needs to be selected, for example in case of UPF if a PDU session uses SSC mode 1, the SMF may select the UPF taking into account that the UPF which may be switch-off at one point in future and therefore is not the preferable UPF to be selected, while for PDU session with SSC mode 2 & 3 this may not be a problem. The Scheduled Switch-Off Time may be taken into consideration in different way for different NFs. How the Scheduled Switch-Off Time is considered, it is implementation specific.

Table 6.X.2-1: Energy related information

| Information | Description | Category |
| --- | --- | --- |
| Scheduled switch-off time | The schedule of the energy states, e.g., the time of entering to/exiting from the Energy Saving State regarding energy saving. (NOTE 1) | Optional |
| NOTE 1: Energy states refers to state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310. This can be used for e.g., UPF selection for PDU Session with SSC mode 1, and SMF may not select the UPF about to be in energySaving state in that case. |  |  |

The consumer NF of NRF discovery service may select the NF or NF instance considering the Scheduled switch-off time information in the discovered NF profile.

NOTE:	The information within NF profile specified in Table 6.X.2-1 is expected not to change frequently, e.g., constantly reflecting changes in energy consumption.

### 6.X.3	Impact

The impacts are:

-	NRF

-	support the NF profile extension.

-	extension of SBI to send request of NF based on Scheduled switch-off time support.

-	SMF

-	support of UPF selection based on energy information.

-	support of SBI extension toward NRF.

# * * * * End of change * * * *