3GPP TSG-SA WG2 Meeting #169	S2-2505198

Fukuoka, Japan, May 19 – 23, 2025													(revision of S2-2503532)

Source:	Xiaomi

Title:	KI#1: New Sol: CP based solution for supporting IMS voice over NB-IoT via GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1.

1. Introduction/Discussion

A solution is proposed to address Key Issue#1 to support IMS voice call over NB-IoT via GEO connecting to EPC using Control Plane CIoT EPS Optimisation.

IMS signalling and IMS voice data have different QoS requirements, e.g. IMS signalling requires high reliability while IMS voice data requires short delay. To meet the different requirements for the data flows of IMS voice service, more than one bearer is required to carry IMS signalling and IMS voice data respectively. Dedicated bearer is needed to provide QoS guarantee for the IMS voice data transmission. As specified in TS 23.401, Dedicated bearer, and GBR bearers are not supported in the NB-IoT RAT.

Moreover, "IMS voice over PS session Supported Indication" and "Voice domain preference and UE’s usage setting" were defined in TS 23.401 to support IMS voice service. But these are not applied to the NB-IoT RAT.

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over GEO via NB-IoT access connecting to EPC.

2. Proposal

It is proposed to capture the following text in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| x | x |  |  |  |

6.X	Solution #x: Support of QoS differentiation for IMS voice service over NB-IoT NTN

6.x.1	High level principles

The following principles are applied to this solution:

-	Proceduce on Data Transport in Control Plane CIoT EPS Optimisation is used as baseline.

-	EPS enhancement to support dedicated bearer for the IMS voice service over NB-IoT via GEO.

-	Dedicated bearer(s) can be pre-established during IMS PDN connection establishment to reduce delay for the IMS voice service over NB-IoT via GEO

6.x.2	Description

This solution addresses KI#1 to support IMS voice call over NB-IoT via GEO connecting to EPC

The TS 23.401 specifies only default bearer is supported when using NB-IoT (i.e. dedicated bearer is not supported). This may not support the QoS requirement of an IMS voice service considering the IMS signalling and the IMS voice data transmission have different QoS requirement. Supporting multiple bearers for IMS voice service, i.e. more than default bearer, needs to be enhanced by the EPS connecting to the NB-IoT NTN via GEO.

As specified in TS 23.401, if an IMS voice service over PS session is supported by the serving PLMN, it shall send a support indication toward the UE in order to be taken by the UE with “IMS voice over PS” capability into account when establishing a voice bearer for the IMS voice. However, the support indication is not supported over the NB-IoT RAT.

A UE supporting IMS voice shall send Voice domain preference and UE’s usage setting to EPC. This was defined in TS 23.401 but is not applied to the NB-IoT RAT.

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over NB-IoT via GEO connecting to EPC. The solution is based on CP CIoT EPS Optimisation

Following procedure is an example to create one default bearer and one dedicated bearer to suppot IMS voice service

6.x.3	Procedures

Figure 6.x.3.1: IMS signalling and voice data delivery in CIoT EPS Optimisation over NB-IoT via GEO satellite

Attach

0. A GEO satellite of a NB-IoT access may broadcast its support of IMS voice service over NB-IoT. This may help UE to determine whether camp on the cell.

1. the UE, camping on the NB-IoT cell supporting IMS voice service, initiates the Attach procedure by the transmission of an Attach Request to the MME, including the Preferred Network Behaviour, Voice domain preference and UE’s usage settings. The Preferred Network Behaviour indicates "Control Plane CIoT EPS Optimisation is preferred". The Voice domain preference sets "IMS PS Voice only" and the UE’s usage settings sets "voice-centric".

2. The MME procees the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates Control Plane CIoT EPS Optimisation is supported in the Supported Network Behaviour information. If IMS voice service over NB-IoT is supported when the UE is using the NB-IoT NTN, the Support indication is contained in the message as well.

PDN connection establishment for IMS voice service

3. Based on the IMS voice over NB-IoT Support indication, the UE can initiate a PDN connection establishement for IMS service by using an APN related to the IMS service.

4. In order to provide differentiate QoS to the IMS signalling and IMS voice data, the network can be enhanced to support establishing more than one bearer for the IMS voice service (e.g, default bearer for IMS signalling and one dedicated bearer for IMS voice data). Moreover, considering the dedicated bearer establishment triggered by IMS call setup will cause additional delays, the dedicated bearer can be pre-established combing with the default bearer establishment. Two EPS bearers (EPS bearer ID marked as EBI-1 and EBI-2) are established for the IMS PDN connection over NB-IoT.

Editor’s note:	Whether new QCI is needed for supporting IMS voice service over NB-IoT NTN via GEO is FFS.

5. The MME sends PDN Connectivity Accept including EBI-1 and EBI-2 to the UE.

NOTE:	the PDN connection for the IMS voice service can be established during the Attachment procedure or as a separate procedure.

IMS call setup

6. The UE can initiate IMS signalling negotiation with remote UE by using CP CIoT EPS Optimisation as defined in clause 5.3.4B.2. The NAS PDU carries the encrypted uplink IMS signalling and EBI-1 to the eNodeB of the NB-IoT NTN.

6a. the eNodeB of the NB-IoT NTN, based on configuration, may retrieve the EPS negotiated QoS profile from the MME. The QoS profile may be applied to prioritise between requests from different UEs.

7. The NAS PDU sent in step 6 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

8. The MME checks the integirity of the incoming NAS PDU and decrypts the data it contains.

9. Based on the received EBI-1, the MME mappes the uplink IMS signalling data to the default bearer (i.e. EPS bearer-1) and deliveries it to the P-GW.

10. The Downlink IMS signalling is delivered via the default bearer, details can refer to steps 9-12 of Figure 5.3.4B.2-1 of TS 23.401.

11. The IMS signalling negotiation may trigger P-CSCF to send service request to the PCRF which provides the QoS requirement for the IMS voice data transmission. The PCRF may update the PCC rule for the IMS voice data and provides it to the P-GW.

12. The updated PCC rule may trigger P-GW to initiate bearer update to the S-GW and MME.

13. When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer (i.e. EPS bearer-2).

14. The MME encrypts and integrity protects Downlink IMS voice data.

15. The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message.

16. The NAS PDUwith IMS voice data is delivered to the UE via a Downlink RRC message.

17. The uplink IMS voice data transmission via the dedicated bearer can refer to steps 16-19 of Figure 5.3.4B.3-1 of TS 23.401.

Editor’s note: 	Whether using one or more than one SRB to deliver NAS PDU of IMS signalling data and NAS PDU of IMS voice data is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to provide QoS to the IMS voice service via SRB is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to enhance or optimize IMS registration, IMS call setup procedures to enable IMS voice over NB-IoT NTN via GEO is FFS.

6.x.4	Impacts on services, entities and interfaces

UE:

-	Request for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support multiple bearers when using NB-IoT NTN via GEO for IMS voice service

-	Set and provide Voice domain preference and UE’s usage setting when using NB-IoT NTN via GEO for IMS voice service.

MME:

-	Determine whether to support IMS voice and provide IMS voice service over PS session support indication to UE when using NB-IoT NTN via GEO

-	Support for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support to establish multiple bearers, including GBR bearer for UE using IMS service over NB-IoT NTN via GEO

S-GW/P-GW:

-	Support to establish multiple bearers, including GBR bearer, for UE using IMS service over NB-IoT NTN via GEO

* * * End of Change * * * *