3GPP TSG-WG SA2 Meeting #169	S2-2505212

19th – 23th May 2025, Fukuoka, Japan

Source:	China Mobile

Title:	Update on AIOTF selection

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to update the AIOTF Selection mechanism in TS 23.369.

- Introduction

In clause 5.3.1 of current specification, when NEF or AF selects AIOTF, Target Area information is mentioned as the selection factor. However the following issues need further clarifications:

- It is not clear how the Target Area information is derived and what the Target Area information includes. It is proposed to clarify that NEF maps the external Target Area information received from the AF to Target Area information and uses it to discover serving AIOTF via NRF. According to LS S2-2504523/R3-252481, AIoT Area information is defined by RAN3 and is used to represent the supporting areas of NG-RAN nodes or readers, which is available in OAM. It is therefore proposed to clarify that Target Area information includes the list of AIoT Areas. This mapped Target Area information will be sent to AIOTF and further used by AIOTF to select NG_RAN node.

- In real deployment, AIOTF should be able to be deployed flexibly, i.e. AIOTF can be deployed to serve specific customers, or specific IoT devices . Therefore it shall be also supported to select AIOTF based on AF identifier or specific AIoT device ID provided by AF.

- It needs description that AIOTF registers the above information it serves to NRF as NF profile to support NRF based AIOTF selection.

- It is also mentioned in the specification that NEF may use last known AIOTF instances stored in ADM. But it is not clear how NEF handles the situation that multiple AIoT devices have multiple last known AIOTF instances stored. It is proposed to clarify that NEF handles it by implementation for simplicity.

- It is clarified that if multiple AIOTFs are selected, the NEF need to send multiple AIoT service requests to each AIOTF with corresponding Target Area Information.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369.

# * * * * First change * * * *

### 5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is supported by the NEF and AF to determine an AIOTF to handle AIoT Services.

The AIOTF may register its NF profile containing list of serving AIoT Area(s), serving AF ID(s) and serving AIoT device ID(s) as specified in TS 23.502 [4] to the NRF.

When the NEF or AF performs AIOTF discovery and selection for an AIoT service operation request, the following applies:

-	The NEF or AF shall utilize the NRF to discover AIOTF instance(s) unless AIOTF information is available by other means, e.g. locally configured in the NEF or AF. The NEF or AF may provide to the NRF one or the combination of the following information , when trying to discover AIOTF instance(s):

-	Target Area information mapped from external Target Area information received in AIoT service operation request;

NOTE 1:	Target Area information includes the list of AIoT Area(s) which is locally configured or provisioned from OAM.

-	AF identifier(s);

-	AIoT device ID(s).

The NRF provides AIOTF instance(s) to the NEF or AF.

NOTE 2:	The local configuration in the NEF can be used for example to select a specific AIOTF instance for the request from a given AF.

-	The NEF or the AF selects the AIOTF instance(s) based on the available AIOTF instance(s) (obtained from the NRF or locally configured)

When AIoT Device ID information indicates individual AIoT device(s), the NEF may select AIOTF(s) by taking into account the last known AIOTF instance(s) for those device(s) from ADM. If there are multiple AIOTF instances stored for different AIoT devices, whether and how the NEF selects one or multiple AIOTF(s) for the request is up to NEF implementation.

The NEF forwards the AIoT service request to the selected AIOTF. If multiple AIOTFs are selected, the NEF sends the AIoT service request to each selected AIOTF along with its corresponding Target Area information.

# * * * * End of changes * * * *