SA WG2 Meeting #169	S2-2505227

19 - 23 May, 2025, Fukuoka , Japan

Source:	CATT

Title:	New solution of optimized IMS call procedure for GEO NB-IoT access

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This paper proposes new solutions for KI#2 IMS enhancement for GEO NB-IoT NTN access.

1	Discussion

This solution proposes to use compact form of SIP headers and skip precondition procedure to reduce the call setup time for GEO NB-IoT access.

2	Proposal

It is proposed to include the following changes in TR 23.700-19.

* * * * Start of Changes * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[xx]	IETF RFC 3312: "Integration of resource management and Session Initiation Protocol (SIP)".

[yy]	IETF RFC 3261: "SIP: Session Initiation Protocol".

* * * * Next Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * * Next Change * * * *

6.X	Solution #X: <Solution Title>

6.X.1	High level principles

This solution is to address Key Issue #2: IMS enhancement for GEO NB-IoT NTN access.

6.X.2	Description

This solution re-uses existing mechanism as much as possible to reduce the call setup time:

- Mandatory to use compact form of SIP headers.

- Skip precondition procedure.

6.X.3	Procedures

Figure 6.x.3-1 depicts the signalling flows of the solution.

Figure 6.x.1-1: SIP call procedure

In this figure, P-CSCF A and P-CSCF B are the serving P-CSCF of the MO UE and MT UE respectively, and not all the IMS or HSS nodes are shown. The two UEs may be served by same nodes.

1.	MO UE starts a call by sending SIP Invite message to serving P-CSCF A. The UE is aware the access is GEO NB-IoT, only a specific codec is included in the SDP offer.

Editor note:	it is FFS what codec is supported for GEO voice, pending on SA4 discussion.

If the codec has been pre-negotiated during the IMS register procedure and P-CSCF has stored the codec supported by the UE, the SIP Invite could be sent without the SDP offer.

NOTE:	The codec pre-negotiated procedure is not included in this solution.

The MO UE should skip the precondition procedure defined in IETF RFC 3312 [xx].

The MO UE should use the compact form of the SIP headers defined in IETF RFC 3261 [yy].

2.	When receiving the SIP Invite, P-CSCF A determine to reserve the dedicated resource (dedicated bearer) or use the existing default resource (i.e., IMS default bearer) for the call.

3.	The SIP Invite is transferred via IMS core to the P-CSCF B serving the MT UE.

4.	P-CSCF B determine to reserve the dedicated resource or use the existing default resource for the call.

5.	If the MT P-CSCF is aware that the MT UE is accessing with GEO NB-IoT and already stores the codec supported by the MT UE, which has pre-negotiated during the IMS register procedure, the P-CSCF may send the SIP Invite with the SDP offer containing the codec or without the SDP offer.

The P-CSCF should use the compact form of the SIP headers defined in RFC 3261.

6.	MT UE replies with 180 Ringing.

7.	MT UE answers the call.

8.	MO UE sends the SIP ACK to the MT UE.

6.X.4	Impacts to Services, Entities and Interfaces

UE:

-  Use compact form of SIP headers when accessing the network via GEO NB-IoT.

-  Skip precondition procedure when accessing the network via GEO NB-IoT.

P-CSCF:

-  Use compact form of SIP headers when aware UE is accessing the network via GEO NB-IoT.

* * * * End of Changes * * * *