SA WG2 Meeting #169	S2-2505228

Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #3, New Sol: IMS emergency service for NB-IoT device using GEO satellite access to EPC

E-UTRA satellite access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4 / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#3 in TR 23.700-19.

1. Discussion

1.1	Introduction

This pCR is proposed to add a solution to key issue#3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

1.2	Existing mechanisms for IMS emergency services using E-UTRAN radio access network

For the specific case where the UE has selected to make an emergency call over EPS, the high level procedures (as described in clause 7.1.1 of TS 23.167) shall be used and the UE shall use the following procedures:

-	A UE shall establish an emergency PDN connection or emergency PDU session and perform an IMS emergency registration before initiating a UE detectable emergency session via E-UTRANN.

-	A UE shall not establish an emergency PDN connection or emergency PDU session if the UE initiated a non UE detectable emergency session and is subsequently informed by the network that the ongoing session is an emergency session.

Figure 1: Terminal Detected Emergency Calls

1.3	Analyses on existing mechanisms

The existing mechanisms for IMS emergency services using EPC network can be reused for NB-IOT device, as follows:

- If the UE can detect the emergency session, the IMS emergency session procedures shall be used as described in clause 7.1.1 of TS 23.167. The NB-IOT device shall set the access type "3GPP-NB-IoT(GEO)" to P-CSCF for the access via GEO satellite connecting to EPC in the IMS request.

- If the UE could not detect the emergency session, the IMS emergency session procedures shall be used as described in clause 7.1.2 of TS 23.167. NB-IOT device sends the session establishment request including "3GPP-NB-IoT(GEO)" to P-CSCF. If the P-CSCF detects that this is a request to establish an emergency session, based upon operator policy (e.g. checking "3GPP-NB-IoT(GEO)"), the P-CSCF may reject the session initiation request with an indication that this is for an emergency session or the P-CSCF in the visited PLMN or the P-CSCF in the home PLMN for a non-roaming UE may allow the session initiation request to continue by inserting the explicit emergency indication in the session request.

- Domain priority and selection rules for NB-IOT device shall reuse the principles as described in Annex H.5 of TS 23.167.

NOTE 1:	The access types are specified in clause 7.2A.4 of TS 24.229.

1.4	Conclusion

Based on analyses in clause 1.3, it is proposed to reuse the general logic of IMS Emergency Services procedure with the difference that the access type can be set to "3GPP-NB-IoT(GEO)".

2. Proposal

It is proposed to add a new solution for KI#3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	3GPP TS 24.229: "IP multimedia call control protocol based on SIP and SDP; stage 3".

* Second change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 | KI#4 |
| #X |  |  | X |  |

* Third change *

6.X	Solution #X: IMS Emergency Service for NB-IoT device using GEO satellite access to EPC

6.X.1	High level principles

The high level principles of the solution include:

-	The existing mechanisms for IMS emergency services using EPC network can be reused for NB-IOT device using GEO satellite access to EPC.

6.X.2	Description

If the UE can detect the emergency session, the IMS emergency session procedures shall be used as described in clause 7.1.1 of TS 23.167 [7]. The NB-IOT device shall set the access type "3GPP-NB-IoT(GEO)" to P-CSCF for the access via GEO satellite connecting to EPC in the IMS request.

If the UE could not detect the emergency session, the IMS emergency session procedures shall be used as described in clause 7.1.2 of TS 23.167 [7]. The NB-IOT device sends the session establishment request including "3GPP-NB-IoT(GEO)" to P-CSCF. If the P-CSCF detects that this is a request to establish an emergency session, based upon operator policy (e.g. checking "3GPP-NB-IoT(GEO)"), the P-CSCF may reject the session initiation request with an indication that this is for an emergency session or the P-CSCF in the visited PLMN or the P-CSCF in the home PLMN for a non-roaming UE may allow the session initiation request to continue by inserting the explicit emergency indication in the session request.

Domain priority and selection rules for NB-IOT device shall reuse the principles as described in Annex H.5 of TS 23.167.

NOTE 1:	The access types are specified in clause 7.2A.4 of TS 24.229 [x].

6.X.3	Procedures

6.X.3.1	UE Detectable Emergency Session

The UE detectable emergency session procedure is shown in Figure 6.x.3.1-1.

Figure 6.x.3.1-1: Terminal Detected Emergency Calls

The procedure in Figure 6.X.3.1-1 follows the IMS Emergency Services procedure defined in clause 7.1.1 of TS 23.167 [7] with the following differences:

-	Step 6: If the NB-IOT device has sufficient credentials to authenticate with the IMS network, it shall initiate an IMS emergency registration with "3GPP-NB-IoT(GEO)" in the P-Access-Network-Info header field as specified in clause 7.2A.4 of TS 24.229 [x]. If the NB-IOT device does not have sufficient credentials to authenticate with the IMS network, the NB-IOT device shall include the "anonymous user", "emergency service" indications and "3GPP-NB-IoT(GEO)" in the P-Access-Network-Info header field in the emergency session establishment request to the P-CSCF as described in clause 7.4 of TS 23.167 [7].

-	Step 7: The NB-IOT device shall initiate the IMS emergency session establishment using the IMS session establishment procedures containing emergency service indication and "3GPP-NB-IoT(GEO)", or the eCall type of emergency service indication in the case of eCall, any registered Public User Identifier and "3GPP-NB-IoT(GEO)".

6.X.3.2	Non UE Detectable Emergency Session

The non UE detectable emergency session procedure follows the IMS Emergency Services procedure defined in clause 7.1.2 of TS 23.167 [7] with the following differences:

As the NB-IOT device could not detect the emergency session, the session establishment request including "3GPP-NB-IoT(GEO)"will be sent to a P-CSCF in the visited PLMN or a P-CSCF in the home PLMN as per a normal session establishment procedure.

In the case that the P-CSCF detects that this is a request to establish an emergency session, based upon operator policy (e.g. checking access type as "3GPP-NB-IoT(GEO)"), the P-CSCF may reject the session initiation request with an indication that this is for an emergency session or the P-CSCF in the visited PLMN or the P-CSCF in the home PLMN for a non-roaming UE may allow the session initiation request to continue by inserting the explicit emergency indication in the session request.

6.X.4	Impacts to Services, Entities and Interfaces

NB-IOT device:

Should be able to detect an emergency session establishment request for UE Detectable Emergency Session procedure.

* End of changes *