3GPP TSG-WG SA2 Meeting #169	S2-2505241

# Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0387 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS broadcast service area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The RAN3 BL CR R3-252523(38.300)/R3-252525(38.413) triggered a review of the relationship between Cell TAI and ISA(Intended Service Area) for MBS service area. Key information from R3-252523 shows that the relationship between Cell TAI and MBS intended service area can be “and/or”: In change of 16.14.5 NG-RAN signalling The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12]. Key information from R3-252525 shows that ISA IE is defined independently from Cell TAI: In change of 9.3.1.209 MBS Service Area Information IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. MBS Intended Service Area List; 0..<maxnoofIntendedAreasforMBS>. >Intended Service Area Information; M; 9.3.1.x. Summary the reason of change: To align with RAN3, Cells, TAIs, and ISAs can exist independently and stand alone. Any combination of Cell list, TAI list, and ISA list should be allowed, whereby the MBS broadcast service area is defined as the union of their coverage ranges when used in combination. |
| Summary of change: |  | Clarified the relationship between Cell TAI and Intended Service Area to align with RAN3 for MBS broadcast service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Leads to missalignment with NG-RAN specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

6.21	Support of Broadcast MBS Session over NR non-terrestrial networks

TS 38.300 [9] specifies the additional NG-RAN procedures to support Broadcast MBS Session over NR non-terrestrial network (NTN).

Specifically for NR NTN, an MBS service area in the 5GC and towards NG-RAN may be denoted with an NR NTN Intended Service Area as defined in TS 38.413 [15]. NR NTN Intended Service Area list could be used independently, or used in combination with a TAI list and/or cell ID list as defined in TS 38.300 [9].

Specifically for NR NTN, an MBS service area in the 5GC and towards NG-RAN may be denoted with an NR NTN Intended Service Area as defined in TS 38.413 [15] in combination with a TAI list and/or cell ID list, for instance to denote an area smaller than a cell. The NR NTN Intended Service Area is only used in NG-RAN to further restrict the MBS service area denoted by TAIs and/or cell IDs as defined in TS 38.300 [9].

Based on local configuration (e.g. related to the received geographical area from an AF), the NEF may determine that an MBS session is targeting a NR NTN network and may be configured to include the NR NTN Intended Service Area derived from the geographical area information in an MBS service area, and to provide this information to the MB-SMF as part of the MBS session creation procedure (see clause 7.1.1.2) and the MB-SMF then forwards the MBS Service Area during MBS Session Start procedures for Broadcast (see clause 7.3.1) towards the AMF. The AMF provides the MBS service area information towards NG-RAN. NG-RAN delivers the NR NTN Intended Service Area to the UE, and the UE handles this information as specified in TS 38.331 [28].

NOTE 1:	It is recommended that based on service agreement the AF uses a geographic area in an encoding compatible with TS 38.413 [15] for NR NTN.

NOTE 2:	The NR NTN Intended Service Area is transparently transmitted in 5GC, i.e. the MB-SMF selects the AMF based on TAIs, and the AMF also selects NG-RAN nodes in the same manner. (Cell IDs are provided together with related TAIs to enable this selection).

NOTE 3:	Location-dependent MBS services in Clause 6.2 for broadcast communication over NR NTN are not supported in this Release.

# * * * * End of changes * * * *