SA WG2 Meeting #169	S2-2505267

19 - 23, May, 2025, Fukuoka, Japan	(Revision of xxxx)

Source:	OPPO

Title:	New solution: Emergency QoS awareness at eNB for IMS Emergency Call via Control Plane

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to support Emergency QoS awareness at eNB for IMS Emergency Call via Control Plane.

1	Discussion

The following Key issues has been agreed in SA2#168:

- How to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC to comply with the relevant requirements defined in TS 22.261 [8] and TS 22.101 [9].

If only Control Plane CIoT EPS Optimization applies, the emergency service data is transmitted via control plane. During the attach procedure for NB-IoT UEs using Control Plane CIoT EPS Optimization, EPS bearer QoS including the ARP is not provided to the eNB, so that the emergency services traffic cannot be prioritized at the eNB.

To resolve the above issue, MME needs to instruct eNB to prioritize the handling of PDN connection for IMS emergency call if only Control Plane CIoT EPS Optimization is enabled for the UE.

As described in 8.3.11 of TS 36.413 [X], the UE information transfer procedure is for the MME to send the UE information including QoS Parameters and UE Radio capability to the eNB, for a NB-IoT UE using Control Plane CIoT EPS Optimization.

The UE INFORMATIONTRANSFER message sent by MME to eNB is as below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| S-TMSI | M |  | 9.2.3.6 |  | YES | reject |
| UE Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.1.15 | Includes QoS parameters. | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |

Proposal 1: this contribution proposes to reuse the UE INFORMATIONTRANSFER message to provide the QoS Parameters for emergency services to the eNB, so as to assist the eNB to prioritize the emergency service traffic when the eNB make resource allocation decisions between different NB-IoT UEs that are using CP CIoT Optimization.

As described in clause 5.4.4.1 of TS 23.401, for an emergency PDN connection, the PDN GW initiates the deactivation of all bearers of that emergency PDN connection when the PDN connection is inactive (i.e. not transferring any packets) for a configured period of time or when triggered by dynamic PCC. However, the UE’s signaling connection may not be released immediately. The MME may instruct eNB that the UE’s priority can be adjusted after the emergency PDN connection has been released, and then the eNB can update the resource allocation decisions accordingly.

Proposal 2: After the emergency PDN connection has been released, the MME provides updated QoS parameters for the NB-IoT UEs that is using CP CIoT Optimization, and then the eNB can adjust the resource allocation decisions between different UEs accordingly.

2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * * Start of Changes * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  |  | X |  |

## 6.X	Solution #X: Emergency QoS awareness at eNB for IMS Emergency Call via Control Plane

### 6.X.1	High level principles

This solution is to address Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

The high-level principles of this solution are as below:

-	Only Control Plane CIoT EPS Optimization applies for the UE, the IMS emergency related signaling and traffic are both transmitted via NAS PDUs.

-	MME instructs the eNB that the UE is requesting IMS emergency call, so as to assist with resource prioritization decisions between different NB-IoT UEs.

-	After MME accepts the Attach Request and establishes the PDN connection for the IMS emergency call, the MME provides QoS Parameters including the ARP value reserved for emergency services in the UE INFORMATION TRANSFER message to the eNB.

-	After the PDN connection for emergency service has been released, the MME provides the eNB with updated QoS Parameters for the NB-IoT UEs.

### 6.X.2	Description

This contribution is to realize attach and emergency PDN connection establishment for the NB-IoT UE using Control Plane CIoT Optimization.

For a NB-IoT UE needs to receiving or requesting IMS emergency call, the NB IoT UE may initiate Attach procedure with following clarification and enhancements:

-	The UE connects to EPC via GEO access.

-	UE indicates the support for CP CIoT Optimization in Preferred Network Behaviour, and only CP CIoT Optimization will be enabled.

-	If the ESM message container is included in the Attach Request message, the MME establishes a PDN connection for IMS emergency call as part of the Attach procedure. If not, the UE initiates UE requested PDN connectivity procedure to establish a PDN connection for emergency services when Emergency Service is required.

-	During the PDN connection establishment, the MME applies the parameters (e.g. Emergency APN, Emergency QoS profile, Emergency PGW Identity, etc.) from MME Emergency Configuration Data.

-	In order to assist with the resource prioritisation decisions between different NB-IoT UEs, the MME provides the Emergency QoS profile for the UE that is using Control Plane CIoT Optimization to the eNB, after the PDN connection establishment.

-	After the PDN connection for emergency services has been released, the MME provides the updated QoS profile to the eNB, and then the resource prioritisation decisions at the eNB can be adjust accordingly.

### 6.X.3	Procedures

Figure 6.X.3-1 Attach and PDN connection establishment for IMS Emergency Call over GEO via Control Plane

Step 1: If UE needs to perform emergency services, the UE may initiate Attach by the transmission, to the eNB, of an Attach Request Message. The UE indicates its support for Control Plane CIoT EPS Optimization in Preferred Network Behaviour. If the ESM message container is included, the UE shall set the Request Type to "Emergency" in the Attach Request. If the ESM message container is omitted, steps 5-9 are not executed.

Step 2: The eNB forwards the Attach Request message to MME as specified in clause 5.3.2.1 of TS 23.401[5]. The eNB indicates RAT type as NB-IoT (GEO) to MME.

Step 3: Authentication and NAS security setup are performed as specified in clause 5.3.2.1 of TS 23.401[5].

Step 4: MME may retrieve subscription context for the UE from HSS as specified in clause 5.3.2.1 of TS 23.401[5]. If the MME is not configured to support Emergency service, or the MME only supports User Plane CIoT EPS Optimisation, the MME shall reject the Attach Request.

Step 5: MME sends a Create Session Request to SGW as specified in clause 5.3.2.1 of TS 23.401[5]. For an Emergency Attach the MME applies the parameters from MME Emergency Configuration Data. In order to support Control Plane CIoT EPS Optimisation, MME indicates S11-U tunnelling of NAS user data and sends its own S11-U IP address and MME DL TEID for DL data forwarding by the SGW. The MME also includes a Control Plane Only PDN Connection Indicator in Create Session Request to indicate the PDN connection for IMS emergence call only uses the Control Plane CIoT EPS Optimisation.

Step 6: The SGW sends a Create Session Request to PGW indicated by the Emergency PGW Identity as specified in clause 5.3.2.1 of TS 23.401[5].

Step 7: The PCRF set PCC rule for emergency services as specified in clause 5.3.2.1 of TS 23.401[5]. The PCRF may also consider the RAT type, i.e. NB-IoT (GEO) satellite access, for the PCC decision.

Step 8: The PGW returns a Create Session Response to the SGW.

Step 9: The SGW returns a Create Session Response to the PGW. The SGW includes the SGW IP address and TEID for S11-U in Create Session Response to support the Control Plane CIoT EPS Optimisation.

Step 10: The MME sends an Attach Accept contained in S1-AP Downlink NAS transport message to the eNB. The MME indicates the CP CIoT EPS Optimisations it accepts in the Supported Network Behaviour, and also includes CP Only Indicator and Emergency Service Support indicator in the Attach Accept.

In addition, the MME instructs the eNB that the UE is performing emergency services, e.g. IMS Emergency Call, so that the eNB can prioritise resource allocation for the UE requesting emergency services. The MME provides the eNB with the negotiated QoS profile as described in clause 4.7.8 of TS 23.401[5]. The QoS profile includes the QoS parameters (e.g. QCI, ARP) reserved for emergency services. If more than one EPS bearer is established (e.g. one for IMS related signalling and the other for IMS data), the MME sends QoS profile for only one EPS bearer to the eNB. In such case, the MME uses local configuration (e.g. the MME chooses EPS bearer with the QCI corresponding to the highest Priority Level) to determine which EPS bearer's QoS to send to the eNB. The eNB may also maintain the UE in the RRC_CONNECTED state to prevent potential disruptions to IMS emergency call.

Step 11: The eNB forwards the Attach Accept message to the UE in an RRC Direct Transfer message.

Step 12: The UE sends a Direct Transfer message to the eNB, which includes the Attach Complete message.

Step 13: The eNB forwards the Attach Complete message to the MME in an Uplink NAS Transport message.

Step 14: UE requests a PDN connection for emergency services if Emergency Service is required and an emergency PDN connection is not already active, e.g. when a UE has set "Attach without PDN Connectivity is supported" in the Preferred Network behaviour.

Step 15: The MME provides the eNB with the QoS profile including the ARP value reserved for emergency services in a UE INFORMATION TRANSFER message as described in Step 10.

Step 16: If the MME determines that the PDN connection for emergency service has been released (e.g. upon receiving the Delete Bearer Request message), the MME provides the eNB with updated QoS parameters for the NB-IoT UE using Control Plane CIoT EPS Optimisation by sending a UE INFORMATION TRANSFER message, and then the resource prioritisation decisions at the eNB can be adjust accordingly.

### 6.X.4	Impacts to Services, Entities and Interfaces

NB IoT UE:

-	Support IMS Emergency Call over Control Plane.

MME:

-	Provide the eNB with emergency QoS parameters for the UE that is using CP CIoT Optimization in a UE INFORMATION TRANSFER message during the emergency attach.

-	Provide the eNB with updated QoS parameters for the UE that is using CP CIoT Optimization in a UE INFORMATION TRANSFER message after the emergency PDN connection has been released.

eNB:

-	Prioritize the NB-IoT UE that is performing emergency service based on the received emergency QoS parameters provided by MME.

-	Maintain the UE in the RRC_CONNECTED state to prevent potential disruptions to IMS emergency call.

-	Adjust UE’s priority based on the updated QoS parameters provided by the MME after the emergency PDN connection has been released.

* * * * End of Changes * * * *