3GPP TSG-SA WG2#169	S2-2505300

Fukuoka, Japan, 19-23 May 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Support implementation-specific Read and Write operation through generic AIoT Device specific command

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper implements changes to TS 23.369 in line with Proposal 2 in S2-2505298.

# 1.	Discussion and Proposal

As discussed in S2-2505298 [1], should there be no consensus to specify necessary parameters for read and write operations in AIoT NAS to ensure inter-operability, then this essentially means that read and write commands would be transparent containers exchanged between AF and AIoT Device.

As pointed out in [1], an AF implementation could send any type of proprietary command using the Read or Write operations, i.e. there is no guarantee that the AIoT NAS Read operation will trigger reading of data or rather something else. Also, specifying Read and Write AIoT NAS operations but leaving the details to implementation essentially means specifying two procedures for the exact same purpose, to exchange transparent containers between AIoT Device and AF, without any additional benefits. This is not only inefficient from protocol perspective but also unnecessarily increases development, testing and integration efforts.

Therefore, [1] proposed to avoid that by specifying only one generic procedure to exchange AIoT Device specific commands (and the related command response) between AF and AIoT Device. This pCR implements the related changes to TS 23.369.

# 2.	References

[1]	Qualcomm Incorporated: Ensuring an inter-operable Ambient IoT system. S2-2505298.

# >>>>BEGINNING OF CHANGES<<<<

### 5.2.2	AIoT Command service

Two categories of Command service operations are supported: Standardized and AIoT Device-specific commands.

The following standardized commands are supported: Disable.

NOTE:	The format of AIoT Device-specific commands and the format of the related command responses is beyond the scope of 3GPP.

An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

# >>>>NEXT CHANGE<<<<

## 5.6	AF authorization for the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the AIOTF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, permanent disable. |
| Allowed target AIoT Devices | Indicate the allowed AIoT Device(s) for the indicated AF. The information indicating the allowed target AIoT Devices is a list of the permanent AIoT Device ID (see clause 5.7) or the filtering information (see clause 5.8). |

The authorization of the AF for the AIoT includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, using the AF authorization data retrieved from n the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization.

# >>>>NEXT CHANGE<<<<

### 6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID],  NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

# >>>>NEXT CHANGE<<<<

### 7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: AIoT Device-specific command or Permenant Disable.

5)	Command specific parameters:

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

# >>>>NEXT CHANGE<<<<

### 7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (AIoT Device-specific command, or Disable).

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

# >>>>END OF CHANGES<<<<