3GPP TSG-SA2 Meeting #169	S2-2505326

Fukuoka, Japan, May 19th – 23rd 2025

Source:	Interdigital

Title:	KI#1, New Sol: architecture for sensing service enablement per sensing service area

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The architecture and functional description for sensing service enablement are proposed.

# 1. Introduction

This pCR proposes the solution for Key issue#1 system architecture to support sensing.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X | X | X | X |  |  |  |

# * * * * Second change (All text new) * * * *

## 6.X	Solution #X: architecture for sensing service enablement per sensing service area

### 6.X.1	High-level solution Principles

The Sensing NF manages sensing service per sensing service area. The Sensing NF authorizes the sensing service request from sensing service consumer based on the level of service agreement and the capability of sensing entities at the target sensing service area.

### 6.X.2	Description

This solution addresses key issue#1.

For the sensing service, a sensing NF may be defined. The supported functionalities of the sensing NF are authorization and revocation of sensing service request from sensing service consumers, capability management of sensing entities, sensing entity discovery and selection, coordination of sensing operation, and exposure of sensing result to sensing service consumer.

For sensing service enablement, sensing service area is defined and managed by the sensing NF. Sensing service area can be mapped to a list of cells, tracking area, etc. Each sensing NF can be configured to serve several sensing service areas.

When a sensing service is requested, the service is requested to be served at a sensing service area or multiple sensing service areas (e.g., object tracking). Based on the determined sensing service area, multiple sensing NFs may be involved for a sensing service request. When a service is requested for sensing and/or tracking a UE, the sensing service area may be decided based on the location of the UE.

A sensing NF may be designated for sensing service authorization from sensing service consumer for covering sensing service request needs support from multiple sensing service area. The designated sensing NF may manage the capability registration of sensing entity.

When a designated sensing NF is used for sensing service authorization, actual sensing service coordination may be performed by sensing NF serving target sensing service area.

Based on the sensing service area and capabilities of sensing entities, the involved sensing entities may be selected for the requested sensing service.

### 6.X.3	Procedure for sensing service

Figure 6.X.3-1 Procedure of the sensing operation per sensing service area

0.	It is assumed that sensing entities have registered their capabilities as sensing entities to the sensing NF.

1.	The AF requests a sensing service request. The request includes sensing service type (object detection, object tracking, environment sensing, etc), sensing service requirements (e.g., accuracy, latency, resolution, etc) and time information when the sensing service is needed (e.g., time for sensing measurement, time for sensing report). The service request may include a target region or a target UE.

When AF is an untrusted entity, the request may be sent to the sensing NF via the NEF.

When a sensing service request is for a sensing operation relating to a target UE or based on target UE’s location, the target UE’s information may be included.

2.	After receiving a service request, a (authorizing) sensing NF may authorize the sensing service request from the AF. (refer to 6.X.4)

The (authorizing) Sensing NF may authorize the sensing service based on the SLA and availability of sensing entities which may satisfy the sensing operation with the requirement at the target sensing service area.

If the requested sensing service is object tracking, the (authorizing) sensing NF may verify the availability of sensing operation over the sensing service area covering the path.

3.	The (authorizing) sensing NF may send a service response to the AF (via NEF). It may include whether the service request is accepted or rejected with relevant cause value.

4.	When there is a (authorizing) sensing NF which manages sensing operation at the target sensing service area, the (authorizing) sensing NF may send a request for sensing operation to a sensing NF.

The sensing operation request may include requested sensing service type, time for sensing operation, and sensing service requirement, requested target sensing mode and candidate list of sensing entity.

5.	When receiving a sensing operation request, a (distributed) sensing NF for the target sensing service area may perform discovery and selection procedure (refer to 6.X.5)

6.	The (distributed) sensing NF coordinates the sensing operation with the selected sensing entities.

Editor’s note: How sensing NF coordinates the sensing operation will be determined in cooperation with RAN WG.

7.	Sensing entities perform sensing operations as coordinated by sensing NF

Editor’s note: How sensing entities perform sensing operations will be determined in cooperation with RAN WG.

8.	Sensing entities may provide the collected sensing data to the sensing NF (via CP or via UP).

When the UP is used to transport the collected sensing data, the information to set up the UP path may be included in step 6 or may be configured after sensing entity registered its capability.

9.	The (distributed) sensing NF may calculate the sensing result based on the collected sensing data.

10.	The (distributed) sensing NF provide sensing result to the AF (via the (authorizing) sensing NF)

### 6.X.4	Sensing Service Authorization and Revocation

A sensing NF manages the service request for sensing from sensing service consumer. There may be a designated sensing NF for authorization of sensing service (i.e. an authorizing sensing NF).

The requested sensing service is authorized based on the service level agreement and the availability of sensing entities in the sensing serving area.

Upon receiving a request (via NEF), the (authorizing) sensing NF determines the target sensing service area(s) and the candidate sensing mode to support the requested sensing operation.

The (authorizing) sensing NF verifies whether the requested sensing mode can be performed in the requested target sensing service area(s) based on the available list of sensing entities in the sensing area(s).

If the authorization is not granted, the (authorizing) sensing NF may send a response with reject cause to the requesting sensing service consumer.

When the requested sensing service is authorized and accepted, the sensing operation is handled by (distributed) sensing NF(s) which serve the target sensing service area(s).

When the sensing service cannot be fulfilled by the sensing entities, the (authorizing) sensing NF may revoke the sensing service authorization of the sensing service consumer unless sensing operation can be fulfilled by other sensing entities including newly discovered and/or selected sensing entities.

### 6.X.5	Sensing entities discovery and selection

It is assumed that every sensing entity registered its sensing capability at the (authorizing) sensing NF.

Editor’s note: How sensing entity registers its sensing capability at the sensing NF is FFS.

When a sensing entity registers its supported sensing capability at the sensing NF, it may also register its location or supported sensing service area and supported sensing mode and its role (as sensing transmitter, sensing receiver, or both). In this release, the supported sensing capability only considers 3GPP based sensing capabilities.

A sensing NF may discover and select sensing entities for a target sensing area. When a sensing NF selects sensing entities, it shall consider the capability of sensing entity whether it may satisfy the requirement from sensing service consumer.

A (distributed) sensing NF may communicate with a (authorizing) sensing NF to gather the candidate list of sensing entities with their capabilities.

When a sensing entity is not available for performing a sensing operation for any reason (e.g. overloaded, energy consumption limit, etc), it is informed at the sensing NF and the sensing NF does not consider the sensing entity for a sensing operation.

When the sensing operation does not satisfy the requirements of the requested sensing service, sensing NF may update the sensing operation with newly discovered and selected sensing entities.

Editor’s note: When and how sensing entities are updated from a sensing service by the sensing NF is FFS.

### 6.X.6	Impacts on services, entities and interfaces

AF (as sensing service consumer):

-	Support a service request for sensing procedure

Sensing NF:

-	Support sensing service authorization and coordination of sensing operation with sensing entities

Sensing entities:

-	Support a sensing operation by coordination of sensing function.

# * * * * End of changes * * * *