3GPP TSG-SA2 Meeting #169	S2-2505329

Fukuoka, Japan, May 19th – 23rd 2025

Source:	Interdigital

Title:	KI#5, New Sol: Procedures for periodic sensing request with target service area or target UE

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The solution for KI#5 sensing result exposure is proposed.

# 1. Introduction

This pCR proposes the solution for Key issue#5 sensing result exposure.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  |  | X |  |

# * * * * Second change (All text new) * * * *

## 6.X	Solution #X: procedures for handling periodic/event triggered sensing request

### 6.X.1	High-level solution Principles

Periodic or event triggered sensing service request is supported.

When a sensing service for periodic or event triggered sensing report is requested for a sensing service area, the sensing NF may perform periodic sensing operation with the selected sensing entities.

When a sensing service is requested for a target UE and the sensing service area is based on UE’s location, sensing NF may request the target sensing UE to initiate a periodic sensing operation.

### 6.X.2	Description

When the AF requests a sensing service for periodic or event triggered sensing report, the sensing service report start time, end time, periodicity, and triggering condition (if event triggered report is requested) are included.

When the periodic or event triggered sensing service request is authorized, the (authorizing) sensing NF may assign a transaction ID to correlate the received sensing reports and provide to the sensing service consumer.

When a (distributed) sensing NF receives periodic or event triggered sensing report, the (distributed) sensing NF discover and select sensing entities may support the periodic sensing operation with the requested period.

When a sensing service request is based on the target UE’s location, the (distributed) sensing NF may request the target UE to initiate sensing service request based on the requested time.

The architecture and functional split are based on the solution in S2-2505326.

### 6.X.3	Procedure for sensing service with periodic/event triggered sensing report

#### 6.X.3.1	Procedure for sensing service

Figure 6.X.3-1 Procedure of the periodic sensing operation per target service area

- The AF requests a service request for sensing. The request includes sensing service type (object detection, object tracking, environment sensing, etc) and sensing service requirements (e.g., accuracy, latency, resolution, etc). The service request for sensing may include a target region or a target UE.

The service request for sensing may include sensing report type such as one time service report, periodic sensing service report, and event triggered sensing service report.

When a periodic sensing service report is requested, sensing report start time, sensing report ending time and periodicity of sensing report may be included.

When an event triggered sensing service report is requested, sensing report start time, sensing report ending time and sensing report triggering condition (e.g., detecting an object in the target sensing service area, detecting an object leaving target sensing service area, etc)

When the AF is an untrusted entity, the request may be sent to the sensing NF via NEF.

2.	The (authorizing) sensing NF may acquire the target UE’s current location to derive the target sensing service area.

3.	The (authorizing) sensing NF may authorize the sensing service request from the AF.

4	The (authorizing) sensing NF may send a service response to the AF (via NEF). It may include whether the service request is accepted or rejected with proper cause value.

5.	When there is a (authorizing) sensing NF which manages sensing operation at the target sensing service area, the (authorizing) sensing NF may send a request for sensing operation to a sensing NF.

The sensing operation request may include requested sensing service type and requirement, requested target sensing mode and candidate list of sensing entity.

6.	When receiving a sensing operation request, a (distributed) sensing NF for the target sensing service area may perform discovery and selection procedure. The (distributed) sensing NF selects sensing entities for periodic sensing operation with the requested period.

When the requested sensing operation is based on target UE’s location, the sensing NF may request the target UE initiate sensing operation based on the requested time as shown in clause 6.x.2.2.

7.	The (distributed) sensing NF sends a sensing request to the sensing entities with the requested sensing mode, list of sensing entities, time information for periodic sensing operation, and configuration for coordination of the sensing operation with the selected sensing entities.

Editor’s note: How sensing NF coordinates the sensing operation will be determined in cooperation with RAN WG.

The sensing operations from step 8 to step 11 repeat based on the configured duration and periodicity.

If the event triggered report is requested, the operation may stop when triggering condition is met.

8.	The sensing entities perform sensing operation as coordinated by sensing NF

Editor’s note: How sensing entities perform sensing operations will be determined in cooperation with RAN WG.

9.	The sensing entities may provide the collected sensing data to the sensing NF (via CP or via UP).

When the UP is used to transport the collected sensing data, the information to set up the UP path may be included in step 6 or may be configured after sensing entity registered its capability.

10.	The (distributed) sensing NF may calculate the sensing result based on the collected sensing data.

If the sensing NF detects the sensing operation may not satisfy the requirement for the requested sensing operation, it may update the sensing entities for the remaining period.

Editor’s note: It is FFS when and how sensing entities may be updated during sensing operation.

11.	The (distributed) sensing NF may provide sensing result to the AF (via the (authorizing) sensing NF) based on the requested reporting manner (periodic report or event triggered report). The sensing response may include transaction ID to associate the pending sensing service request.

#### 6.X.3.2	Procedure for sensing service

Figure 6.X.3-2 Procedure of the periodic sensing operation with target UE

1.	The AF requests a service request for sensing as shown in clause 6.x.2.1. In this case, the requested sensing service is based on the location of target UE.

2. – 5.	steps as shown in clause 6.x.3.1

6.	The (distributed) sensing NF may send a sensing request to the target UE which request to initiate a sensing service request with the requested sensing mode, indication for periodic sensing or event triggered sensing, and requested time information. The sensing request includes the transaction ID which will be used to associate the upcoming UE initiated sensing service request by the target UE with the pending sensing service request.

7.	The target UE may send a sensing response with acknowledgement.

8.	After receiving the acknowledgement, the (distributed) sensing NF may send a sensing response with an indication that the target UE acknowledged the UE initiated sensing request for the requested sensing operation with requested time.

9.	The (authorizing) sensing NF may send a service response for sensing to the AF.

10.	The target UE performs UE initiated sensing operation periodically for the requested sensing mode with requirements based on the requested time.

Editor’s note: How UE initiated sensing operation proceeds is to be handled by other solution.

### 6.X.4	Impacts on services, entities and interfaces

AF:

-	Support sensing request and revocation service with request for periodic, event triggered sensing report

Sensing NF:

-	Support sensing service authorization and revocation and coordinate sensing operation with sensing entities

-	Request a UE initiated sensing service request to the target UE

Sensing Entity:

- Performs periodic sensing operation in coordination of sensing NF.

UE

- Response to the sensing operation request from the sensing NF

- Support UE initiated sensing service request

# * * * * End of changes * * * *