3GPP TSG-SA2 Meeting #169	S2-2505334

Fukuoka, Japan, May 19th – 23rd 2025

Source:	NTT DOCOMO

Title:	KI#1 New Solution: QoS Mapping for IMS voice service over GEO satellite

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: A mechanism for QoS mapping for IMS voice service over GEO is proposed in the context of FS_5GSAT_Ph4_ARC.

# 1. Discussion

In TR 23.700-19, it is proposed to study the support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC as below:

-	System enhancements to support IMS voice service over NB-IoT NTN via GEO satellite connecting to EPC.

- 	Whether and how to support QoS for IMS voice service over GEO satellite.

In this contribution, a mechanism to support QoS mapping for IMS voice service over GEO satellite, the associated system architecture, the bearer mapping and the prioritization of different Signalling Radio bearers (SRBs) in case of Control Plane CIoT EPS Optimization is addressed.

Furthermore, a mechanism to separate radio bearers in case of non-IP PDN types (regardless of whether Control Plane CIoT EPS Optimization is used or not) is also addressed in this contribution.

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-19.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| X | X |  |  |  |

# * * * * Second change (All New Text) * * * *

## 6.X	Solution #X: QoS Mapping for IMS voice service over GEO satellite

### 6.X.0	High-level solution Principles

The high-level principles of the solution are as follows:

-	Architecture options with additional SRB(s) to support the transmission of SDFs for IMS voice and IMS signalling.

-	QoS mapping for IMS voice service over GEO by:

-	establishing dedicated SRB(s) and the associated EPS bearer id(s).

-	inclusion of EPS bearer identities (for the corresponding SRB(s)) in the “RRC Connection Reconfiguration Request”.

-	Separation of radio bearers (for both Control Plane CIoT EPS Optimization and User Plane CIoT Optimization) based on different traffic types for non-IP PDN connections.

### 6.X.1	Description

The solution provides a mechanism to support QoS mapping for IMS voice service over GEO satellite.

As per the current specifications, when Control Plane CIoT EPS Optimization—as specified in TS 23.401 [5]—is employed, the data transfer is done in NAS PDUs using the Signalling Radio Bearers (SRBs) that are shared with any other NAS and RRC signalling for the UE.

Further, NB-IoT only supports the establishment of a default EPS bearer, with no support for dedicated bearer setup. These limitations may not fulfill the QoS requirements of IMS-based voice services, where, for instance, IMS signalling may demand a higher QoS level than the corresponding voice media. Instead, the SDFs for voice and IMS signalling should be transmitted over distinct Signalling Radio Bearers (SRBs) across the air interface to satisfy their respective QoS requirements. To accommodate this, enhancements to support the establishment of dedicated EPS bearers—that can be mapped to dedicated SRBs—are required within the EPS when interfacing with the NB-IoT-based NTN employing Control Plane CIoT EPS Optimization.

The first part of the solution outlines the enhancement to enable the establishment of dedicated EPS bearers over NB-IoT NTN in order to support IMS voice services. The proposed approach is based on the Control Plane CIoT EPS Optimization framework. The solution leverages the use of new dedicated SRBs specifically allocated for the transmission of IMS signalling and voice packets.

Given the inherent increase in round-trip latency associated with GEO satellite propagation delays, the usage of a non-IP PDN type, rather than an IP-based PDN to reduce the protocol overhead and to meet data rate and call setup time requirements could be an alternative. The second part of the solution outlines the enhancement to separate radio bearers in case of non-IP PDN types.

### 6.X.2	Bearer Mapping

To support new SRBs, there could be two different architecture options. Figure 6.X.2-1 describes the first option wherein two new SRBs are used for IMS signalling and IMS voice. Figure 6.X.2-2 describes the second option, wherein 1 new SRB is used for IMS voice, and IMS signalling uses the existing SRB for NAS/RRC signalling.

Figure 6.X.2-1: Option 1: Two New SRBs for IMS signalling and IMS voice.

Figure 6.X.2-2: Option 2: New SRB for IMS voice; IMS Signalling uses existing SRB.

Editor’s note: Whether new SRBs are needed and can be supported depends on RAN.

In IMS voice sessions over E-UTRAN/EPS, the dedicated EPS bearer used for voice packet transmission is typically established on-demand using dynamic PCC, with QCI-1—a GBR bearer—commonly applied. However, when employing Control Plane CIoT EPS Optimization, an alternative approach is required to support IMS voice in this context. The following is proposed:

-	In accordance with the steps outlined in TS 23.401 [5], the “Initial Context Setup Request” or Bearer Setup Request from the MME to the eNodeB shall be able to indicate a request to establish dedicated SRB(s) and the associated EPS bearer id(s). Either one or two dedicated SRB(s) are established, depending on whether option 1 or option 2 in clause 6.x.2 is used.

- The subsequent “RRC Connection Reconfiguration Request” from the eNodeB towards the UE shall include the associated EPS bearer identities for the corresponding SRB(s).

- The subsequent EPS Session Management (ESM) requests from MME to UE carries the TFT filters for these EPS bearer identities.

- In UL direction, the UE shall map the UL traffic based on the TFT to the EPS bearer identities and the EPS bearer identities to the corresponding SBR(s) (as indicated in the RRC request from the eNodeB).

- In DL direction, the PGW shall map the DL traffic to the corresponding EPS bearer identities as configured by the MME. The eNodeB shall map the DL traffic to corresponding SRB(s) as configured by the MME.

### 6.X.3	Prioritization of different SRB(s)

The scheduling priority between the SRBs must be predefined within the UE for uplink and within the eNodeB for downlink transmission. For instance, if two dedicated SRB(s) are used, the SRB designated for IMS signalling may be assigned a higher scheduling priority than the SRB designated for the voice media. If only one dedicated SRB is introduced (i.e. option 2 as in Figure 6.X.2-2), the existing SRBs would continue to carry NAS/RRC traffic along with IMS signalling, while the new dedicated SRB could be utilized exclusively for RTP/RTCP-based voice media, assigning it a lower priority than for the existing SRBs.

### 6.X.4	Separation of Radio Bearers for Non-IP PDN

An additional challenge in supporting voice traffic over NB-IoT NTN via GEO satellite lies in fulfilling the data rate and call setup time requirements, taking into account the increased round-trip delay inherent to GEO satellite links. To address this, one alternative approach would be the use of a non-IP PDN type, rather than an IP-based PDN. This would for instance reduce the protocol overhead. The use of a non-IP PDN type could apply regardless of whether Control Plane CIoT EPS Optimization is used or not.

However, in case of non-IP PDN type, the TFT filters can no longer use IP 5-tuple (i.e. source/destination IP addresses port numbers and transport protocol) to separate the different radio bearers. Hence, the following is proposed:

-	The separation of IMS signalling and voice media shall be based on the traffic type (SIP vs. RTP/RTCP).

-	In UL direction, the UE shall map the UL traffic based on the traffic type to the corresponding EPS bearer identities and shall use the corresponding radio bearer for the EPS bearer as configured in the RRC.

-	In DL direction, the PGW shall map the DL traffic based on the traffic type to the corresponding EPS bearer identities as configured by the MME. The eNodeB shall map the DL traffic to the corresponding radio bearers as configured by the MME.

-	If Control Plane CIoT EPS Optimization is used, the prioritization between the radio bearers would be same as described in clause 6.X.3.

### 6.X.5	Impacts on services, entities and interfaces

MME:

-	Based on the UE support for dedicated SRB(s), and possibly based on the user subscription, decides to utilize dedicated SRB(s) with the UE.

-	Supports establishing of dedicated SRBs in Initial Context Setup Request or Bearer Setup Request.

-	Supports indication of EPS bearer identities also for SRB(s).

-	For non-IP PDN type, supports indication of traffic type in TFTs in Session Management request.

eNodeB:

-	Supports creation of SRB(s) as requested by MME in Initial Context Setup Request or Bearer Setup Request.

-	Supports indication of EPS bearer identities also for SRB(s) in RRC connection reconfiguration request.

UE:

-	Indicates the support for dedicated SRB(s) to MME.

-	Supports indication of EPS bearer identities also for SRB(s) in RRC connection reconfiguration request.

-	For non-IP PDN type, supports indication of traffic type in TFTs in Session Management request.

PGW:

-	For non-IP PDN type, supports indication of traffic type in TFTs in EPS bearer establishment/modification procedures with SGW/MME.

PCF/PCRF:

-	For non-IP PDN type, if dynamic PCC is employed, supports indication of traffic type to PGW.

# * * * * End of changes * * * *