SA WG2 Meeting #169	S2-2505370

# Fukuoka, Japan, 19th- 23rd May  2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0743 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE LCS privacy check for ground truth data collection for UE positioning ML model training in NWDAF containing MTLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure described in clause 6.22.3 covers two scenarios as described below. We propose to include the changes in step 4 to cover the scenario in case b below: - LMF collects UE positioning input data and ML model training is performed in LMF. In this case, privacy profile check for LCS client list is to assure LMF is a LCS client. - LMF collects UE positioning input data for ML model training in NWDAF. In this case, privacy profile check for LCS client list is to assure LMF (function responsible for UE positioning data collection) and NWDAF(function using the UE positioning data collected to train ML model) are in the LCS client list is required. This CR uses S2-2504873 as the baseline and proposes following additional changes: Include the changes in step 4 to cover case b where the LMF and NWDAF are to be included in the privacy profile LCS client list since the UE location input data is collected by the LMF and used by the NWDAF containing MTLF for corresponding ML model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update step 4 to include if NWDAF MTLF is authorized to retrieve UE location (ground truth data as part of data collection for ML model training and performance monitoring), based on the UE privacy profile check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for Data collection by LMF to train the AI/ML based positioning model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN is used to e.g. train the ML Model for LMF-based AI/ML positioning.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection from the NG-RAN is required e.g. to train an ML Model for UE positioning for a number of UEs or to monitor the ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, e.g. when training an ML model using the PRU(s) associated to this LMF. The LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF to discover other PRU serving LMF(s) which has associated PRUs in the area of interest and send an Nlmf_Location_MeasurementData Request service operation to the selected PRU serving LMFs to collect the measurement data from the NG-RAN for the PRUs and the PRU locations.

If the LMF does not know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, the LMF tries to get the list of SUPIs from the AMF. Before that the LMF discovers the AMF(s) that serves the area of interest via the NRF using Nnrf_NFDiscoveryRequest.

3.	The LMF subscribes to the list of SUPIs in an area of interest from the AMF(s) using Namf_EventExposure_Subscribe request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest").

The AMF sends Namf_EventExposure_Subscribe response or Namf_EventExposure_Notify (list of SUPIs in the area of interest).

For each SUPI in the area of interest, the following steps are performed.

4.	The LMF checks the LCS privacy profile for the SUPI with UDM using Nudm_SDM_Get including subscription data type set to " LCS privacy" for this SUPI. The UDM returns the target UE Privacy setting of the UE identified by its SUPI, if the SUPI of the target UE is not allowed to be located, then step 5 to 9 are skipped and this target UE input data is not used for model training at LMF. If in step 1, the LMF initiated the data collection upon the request of an NWDAF containing MTLF, then LMF determines if the NWDAF MTLF which is selected to train the AI/ML model for UE positioning is authorized to retrieve UE location (ground truth data as part of data collection for ML model training and performance monitoring), based on the UE privacy profile.

NOTE:	The LCS privacy profile check for the target UE is not needed when the target UE is a PRU.

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "LCS privacy" for this SUPI using Nudm_SDM_Subscribe. If the LCS requests is allowed, then step 6 follows, otherwise no data is collected for this SUPI, i.e. Steps 6 to step 9 are not performed.

6.	The LMF requests input data for the UE from the NG-RAN.

7.	The LMF requests ground truth data from the UE.

8.	The LMF may determine that the UE is no longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11, the LMF may unsubscribe to be notified on LCS privacy updates if the UE is not in the area of interest any longer.

9.	The UDM may notify the LMF on changes of LCS privacy profile at any time after step 5 using Nudm_SDM_Notification including SUPI and Subscription data type set to "LCS privacy". If LCS request is no longer granted for a user for which data has been collected the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of LCS privacy updates from UDM for each SUPI for which the LCS request has been revoked, using Nudm_SDM_Unsubscribe including SUPI and Subscription data type set to "LCS privacy".

10.		The LMF request NG-RAN to stop reporting input data for the target UE.

11.	The LMF stop any retrieval of ground truth data from the UE.

The measurements and ground truth data are used for ML model training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for ML model performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such steps 6 and 7 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

* * * End of Changes* * *