SA WG2 Meeting #169	S2-2505381

19 - 23 May, 2025, Fukuoka, Japan

Title:	[DRAFT] Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R2-2503169 (S2-2504512)

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	Nokia [will be SA2]

To:	RAN2

CC:	RAN1, RAN3, SA5, SA3

Contact person:	Yannick Lair

yannick.lair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks RAN2 for the LS on signalling feasibility of dataset and parameter sharing. SA2 discussed the non-OTA solutions as per information provided by RAN2 below:

Table 1. non-OTA candidate solutions

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5, SA3 | Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2, SA3 | Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2) |
| gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5, SA3 | Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

On the question from RAN2, SA2 concluded the following:

- The second option, i.e. where CN is NW-side dataset/model parameter collection entity, is not supported within existing architecture. Although the non-OTA candidate solutions table indicates only SA2 and SA3 as impacted WGs, the second option would also have impact on RAN3 to allow retrieving data from gNBs.

- The third option, i.e. where gNB is NW-side dataset/model parameter collection entity and the gNB would provide dataset/model parameter to CN which would then provide dataset/model parameter to UE-side training entity, is also not supported within existing architecture.

SA2 would like to understand better the difference between option 3 and the other 2 options:

- Option 3 with gNB -> OAM -> UE-side training entity seems identical to option 1 where the datasets/model parameters would anyway come from gNBs.

- Similarly, option 3 with gNB -> CN -> UE-side training entity seems identical to option 2 where the datasets/model parameters would anyway come from gNBs.

SA2 would like to ask RAN2 about the differentiating point(s) between option 3 and the other two options (option 1 and option 2).

SA2 could further investigate solutions according to either option 2 or option 3 once RAN2 come up with requirements.

# 2	Actions

To RAN2

ACTION:

SA2 requests RAN2 to take above reply into account and to answer SA2 questions.

# 3	Dates of next TSG SA WG 2 meetings

SA2#170		August 25 – 29, 2025      		Goteborg, SE

SA2#171		October 13 – 17, 2025      		China