3GPP SA WG2# 169	S2-2505383

Fukuoka, SE, April 19 – 23, 2025	(revision of xx-yyxxxx)

# Source:	NEC

# Title:	Support of simultaneous support for 3GPP access and non-3GPP access without UE impact in Rel-20

# Document for:	Agreement

# Agenda Item:	30.8

Abstract: The objective of the contribution is to demonstrate a solution with no UE impact which can provide simultaneous services to 3GPP access and non-3GPP access under INS. The discussion paper further proposes to add an objective to the Indirect Network Sharing phase2 WID to solve the case when simultaneous 3GPP and non-3GPP services cannot be provided under INS coverage.

# 1	Introduction

The CR 5925 captured in the TS 23.501 showed that the UE can be provided simultaneous services over 3GPP access and non-3GPP access when the UE is under INS coverage. The CR captures the network behaviour that when the UE is registered over 3GPP access and the UE is registering over non-3GPP access then the UE network will reject the registration over 3GPP access. This is seriously impacting the user experience as user under INS coverage is not able to access any service over non-3GPP access e.g. IMS voice over WiFI, Browsing over WiFi is not possible.

# 2	Discussion

The CR  captures that the UE can be provided services over non-3GPP access and 3GPP access under INS coverage simultaneously under one of the following two cases, otherwise the registration over the non-3GPP access will be rejected registration when the UE is registered over 3GPP access.

NOTE 8:	In this Release, if the participating operator deploys the Indirect Network Sharing and non-3GPP access (e.g. N3IWF) to 5GC simultaneously, and if the same PLMN ID is used for 3GPP access via Indirect Network Sharing and non 3GPP access (e.g. via N3IWF) to 5GC of HPLMN/participating PLMN, and if there is no N2 interface between the AMF supporting Indirect Network Sharing at the hosting PLMN and the non-3GPP access (e.g. N3IWF) to 5GC of the HPLMN, then the UE cannot be served simultaneously by the 3GPP access via Indirect Network Sharing and non-3GPP access (e.g. via N3IWF) to 5GC of the HPLMN.

To avoid this case, if the participating operator plans to deploy the Indirect Network Sharing and non-3GPP access (e.g. N3IWF) to 5GC simultaneously, it is recommended that one of following measures is used:

1)	the different PLMN IDs are used for 3GPP access via Indirect Network Sharing and non 3GPP access (e.g. via N3IWF) to 5GC of HPLMN.

2)	the N2 interface is deployed between the AMF(s) of the hosting PLMN supporting Indirect Network Sharing and the non-3GPP access (e.g. N3IWF) to 5GC of the HPLMN.

If none of the two measures in NOTE 8 can be used, the network rejects the UE access via non-3GPP access (e.g. via N3IWF) to 5GC of HPLMN with an existing cause value that will not allow access to 5GC of HPLMN via non-3GPP access (e.g. via N3IWF) when it detects the UE has been registered to its HPLMN ID over 3GPP access via Indirect Network Sharing.

NOTE 9:	If none of the two measures in the NOTE 8 can be used and the UE registers the non 3GPP access (e.g. via N3IWF) to 5GC of HPLMN first, then the network can accept the registration to HPLMN ID over 3GPP access via Indirect Network Sharing. How to handle this is up to implementation.

The above two deployment options will bring network complexities specially deploying N2 interface between participating PLMN N3IWF and host AMF. These two are not practical solutions as most of the PLMNs support single MCC and MNC for both 3GPP access and non-3GPP access and also deploying N2 interface between the host AMF and participating PLMN is not practical.

SA2 needs to find a solution which can provide a solution without UE impact based on common deployment scenario of one MCC and MNC of 3GPP access and non-3GPP access and no N2 interface between N3IWF of participating operator and the AMF of the host operator. This discussion paper in section 3 provides a solution which can demonstrate that the UE can be provided with simultaneous services over 3GPP access and non-3GPP access under INS coverage.

# 3	Solution

The following solution can provide the simultaneous services over 3GPP access and non-3GPP access when the UE

is under INS coverage.

- The UE is under INS coverage of the host PLMN and the UE initiates registration to the host PLMN.The UE sends registration request message to the host AMF.

- The host AMF sends Nudm_UECM_Registration Request message containing Indirect Network sharing Indicator and Host PLMN ID to the UDM indicating the UDM that the UE is under indirect network sharing.

- The UDM sends Nudm_UECM_Registration Response message to the UDM.

  - The UE and 5GS completes the registration procedure as defined in the sub clause 4.2.2.2. of the TS 23.502.

- The AMF initiates UE policy establishment as defined in the subclause 4.16.11 of the TS 23.502. The AMF sends Npcf_UEPolicyControl Create Request (SUPI) to the PCF.

- The PCF sends Nudr_DMQuery Request to the UDR/UDM.

- The UDR/UDM sends Nudr_DMquery Response containing UE subscription, host PLMN ID and indirect sharing indicator.

- When the PCF determines that the UE is under the INS coverage of the host PLMN, the PCF sets N3IWF identifier configuration to either FQDN of the N3IWF of the host PLMN or IP address of the N3IWF of the host PLMN.

- The PCF sends the Npcf_UEPolicyControl Create response message containing  UE ANDSP policy containing the N3IWF identifier configuration.

- The AMF executes the  UE Configuration Update procedure to transfer the UE policy to the UE.

- The registration procedure for the non-3GPP access is triggered. The UE determines that the UE is in HPLMN, and therefore, the UE uses the N3IWF identifier to find the N3IWF as defined in sub clause of TS 23.501.

- The UE selects N3IWF of the host as defined in the subclause 6.3.6 of TS 23.501.

- The UE sends the registration request message to the N3IWF of the host PLMN and the UE includes 5G-GUTI assigned to the UE for 3GPP access in the AN parameter to the N3IWF. The N3IWF will select the host AMF based on the GUAMI of the 5G-GUTI.

- The host AMF will proceed for the registration procedure as defined in subclause of TS 23.502.

# 4	Proposal

It is proposed to add following objective to the Indirect Network Sharing phase2 WID:

WT#4 How to support simultaneous services over 3GPP access and non-3GPP access in indirect network sharing case.