3GPP TSG-WG SA2 Meeting #169 	S2-2505385

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-250xxxx)

# Source:	Apple (Moderator)

# Title:	New SID on Short Message Service to Emergency Response Centre

# Document for:	Approval

# Agenda Item:	30.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Short Message Service to Emergency Response Centre

Acronym: FS_SMS2EC_Ph2-ARC

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| SMS2EC_Ph2-REQ | SA1 | 1070052 | Stage 1 of SMS to Emergency Centre Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

3GPP specifications support Emergency Calls with voice and other media, but this does not include SMS.

Service requirements for Short Message Service to Emergency Response Centre have been specified in Rel-19 in clause 36 of TS 22.101.

# 4	Objective

The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101.

The study includes the following work tasks:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification methods as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE location, according to the local regulations (in roaming case, the local regulations of the VPLMN). How to route Short Message Replies back to the subscriber in accordance with national regulations.

NOTE 1: 	This includes scenarios where the UE is connected via non-terrestrial or roaming access (e.g., NB-IoT NTN used for SMS or visited PLMN), and the emergency number dialed may originate from a different regulatory region. The routing must ensure the message reaches the correct geographically relevant PSAP based on the UE’s real-time, accurate location, not the origin of the emergency number or the home PLMN.

NOTE 2:	Depending on the type of emergency service, Short Messages can be routed to different emergency response centres.

NOTE 3:	This study assumes that Short Message Replies to a Short Message to Emergency Response Center are treated as regular Short Messages in a similar manner as call back is handled to an emergency call.

For EPS and 5GS (including both PLMN and SNPN), only 3GPP access (including TN and NTN) is in scope.

Editor’s Note: The support for SNPN needs further discussion. 2 companies propose to support it, while 3 companies propose not to support it.

UE in limited service state will be supported in this study item.

Editor’s Note: The support for UE in limited service state needs further discussion. 5 companies propose that UE in limited service state needs to be supported. 4 companies propose that UE in limited service state shall not to be supported.

The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 | 1 | 0.5 | No | WT-1 is self-contained |
| WT-2 | 1 | 0.5 | No | WT-2 is self-contained |
| WT-3 | 1.5 | 1 | No | WT-3 is self-contained |

Total TU estimates for the study phase: 3.5

Total TU estimates for the normative phase: 2

Total TU estimates: 5.5

Editor’s Note: TU numbers need to be re-visited once support for UE in limited service state, supported access networks are decided.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.abc-de | Study on Short Message Service to Emergency Response Centre | TSG#109 (2025-09) | TSG#110 (2025-12) |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Security aspects will be analysed by SA3.

# 9	Supporting Individual Members

Supporting IM name

Apple

Nokia?

ViaSat

Ericsson?

Qualcomm?

Deutsche Telekom?

Google

Skylo Technologies?

Sateliot?

Novamint?

Iridium Satellite

EchoStar

Orange

Verizon