SA WG2 Meeting #169	S2-2505397

Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-25xxxxx)

Source:	CATT

Title:	KI #1, New Sol: Sensing Architecture

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#1 in TR 23.700-14.

1. Introduction

This pCR is proposed to add a new solution to key issue#1: System Architecture to Support Sensing.

2. Proposal

It is proposed to add the following changes in TR 23.700-14.

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 |  |  |  |
| #X | x |  |  |  |

* Second change (all text new)*

6.X	Solution #X: Sensing Architecture

6.X.1	High level principles

The principles of the sensing architecture are the following:

-	Introduce a new NF, Sensing Function (SF), in 5GC to support sensing service.

-	The SF can communicate with NG-RAN directly or via AMF. In the case of direct communication, a new interface (Nx) is introduced between the NG-RAN and the SF.

6.X.2	Sensing Architecture

The overall 5GS system architecture for sensing is shown in Figure 6.X.2-1.

Figure 6.X.2-1: Overall Sensing Architecture

The NG-RAN acting as the Sensing Entity supports different connectivity to SF (Sensing Function):

-	Direct connectivity: SF communicates with NG-RAN directly. To support the communication, the Nx interface is introduced to support the communication between SF and NG-RAN.

-	Indirect connectivity via AMF: SF communicates with NG-RAN indirectly via AMF. To support the communication, the N2 interface and Namf service based interface are reused.

Based on the overall sensing architecture in Figure 6.X.2-1, the SF may be further decomposed into control plane function (SF-C) and user plane function (SF-U), in this case, the sensing architecture (SF with C/U split) is shown in Figure 6.X.2-2.

Figure 6.X.2-2: Sensing Architecture (SF with C/U split)

6.X.3	Interfaces

The sensing architecture includes the following reference points:

-	Nx: reference point between the NG-RAN and the SF.

-	N2: reference point between the NG-RAN and the AMF.

-	In the case of SF with C/U split, instead of Nx reference point, the following reference points are supported:

-	Na: reference point between the NG-RAN and the SF-C.

-	Nb: reference point between the NG-RAN and the SF-U.

-	Nc: reference point between the SF-C and the SF-U.

The sensing architecture includes the following service based interfaces:

-	Nsf: service based interface exhibited by SF.

-	Nnrf: service based interface exhibited by NRF.

-	Nnef: service based interface exhibited by NEF.

-	Namf: service based interface exhibited by AMF.

-	Naf: service based interface exhibited by AF.

6.X.4	Functional Entities

6.X.4.1	SF

The SF supports the following functions:

-	Support connectivity to an NG-RAN via a direct interface or via an AMF.

-	Sensing Entity information storage and maintenance.

-	Sensing Entity selection.

-	AMF selection in the case of indirect communication with NG-RAN.

-	Support a request for one time, periodic or event triggered sensing result from AF or NEF.

-	Report sensing result to AF directly or via NEF.

-	Determine the sensing mode(s).

-	Sensing measurement data collection.

-	Sensing result calculation.

If the SF supports C/U split, the SF-U is responsible to collect the sensing measurement data.

6.X.4.2	AMF

The AMF is responsible to relay signalling for sensing service between the NG-RAN and the SF.

6.X.4.3	NEF

To support sensing service, the NEF supports the following functions:

-	Sensing service exposure to AF.

-	SF selection for sensing service.

-	Authorization of untrusted AF’s sensing request.

6.X.4.4	NRF

To support sensing service, the NRF supports the following functions:

-	Support SF and associated profile.

-	SF discovery.

6.X.4.5	AF

The AF can request sensing service from SF directly (for trusted AF) or via NEF (for untrusted AF).

6.X.5	Impacts to Services, Entities and Interfaces

New NF introduced: SF.

New interfaces: Nx, Na, Nb, Nc, Nsf.

Impacts to existing NFs:

-	AMF: relay signalling for sensing service between the NG-RAN and the SF.

-	NEF:

-	Sensing service exposure to AF.

-	SF selection for sensing service.

-	Authorization of untrusted AF’s sensing request.

-	NRF:

-	Support SF and associated profile.

-	SF discovery.

-	AF: request sensing service from SF directly (for trusted AF) or via NEF (for untrusted AF).

* End of changes *