3GPP TSG-SA2 Meeting #169	S2-2505399

Fukuoka, May 19th – 23th 2025

Source:	CATT

Title:	KI#4, New Sol: Sensing Data Collection and Transport

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Sensing Data and the Associated Information Collection and Transport for FS_Sensing_ARC are proposed.

1. Introduction

This pCR proposes the solution for Key issue#4 Sensing Data and the Associated Information Collection and Transport

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #4> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

# * * * * Second change (all text new)* * * *

6.X	Solution #X: Sensing Data Collection and Transport

6.X.1	Description

This solution aims to address key issue#4 Sensing Data and the Associated Information Collection and Transport.

The principles of the solution include:

-	The SF triggers sensing data collection in sensing entity which is responsible to transport sensing data to SF.

-	If the SF is splited into SF-C and SF-U, the SF-U is responsible to receive and store the sensing data, calculate and expose the sensing result.

6.X.2	Procedures

Figure 6.X.2-1: Sensing Data Collection and Transport procedure

- SF/SF-C receives the trigger to generate sensing result, e.g. sensing request from NEF is received.

- The SF/SF-C selects Sensing Entity(ies). If Sensing Function is splited into SF-C and SF-U, the SF-C also selects SF-U(s) based on information of SF-U, e.g. sensing service area, sensing data processing capability, sensing data storage capability, load, etc.

- The SF/SF-C sends Sensing Measurements Configuration message to the chosen Sensing Entity(ies). The Sensing Measurements Configuration message includes sensing data transfer configuration information which. is used to transport the sensing data from Sensing Entity(ies) to SF/SF-U, e.g., the SF/SF-U address(es).

- If Sensing Function is splited into SF-C and SF-U, the SF-C sends Sensing Result Generation Configuration message to the chosen SF-U(s) to instruct the SF-U(s) to receive, store and/or process the sensing data, and transport the sensing result to sensing service consumer.

- The Sensing Entity(s) obtain the sensing data.

- The Sensing Entity(s) sends the sensing data to the SF/SF-U.

- The SF/SF-U generates the sensing result using the received sensing data. The SF/SF-U stores the received sensing data and the sensing result.

- If Sensing Function is spited into SF-C and SF-U, the SF-U reports the sensing result to the sensing service consumer received in step 3.

6.X.3	Impacts on Services, Entities and Interfaces

SF/SF-C/SF-U:

-	Support sensing data collection

-	Support sensing data and sensing result storage.

Sensing Entity:

-	Perform sensing measurement and send the measurement data to SF/SF-U.

# * * * * End of changes * * * *