SA WG2 Meeting #169	S2-2505400

Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-25xxxxx)

Source:	CATT

Title:	KI #5, New Sol: One time and event triggered sensing result exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: Introduce a new solution to key issue#5 in TR 23.700-14.

1. Introduction

This pCR is proposed to add a new solution to key issue#5: Sensing Result Exposure.

2. Proposal

It is proposed to add the following changes in TR 23.700-14.

* First change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#5 |  |  |  |
| #X | x |  |  |  |

* Second change (all text new)*

6.X	Solution #X: One Time and Event Triggered Sensing Result Exposure

6.X.1	High level principles

The high level principles of the solution include:

-	The network can expose one time or event triggered sensing result(s) to AF.

-	To support the one time sensing result exposure, when SF receives the sensing request, it triggers the sensing procedure to obtain the sensing result and sends the sensing result to AF directly or via NEF.

-	To support the event triggered sensing result exposure, when the event is detected, the SF triggers the sensing procedure to obtain the sensing result and sends the sensing result to AF directly or via NEF.

6.X.2	Description

The network can expose one time or event triggered sensing result(s) to AF based on AF request.

-	One time sensing result exposure: when the SF receives the sensing request for one time sensing result, it triggers the sensing procedure to obtain the sensing result and sends the sensing result to AF directly or via NEF.

-	Event triggered sensing result exposure: when the SF receives the sensing request for event triggered sensing result, it is responsible to detect whether the event happens or not. If the event is detected, the SF triggers the sensing procedure to obtain the sensing result and sends the sensing result to AF directly or via NEF. The following types of event are supported.

-	Area: An event where the sensing target enters, leaves or remains within the target sensing service area.

-	Periodic: An event where a defined periodic timer expires.

-	Tracking: An event where the sensing target is detected, the moving path of the sensing target in a specific time period is requested.

Editor's note:	whether and what new event type(s) is needed is FFS.

6.X.3	Procedures

The procedure to support sensing result exposure is shown in Figure 6.X.3-1.

Figure 6.X.3-1: Sensing Result Exposure

1.	The AF invokes Nnef_Sensing Request Service operation to the NEF. The service operation includes AF ID, target sensing service area specified in TS 22.137[2] and sensing QoS. The sensing QoS includes LCS QoS, accuracy of velocity estimate. The service operation may also include the information about the sensing target which is the typical size of the sensing target specified in clause 6.2 in TS 22.137.

To request sensing result reporting when specific event is detected, the service operation may also include information about the event,

-	Area: An event where the sensing target enters, leaves or remains within the target sensing service area.

-	Periodic: An event where a defined periodic timer expires. In this case, the AF also provides the time interval between successive sensing results.

-	Tracking: An event where the sensing target is detected, the moving path of the sensing target in a specific time period is requested.

2.	The NEF authorize the AF request.

3.	The NEF discovers and selects SF(s) to handle the request via NRF.

4.	The NEF invokes the Nsf_Sensing Request(AF ID, target sensing service area, information about the sensing target, sensing QoS) service operation towards to the selected SF(s).

5.	The SF sends the Nsf_Sensing Response to the NEF containing the accept or reject result for the sensing request.

6.	The NEF sends the Nnef_Sensing Response to the AF containing the accept or reject result for the sensing request.

7.	The SF performs the sensing procedure. If the AF requests sensing result reporting when specific event is detected, the SF is responsible to detect whether a specific event happens. When an event is detected, the steps 7-9 are performed.

8.	The SF sends the sensing result to the NEF by sending the Nsf_Sensing_Notify message including the location and velocity of the sensing target and the timestamp.

9.	The NEF informs the AF of the sensing result by sending the Nnef_Sensing_Notify.

6.X.4	Impacts to Services, Entities and Interfaces

NEF:

-	Support one time and event triggered sensing result exposure.

SF:

-	Trigger the sensing procedure when sensing request for one time sensing result is received.

-	Support event detection, trigger the sensing procedure when the event is detected.

-	Provide the sensing result to NEF.

AF:

-	Request one time and event triggered sensing result(s) from NEF.

* End of changes *