3GPP TSG|WG-SA2 Meeting #169	S2-2505411

Fukuoka, Japan, May 19 – 23, 2025

Source:	Verizon, Oracle

Title:	Discussion on QoS Flow Sustainability

Agenda Item:	9.23.2

Document for:	Information

# 1.	Background

In TS 23.288 clause 6.9, QoS Sustainability Analytics can provide information regarding the potential QoS change statistics for an analytics target period in the past, in a certain area, or the likelihood of a QoS change for an analytics target period in the future in a certain area.   The consumer of this analytics provides reporting threshold(s) to indicate the conditions on the analytic level for triggering the reporting. In other words, when the analytics result crosses the threshold of the expected QoS KPI, a report is triggered.

# 2.	Discussion

The threshold for reporting (mentioned above) is different for GBR and non GBR flows.

For non GBR flow, the Reporting Threshold(s) refer to the RAN UE Throughput KPI and/or end-to-end delay KPI (i.e. RAN KPI + GTP delay KPI).

For GBR flow, the reporting threshold refers to the flow Retainability KPI defined in TS 28.544 clause 6.5.1.1. This KPI basically indicates how often the end user abnormally loses the QoS flow during the time the QoS flow is used.  This is a measure of a total loss of the QoS flow.

The QoS flow Retainability KPI is not sufficient to determine the ability of the wireless system to maintain a flow with some level of acceptable degradation. In addition, when the flow is degraded (i.e. not dropped) an Alternate QoS Profile may be used instead if the QoS Notification Control is enabled.   If the OAM system has the ability to monitor and capture statistics of downgraded QoS flows by counting the QoS degradation events per a period of time (flow degradations/second), why not use such statistics to determine the probability of experiencing QoS degradation at the area the measurements are performed?

When such information is available the following may be done:

- Help the consumer of the analytics to potentially sustain the flow when the level of KPI degradation is nevertheless acceptable and/or adoptable

- Enable the consumer to activate Alternative QoS Profile

- Provide a good RAN deployment tool aiming to improve the performance of QoS flow KPIs in a given area of interest.

# 3.	Proposal

Proposal: Generate LS to SA5 asking the following:

- Request to understand the present or future ability of SA5 to monitor and capture statistics of downgraded QoS flows.

- In case statistics of downgraded QoS can be collected, ask SA5 to add a QoS Maintainability KPI on the basis of IP flow level. The KPI will reflects the probability to experience a QoS degradation of any kind during the time when the QoS flow is used.

- If/when QoS Maintainability is provided by SA5, add the following or similar red text in TS 23.288 clause 6.9.1:

“ - for a 5QI of GBR resource type, the Reporting Threshold(s) refer to the QoS flow Retainability KPI, related to the loss rate of the QoS flow, and/or QoS flow Maintainability KPI related to the probability of experiencing a QoS degradation”