3GPP TSG SA WG2 Meeting #169	S2-250xxxx

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	new solution for 3rd party initiates the Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This paper proposes new solutions “3rd party initiates the Sensing Service” to cover partially the key issue#2, KI#3, KI#5.

* * * Start of Changes (ALL NEW TEXT) * * *

6.X	Solution #X: < 3rd party initiates the Sensing Service >

6.X.0	High-level solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

In this solution, following principles are assumed:

- NEF performs the authorization of sensing service request from sensing consumer (e.g., 3rd party) and responsible to discover and select the SF

- SF provides the parameters/configuration to the sensing entity for specific sensing service and performs Sensing entity discovery and selection. And calculate the sensing result.

6.X.1	Description

This solution partially covers the parts of KI#2 “Authorization and Revocation to Support Sensing Service”, and part of KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection. And KI#6 Configuration Parameter Provisioning of Sensing Entities.

In this solution, there are following assumptions:

- NEF checks the authorization of the sensing service request from sensing service consumer (e.g., 3rd party).

- NEF stores the mapping between sensing target geographical area and the sensing service area offered by Sensing Function.

- NEF is also responsible for SF discovery and selection. Based on the sensing service requirements, e.g., sensing target geographical area, etc.

- SF is responsible for Sensing Entity(ies) discovery and selection based on the sensing service requirements, e.g., sensing target geographical area, etc.

- SF provides the parameters/configuration to the selected Sensing entity(ies)

- SF collects the sensing data from sensing entity(ies) and calculate the sensing result.

Editor’s note, how to perform the reselection of Sensing Function is FFS.

6.X.2	Procedures

The general procedure for 3rd party initiated Sensing Service as shown in figure 6.X.2.

Figure 6.X.2-1 general procedure for 3rd party initiated Sensing service

Procedure:

0, It is assumed that NEF stores the mappings between target sensing geographical area and sensing service area offered by SF.

1, the sensing service consumer (e.g., 3rd party AF) determines to send the sensing service request to NEF, including Sensing Service requirements (sensing time, sensing duration, sensing accuracy, target sensing geographical area, location of sensing transmitter or/and receiver, etc.), and Sensing assistance information (e.g., map info, etc.) as defined in TS 22.137.

2, NEF checks whether the sensing service consumer is authorized to request the Sensing service, when receiving the sensing service request.

Editor’s note: the details authorization will be defined in SA3.

3, If authorized, NEF performs SF discovery and selection to find an appropriate SF, for example, based on the target sensing geographical area and the stored mapping information in step 0. Alternatively NEF can discover and select the SF via NRF.

If there is no SF found, the NEF feedbacks the result to the sensing service consumer with failure cause.

4, The NEF sends the Sensing Service request to the selected Sensing Function, including the parameter as specified in step 1.

5, The Sensing Function performs the discovery and selection of sensing entity(ies), by considering the sensing service requirements, e.g., specific time, sensing duration, the target sensing geographical area, etc.

If no appropriate sensing entities found, SF sends the response to Sensing service consumer via NEF with failure cause.

6, SF provisions the parameters/configurations for the Sensing entity(ies), including Sensing service identifier, Sensing Service requirements [sensing accuracy, specific time, sensing duration, etc.]

7-8, SF sends the Sensing service response to Sensing Service consumer via NEF with result (accept)

9, The sensing entity (i.e., sensing transmitter(s) or/and sensing receiver(s)) starts to detect the target area /target, based on the parameters/configuration received from SF. SF calculates the sensing result based on the sensing data collected from Sensing Entity(ies). After that SF delivers the sensing result to the Sensing Service consumer.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

- NEF:

  - NEF checks the authorization of the sensing service request from sensing service consumer (e.g., 3rd party).

  - NEF stores the mapping between sensing target geographical area and the sensing service area offered by Sensing Function.

  - NEF is also responsible for SF discovery and selection. Based on the sensing service requirements, e.g., sensing target geographical area, etc.

- SF:

  - SF is responsible for Sensing Entity(ies) discovery and selection based on the sensing service requirements, e.g., sensing target geographical area, etc.

  - SF provides the parameters/configuration to the selected Sensing entity(ies)

  - SF collects the sensing data from sensing entity(ies) and calculate the sensing result.

* * * End of Changes * * *