3GPP TSG-WG SA2 Meeting #169	S2-2505430

Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	NRF Solution to Key Issue #3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: : Sensing Function Discovery and Selection of ISAC using NRF is proposed.

# 1. Introduction/Discussion

This contribution aims to provide a solution to Sensing Function selection to address Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection.

FIRST CHANGE

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

Second CHANGE (all new)

## 6.x	SF (Sensing Function) Selection

## 6.x.1	General Concepts of SF Selection

More than one SF in the HPLMN can serve the ISAC requests. SF discovery and selection functionality is supported by trusted AF, AMF, LMF, NEF.

Trusted AF, NEF, LMF, AMF may be configured with SF address(es). Those NF may also query the NRF to get SF instance address(es).

There could be following selection criteria when selecting SF:

-	The expected sensing QoS

-	The target geographic area/location area included or deduced based on the sensing request

-	The onboard UE(s), if involved, their profiles

-	The attributes (e.g., capabilities,  processing capacity, etc) of SF instance

-	The context of the request (e.g., the SF may be directly/indirectly deduced from the information in the request).

-	By regulatory requirements, whether the selected SF instance is compliant with emergency or legal use cases or not.

## 6.x.2	SF Selection Procedures

#### 6.x.2.1	SF Selection by Network Function

When AF issues ISAC Request(s), there could be NEF receives the request (for the untrusted AF) or SF receives the request directly.

- For trusted AF, based on its context, it can query NRF for the SF instance identification.

- For untrusted AF, based on its request, NEF can query NRF for the SF instance identification.

- When UE is onboard for the sensing target or as used to describe the sensing target area, the NEF may forward the sensing request to the serving AMF of the UE and this serving AMF can perform the SF selection.

Figure 6.x.2.1-1 NRF query for SF Selection 	TODO (response)

NRF may contain the most suitable SF instance to serve the request:

Option 1.a:  AF can be trusted (e.g., an internal NF) which may bypass NEF, as an internal NF, it may query NRF and invokes SF directly.

AF may use its request context for the SF selection. The criteria could be based on:

-	The expected sensing QoS

-	The attributes (e.g., capabilities,  processing capacity, etc) of SF instance

-	The context of the request (e.g., the SF may be directly/indirectly deduced from the information in the request).

-	By regulatory requirements, whether the selected SF instance is compliant with emergency or legal use cases or not.

Option 2.b: Core Network Functions can perform the SF selection after receiving the ISAC request. The criteria could include:

-	The achievable sensing QoS

-	The target geographic area/location area included or deduced based on the sensing request

-	The (onboard) UEs, if involved, their profiles

-	The attributes (e.g., capabilities,  processing capacity, etc) of SF instance

-	The context of the request (e.g., the SF may be directly/indirectly deduced from the information in the request).

### 6.X.3	Impacts on services, entities and interfaces

Editors’ Note: The impacts on the core network functions and sensing entities are FFS

END OF CHANGES