3GPP TSG-SA WG2 Meeting #169	S2-2505535

Fukuoka, Japan, May 19 – 23, 2025

Source:	vivo

Title:	Methodology of call setup time estimation for GEO voice call

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes the method on how to estimate call setup time for solutions, in order to compare different solutions on optimizing call setup procedure.

1. Discussion

Solutions for optimizing call setup procedures need to estimate the call setup time for final evaluation. If different solution uses different calculation method, the time estimations will be invalide for evaluation metric. Hence, we need a standardized method for call setup time estimation.

The formula for calculation ECST of satellite to terrestrial and example IMS message size is shown in DP S2-2505036, according to the DP, the parameters for using the formula to calculate ECST of satellite to terrestrial are proposed as following:

Dpropagation: 0.28s, considering the NBIoT (GEO) RAT type;

ETterr: 2.5s, the half of VoLTE (4-5s based on experience);

Proposal:‌ Use baseline parameters for ECST calculation unless optimzed.

2. Proposal

The following proposal need to be considered on ECST calcuation for solutions addressing call setup optimization.

Proposal:‌ Use baseline parameters for ECST calculation unless optimzed.

* * * First of Change * * * * (all new text)

Annex X:
Methodology for estimating call setup time

X.1	Formula for estimated call setup time calculation

X.1.1	General

To ensure consistency in evaluating solutions based on estimated call setup time, solutions for evaluation must adhere to a standardized baseline methodology‌ for calculating the estimiated call setup time. This requirement enables direct comparison of latency performance across competing solutions through quantitative analysis.

The following is the example formula for calculation of estimated call setup time (ECST) from satellite to terrestrial, based on ideal radio circumstance (e.g., no retrying and retranmission, no segmentation, etc.) and no simultaneous transmission for multiple signals:

ECSTsatToTerrs≈ETstateTransition                               [MO]s+ETimsDelay                   [MO]s+ETepsDelay                   [MO]s+ETterr[s]

The following is the example formula for calculation of estimated call setup time (ECST) from satellite to satellite based on the same condition as above:

ECSTsatToSats≈ETstateTransition                               [MO]s+ETstateTransition                               [MT]s+ETimsDelay                   [MO]s+ETimsDelay                   [MT]s+ETepsDelay                   [MO]s+ETepsDelay                   [MT]s

Wherein, ETstateTransition is the estimated time of state transition from RRC-IDLE to RRC-CONNECTED (including paging for MT); ETimsDelay is the estimated delay of IMS procedure for call setup; ETepsDelay is the estimated delay of EPS procedure for call setup. The parameter ETterr is the transmission and propgation delay considering terristrial side, half of VoLTE (4-5s based on experience), 2.5s is assumed.

X.1.2	Estimated time of IMS procedure for call setup

The following is the formula for calculation of estimated time for IMS signalling exchange:

ECSTsatToTerrs≈ECSTimsDelays+ECSTepsDelays+Tterr[s]

ECSTimsDelays≈SsignallingOfIMSbits+SimsHeaderbitsRtransmissionbpss+Dpropagation×3×NUL-IMS+NDL-IMS[s]

NOTE 1:	It needs to be multiplied by 3 for sending an UL signalling considering 2 additional signalings for uplink resource scheduling.

NOTE 2:	For solution addressing Key Issue #2, ETimsDelay can be used for evaluation.

Wherein, variable SsignallingOfIMS is the size of IMS signalling, e.g., SIP or I1 message; variable SimsHeader is the size of protocol overhead (typically UDP/IP/PDCP/RLC/MAC) that carrying IMS signalling, typical value is 52 Bytes (UDP/IPv6 used); variable Rtransmission is the transmission rate; variable NUL-IMS and NDL-IMS are the number of UL and DL IMS messages respectively.

The parameter Dpropagation is 0.28s according to clause 7.4.1 of TS 22.261 [8].

X.1.3	Estimated time of EPS procedure during call setup

ECSTepsDelays≈SsignallingOfEPSbits+SepsHeaderbitsRtransmissionbpss+Dpropagation×3×NUL-EPS+NDL-EPS[s]

ECSTsatToSats≈ECSTimsDelay                   [MO]s+ECSTimsDelay                   [MT]s+2×ECSTepsDelay[s]

ECSTimsDelay

Wherein, ECSTimsDelayECSTepsDelaySsignallingOfIMSvariable SsignallingOfEPS is the size of EPS signalling, e.g., RRC signalling for bearer resource reservation; SimsHeadervariable SepsHeader is the size of protocol overhead (i.e., PDCP/RLC/MAC) that carrying EPS signalling; RtransmissionNUL-IMSNDL-IMSvariable NUL-EPS and NDL-EPS are the number of UL and DL EPS messages respectively.

DpropagationTterr

X.1.4	Estimated time of state transition

Editor’s note: The formula for calcuating estimated time for state transition from RRC-IDLE to RRC-CONNECTED and estimated time for paging MT UE is FFS.

X.2	Example message size and corresponding transmission delay

Following figure shows the typical IMS mobile call processing in TN scenario:

Figure X.2-1: IMS mobile call processing in TN scenario

Following table shows the example SIP signaling size and corresponding transmission delay:

Table X.2-1: Example SIP message size and corresponding transmission delay in 1kbps

| Sequence | Message Name | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- |
| 1 | INVITE | 2,564 (+52) | 20.93 |
| 2 | 100 Trying | 445 (+52) | 3.98 |
| 3 | 183 Progressing | 1,705 (+52) | 14.06 |
| 4 | PRACK | 1,054 (+52) | 8.85 |
| 5 | 200 OK (PRACK) | 726 (+52) | 6.22 |
| 6 | UPDATE | 1,929 (+52) | 15.85 |
| 7 | 200 OK (UPDATE) | 1,494 (+52) | 12.37 |
| 8 | 180 Ringing | 983 (+52) | 8.28 |
| 9 | 200 OK (INVITE) | 1,365 (+52) | 11.34 |
| 10 | ACK | 840 (+52) | 7.14 |

Following table shows the example EPS signaling size and corresponding transmission delay during the call setup:

Table X.2-2: Example EPS message size and corresponding transmission delay in 1kbps

| Sequence | Message Name | Direction | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- | --- |
| a.1 | RRCConnectionReconfiguration | DL | 92 (+5) | 0.78 |
| a.2 | RRCConnectionReconfigurationComplete | UL | 2 (+5) | 0.06 |

* * * End of Change * * * *